3GPP TSG-RAN WG4 Meeting #87	R4-1806765

Busan, Korea (Republic of), 21 – 25 May 2018

Agenda Item:	6.22.7.2.1

Source:	Ericsson

Title:	CQI reporting tests for sTTI

Document for:	Discussion

# 1	Introduction

RAN4#87bis agreed with the way forward on sTTI demodulation and CQI reporting test [1] and update of the simulation assumption [2]. This contribution discusses the open issue on CQI reporting test.

# 2	Discussion

## 2.1	 Test framework

RAN4 has not concluded the CQI test for sTTI whether it is based on subband or wideband. In our previous contribution [3], we raised the issue on the channel model used for subband CQI test. Since RAN1 agreed that the subband group size for sTTI is 12 instead of 6, which is used for the legacy 1ms TTI, our observation was the reported CQI index values from each subband are almost same with the current 2 path channel model. This results the throughput ratio of the best subband of the randomly selected subband is close to 1.0. To improve the throughput ratio so that gamma>1.0, one possible way is to change the delay position of second path such as 0.20us.

In the last meeting, RAN4 discussed the CQI2MCS table for CQI test [1]. We observed the table in the way forward [1] does not consider the impact due to SPDCCH transmission. Since SPDCCH PRB set will be allocated locally within the system bandwidth, the number of channel bits could be different per subband, and it results in different PDSCH decoding success rate due to different code rate per subband. For example, if we set the number of OFDM symbols to 2 for SPDCCH with subslot-PDSCH, some subband may have almost no channel bits for PDSCH. We can schedule SPDCCH only on a certain subband and BS schedules PDSCH on other subbands. However, we have only 4 subbands in the case of 10MHz channel bandwidth, and if we allocate SPDCCH on one of the subbands, there are only 3 subbands for subslot-PDSCH transmission. We wonder it is worth testing the best subband report out of three subbands.

Considering the issue on subband CQI reporting test, we prefer to adopt the wideband CQI reporting test for sTTI. As we observed in [3], we can reuse the existing wideband CQI test framework to sTTI. Moreover, because of the wideband transmission, it is easier to take into account the impact due to SPDCCH.

Proposal 1: For sTTI CQI test, RAN4 adopts the wideband CQI test with PUSCH 1-1.

## 2.2	CQI-to-MCS table

RAN4#86bis discussed the CQI-to-MCS table [1]. Table 1 lists the available OFDM symbols for slot-PSDCH. For CRS-based transmission, we assume 2 CRS ports and CFI=2. For DMRS-based transmission, we assume 2 CRS ports, CFI=2, 1 DMRS port, and no CSI-RS subframes.

Table 2 lists the available OFDM symbols for subslot-PDSCH. For CRS-based transmission, we assume 2 CRS ports and CFI=2. For DMRS-based transmission, we assume 2 CRS ports, CFI=2, 1 DMRS port, and no CSI-RS subframes. Moreover we assume DMRS is transmitted only in subslots 1, 3, and 5 as same assumption as demodulation requirements. Since there is no PDSCH transmission in subslot 0 because of CFI=2, average is taken over subslots 1 to 5.

Note both tables do not include the symbols for SPDCCH transmission. RAN4 need discuss further the SPDCCH configuration during CQI reporting test.

Proposal 2: RAN4 discusses further the SPDCCH configuration for CQI test.

Proposal 3: RAN4 discusses further whether the CQI to MCS table is derived based on the averaged OFDM symbols over one subframe or other methods.

Table 1	Available OFDM symbols for Slot-PDSCH.

|  | CRS |  |  | DMRS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1PRB | 12PRB | 50PRB | 1PRB | 12PRB | 50PRB |
| Slot 0 | 56 | 672 | 2800 | 50 | 600 | 2500 |
| Slot 1 | 76 | 912 | 3800 | 70 | 840 | 3500 |
| Average | 66 | 792 | 3300 | 60 | 720 | 3000 |

Table 2	Available OFDM symbols for Subslot-PDSCH.

|  | CRS |  |  | DMRS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1PRB | 12PRB | 50PRB | 1PRB | 12PRB | 50PRB |
| Subslot 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Subslot 1 | 32 | 384 | 1600 | 28 | 336 | 1400 |
| Subslot 2 | 24 | 288 | 1200 | 24 | 288 | 1200 |
| Subslot 3 | 20 | 240 | 1000 | 16 | 192 | 800 |
| Subslot 4 | 24 | 288 | 1200 | 24 | 288 | 1200 |
| Subslot 5 | 32 | 384 | 1600 | 28 | 336 | 1400 |
| Average (without subslot 0) | 26.4 | 316.8 | 1320 | 24 | 288 | 1200 |

Based on the available averaged OFDM symbols, we derived the CQI to MCS table for subband CQI test case (Table 3) and wideband CQI test case (Table 4). MCS is selected according to TS36.101 A.2.1.2. The used TBS tables for 12PRB and 50PRB are given in Table 5 and Table 6, respectively, in Appendix. Note this table is based on the OFDM symbol without SPDCCH transmission. It would be changed according to the SPDCCH configuration.

Table 3	CQI to MCS table for subband CQI test.

|  |  |  | sTTI_1 | sTTI_2 | sTTI_3 | sTTI_4 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 12PRB | 12PRB | 12PRB | 12PRB |
| CQI index | Target Coding rate | Modulation | Ch=792 | Ch=317 | Ch=720 | Ch=288 |
| 1 | 0.08 | 2 | 0 | 0 | 0 | 0 |
| 2 | 0.12 | 2 | 0 | 0 | 0 | 0 |
| 3 | 0.19 | 2 | 2 | 2 | 2 | 2 |
| 4 | 0.3 | 2 | 4 | 5 | 4 | 4 |
| 5 | 0.44 | 2 | 6 | 7 | 6 | 6 |
| 6 | 0.59 | 2 | 9 | 9 | 8 | 9 |
| 7 | 0.37 | 4 | 12 | 13 | 11 | 12 |
| 8 | 0.48 | 4 | 14 | 15 | 13 | 14 |
| 9 | 0.6 | 4 | 16 | 16 | 15 | 16 |
| 10 | 0.46 | 6 | 19 | 21 | 18 | 19 |
| 11 | 0.55 | 6 | 21 | 23 | 20 | 22 |
| 12 | 0.65 | 6 | 23 | 26 | 22 | 24 |
| 13 | 0.75 | 6 | 25 | 28 | 24 | 27 |
| 14 | 0.85 | 6 | 27 | 28 | 26 | 28 |
| 15 | 0.93 | 6 | 28 | 28 | 27 | 28 |

Table 4	CQI to MCS table for wideband CQI test.

|  |  |  | sTTI_1 | sTTI_2 | sTTI_3 | sTTI_4 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50 | 50 | 50 | 50 |
| CQI index | Target Coding rate | Modulation | 3300 | 1320 | 3000 | 1200 |
| 1 | 0.08 | 2 | 0 | 0 | 0 | 0 |
| 2 | 0.12 | 2 | 0 | 1 | 0 | 0 |
| 3 | 0.19 | 2 | 2 | 3 | 2 | 3 |
| 4 | 0.3 | 2 | 4 | 5 | 4 | 5 |
| 5 | 0.44 | 2 | 7 | 8 | 6 | 7 |
| 6 | 0.59 | 2 | 9 | 9 | 8 | 9 |
| 7 | 0.37 | 4 | 12 | 13 | 11 | 12 |
| 8 | 0.48 | 4 | 14 | 16 | 13 | 15 |
| 9 | 0.6 | 4 | 16 | 16 | 15 | 16 |
| 10 | 0.46 | 6 | 19 | 21 | 18 | 20 |
| 11 | 0.55 | 6 | 21 | 23 | 20 | 22 |
| 12 | 0.65 | 6 | 23 | 26 | 22 | 24 |
| 13 | 0.75 | 6 | 26 | 28 | 24 | 27 |
| 14 | 0.85 | 6 | 27 | 28 | 26 | 28 |
| 15 | 0.93 | 6 | 28 | 28 | 27 | 28 |

# 3	Conclusion

Proposal 1: For sTTI CQI test, RAN4 adopts the wideband CQI test with PUSCH 1-1.

Proposal 2: RAN4 discusses further the SPDCCH configuration for CQI test.

Proposal 3: RAN4 discusses further whether the CQI to MCS table is derived based on the averaged OFDM symbols over one subframe or other methods.

# 4	References

R4-1805506, “Way forward on sTTI DMRS pattern and CQI2MCS mapping table”, Huawei, Ericsson, Qualcomm.

R4-1805493, “Update of simulation assumption for sTTI UE demodulation and CSI requirements”, Ericsson.

R4-1804652, “Simulation results of CRS based CQI reporting for sTTI”, Ericsson.

R4-1806762, “TBS tables used for sTTI UE demodulation requirements”, Ericsson.

# Appendix

Table 5	TBS table for slot/subslot-PDSCH of 12PRB.

|  |  |  |  | Slot-PDSCH |  | Subslot-PDSCH |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IMCS | Qm | ITBS | TBS with 12PRB | TBSscaled | TBSslot | TBSscaled | TBSsubslot |
| 0 | 2 | 0 | 328 | 164 | 176 | 54.7 | 56 |
| 1 | 2 | 1 | 424 | 212 | 208 | 70.7 | 72 |
| 2 | 2 | 2 | 520 | 260 | 256 | 86.7 | 88 |
| 3 | 2 | 3 | 680 | 340 | 344 | 113.3 | 120 |
| 4 | 2 | 4 | 840 | 420 | 424 | 140 | 144 |
| 5 | 2 | 5 | 1032 | 516 | 520 | 172 | 176 |
| 6 | 2 | 6 | 1224 | 612 | 616 | 204 | 208 |
| 7 | 2 | 7 | 1480 | 740 | 744 | 246.7 | 256 |
| 8 | 2 | 8 | 1672 | 836 | 840 | 278.7 | 280 |
| 9 | 2 | 9 | 1864 | 932 | 936 | 310.7 | 296 |
| 10 | 4 | 9 | 1864 | 932 | 936 | 310.7 | 296 |
| 11 | 4 | 10 | 2088 | 1044 | 1032 | 348 | 344 |
| 12 | 4 | 11 | 2408 | 1204 | 1192 | 401.3 | 408 |
| 13 | 4 | 12 | 2728 | 1364 | 1352 | 454.7 | 456 |
| 14 | 4 | 13 | 3112 | 1556 | 1544 | 518.7 | 520 |
| 15 | 4 | 14 | 3496 | 1748 | 1736 | 582.7 | 584 |
| 16 | 4 | 15 | 3624 | 1812 | 1800 | 604 | 600 |
| 17 | 6 | 15 | 3624 | 1812 | 1800 | 604 | 600 |
| 18 | 6 | 16 | 3880 | 1940 | 1928 | 646.7 | 648 |
| 19 | 6 | 17 | 4392 | 2196 | 2216 | 732 | 744 |
| 20 | 6 | 18 | 4776 | 2388 | 2408 | 796 | 808 |
| 21 | 6 | 19 | 5160 | 2580 | 2600 | 860 | 872 |
| 22 | 6 | 20 | 5544 | 2772 | 2792 | 924 | 936 |
| 23 | 6 | 21 | 5992 | 2996 | 2984 | 998.7 | 1000 |
| 24 | 6 | 22 | 6456 | 3228 | 3240 | 1076 | 1064 |
| 25 | 6 | 23 | 6968 | 3484 | 3496 | 1161.3 | 1160 |
| 26 | 6 | 24 | 7224 | 3612 | 3624 | 1204 | 1192 |
| 27 | 6 | 25 | 7480 | 3740 | 3752 | 1246.7 | 1256 |
| 28 | 6 | 26 | 8760 | 4380 | 4392 | 1460 | 1480 |

Table 6	TBS table for slot/subslot-PDSCH of 50PRB.

|  |  |  |  | Slot-PDSCH |  | Subslot-PDSCH |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IMCS | Qm | ITBS | TBS with 50PRB | TBSscaled | TBSslot | TBSscaled | TBSsubslot |
| 0 | 2 | 0 | 1384 | 692 | 696 | 230.7 | 224 |
| 1 | 2 | 1 | 1800 | 900 | 904 | 300 | 296 |
| 2 | 2 | 2 | 2216 | 1108 | 1096 | 369.3 | 376 |
| 3 | 2 | 3 | 2856 | 1428 | 1416 | 476 | 472 |
| 4 | 2 | 4 | 3624 | 1812 | 1800 | 604 | 600 |
| 5 | 2 | 5 | 4392 | 2196 | 2216 | 732 | 744 |
| 6 | 2 | 6 | 5160 | 2580 | 2600 | 860 | 872 |
| 7 | 2 | 7 | 6200 | 3100 | 3112 | 1033.3 | 1032 |
| 8 | 2 | 8 | 6968 | 3484 | 3496 | 1161.3 | 1160 |
| 9 | 2 | 9 | 7992 | 3996 | 4008 | 1332 | 1320 |
| 10 | 4 | 9 | 7992 | 3996 | 4008 | 1332 | 1320 |
| 11 | 4 | 10 | 8760 | 4380 | 4392 | 1460 | 1480 |
| 12 | 4 | 11 | 9912 | 4956 | 4968 | 1652 | 1672 |
| 13 | 4 | 12 | 11448 | 5724 | 5736 | 1908 | 1928 |
| 14 | 4 | 13 | 12960 | 6480 | 6456 | 2160 | 2152 |
| 15 | 4 | 14 | 14112 | 7056 | 6968 | 2352 | 2344 |
| 16 | 4 | 15 | 15264 | 7632 | 7736 | 2544 | 2536 |
| 17 | 6 | 15 | 15264 | 7632 | 7736 | 2544 | 2536 |
| 18 | 6 | 16 | 16416 | 8208 | 8248 | 2736 | 2728 |
| 19 | 6 | 17 | 18336 | 9168 | 9144 | 3056 | 3112 |
| 20 | 6 | 18 | 19848 | 9924 | 9912 | 3308 | 3368 |
| 21 | 6 | 19 | 21384 | 10692 | 10680 | 3564 | 3624 |
| 22 | 6 | 20 | 22920 | 11460 | 11448 | 3820 | 3880 |
| 23 | 6 | 21 | 25456 | 12728 | 12576 | 4242.7 | 4264 |
| 24 | 6 | 22 | 27376 | 13688 | 13536 | 4562.7 | 4584 |
| 25 | 6 | 23 | 28336 | 14168 | 14112 | 4722.7 | 4776 |
| 26 | 6 | 24 | 30576 | 15288 | 15264 | 5096 | 5160 |
| 27 | 6 | 25 | 31704 | 15852 | 15840 | 5284 | 5352 |
| 28 | 6 | 26 | 36696 | 18348 | 18336 | 6116 | 6200 |