3GPP TSG RAN WG4 Meeting #87	R4-1807393

Busan, Korea, May 21 – 25, 2018

Agenda Item:	7.10.4.3.1

Source: 	Ericsson

Title:	On interruptions for inter-RAT SFTD

Document for:	Discussion

# Introduction

At RAN4#86bis a CR for introduction of core requirements on inter-RAT SFTD measurements, [1], was agreed. For the case when inter-RAT SFTD measurements are carried out without the support of configured measurement gaps, the UE may, depending on its RF capabilities, be allowed to cause interruptions in the PCell and activated SCell(s). The interruptions are however to be limited such that connection to PCell and activated SCell(s) is sustained. The amount of interruptions was however left for further studies.

In this contribution we are proposing requirements regarding the amount of interruptions for the case when the NR carrier belongs to FR1.

# Discussion

In the agreed CR on inter-RAT SFTD core requirements [1], it is stated:

The SFTD measurement shall be conducted with sustained connection to the E-UTRA PCell, however, the UE may be allowed to cause a certain amount of interruptions for reconfiguration of the radio receiver. The interruptions are FFS.

By allowing the UE to cause interruptions in the PCell for radio reconfiguration, the UE can save power since it can turn off parts of the RF circuitry while it is processing already acquired radio samples. The agreed minimum assumption on UE capability with respect to cell detection is that the UE can process 5ms of NR radio samples every 20ms. Hence for a UE with this minimum capability, the active radio time constitutes only 25%. Allowing interruptions thus opens up for power savings during the remaining 75% of the time during which inter-RAT SFTD is executed.

Proposal 1: Interruptions shall be allowed before and after collection of each chunk of radio samples used for inter-RAT SFTD.

Since the inter-RAT SFTD is to be conducted with sustained connection in the PCell, the extent of the interruptions needs to be limited. In E-UTRAN, for requirements such as CGI reading in autonomous gaps, requirements were defined for how many ACK/NACKs the UE shall be able to transmit during the execution of the measurement procedure. We propose that the same methodology is used for securing that the interruptions in PCell are bounded is applied for inter-RAT SFTD carried out without support of configured measurement gaps.

Proposal 2: The amount of interruptions shall be bounded by requirements based on ACK/NACK counting when the UE is continuously allocated on the DL.

The interruptions can be aligned with the PCell subframes, whereby each interruption impacts one UL, DL or UL/DL subframe only. Assuming that only one code word is transmitted per DL subframe, and further, that cell detection requires 10×SMTC period/20ms chunks of 5ms radio samples for SMTC period of 20ms or longer, and MIB decoding requires 2 chunks of radio samples, tentative ACK/NACK numbers are proposed in Table 1. For 10ms SMTC period, the cell detection is assumed to require 6 chunks of 5ms radio samples, and for 5ms SMTC period, 4 chunks of 5ms radio samples.

Table 1: Minimum number of ACK/NACKs to be transmitted by UE when continuously allocated on DL

|  |  | SMTC period |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration |  | 5ms | 10ms | 20ms | 40ms | 80ms | 160ms |
|  |  | A/N |  |  |  |  |  |
| FDD |  | 180 | 172 | 232 | 472 | 952 | 1912 |
| TDD | 0 | 68 | 94 | 192 | 388 | 780 | 656 |
|  | 1 | 102 | 140 | 286 | 578 | 1162 | 976 |
|  | 2 | 130 | 184 | 378 | 766 | 1542 | 1296 |
|  | 3 | 114 | 158 | 324 | 656 | 1320 | 1104 |
|  | 4 | 128 | 182 | 374 | 758 | 1526 | 1280 |
|  | 5 | 120 | 182 | 384 | 788 | 1596 | 1328 |
|  | 6 | 86 | 116 | 236 | 476 | 956 | 800 |
| Time [ms] |  | 200 | 200 | 280 | 560 | 1120 | 2240 |

# Summary and Conclusions

In this contribution we have looked into PCell interruptions for a UE carrying out an inter-RAT SFTD measurement on a NR carrier belonging to FR1. The following proposals are made:

Proposal 1: Interruptions shall be allowed before and after collection of each chunk of radio samples used for inter-RAT SFTD.

Proposal 2: The amount of interruptions shall be bounded by requirements based on ACK/NACK counting when the UE is continuously allocated on the DL.

Tentative minimum number of ACK/NACKs that a UE shall be able to transmit in PCell when being continuously allocated on DL while carrying out the inter-RAT SFTD measurement has been derived (Table 1).

A revision of [1] containing requirements on minimum number of ACK/NACKs to transmit in PCell or activated SCell(s) is provided in [2].

# References

- R4-1805956 “Introduction of inter-RAT SFTD measurement requirements”, Ericsson

- R4-1807395 “Introduction of inter-RAT SFTD measurement requirements”, Ericsson