TSG-RAN Working Group 4 (Radio) meeting 87	R4-1807481

Busan, South Korea, 21st – 25th May 2018

Source:	Ericsson

Title:	Reference angular steps for TRP measurement

Agenda item:	6.28.3.1

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#86bis in Melbourne) it was suggested to use 1o grid as reference for TRP measurement and error calculation [1].

The variations in radiated fields, and therefore the required angular step for accurate TRP measurement is dependent on the electrical size of the radiation source. For example, 1o angular step can be too dense or too sparse depending on the electrical size of the radiation source. Using the proper angular step can help reducing the measurement time for electrically small radiation sources and reduce the TRP error for electrically large sources.

This contribution discusses the reference angular step which is a function of the electrical size (physical size and frequency) [2, 3]. Different array sizes are used to demonstrate the error in TRP estimation vs. the angular step.

The results presented in this contribution are based on the assumption that the power flux density is measured without error. Eventual errors in power flux density will add to the TRP errors presented herein.

# Discussion

For a device with dimensions d, w, h defined in Figure 1, the cylindrical and spherical diameters, Dcyl and D are given as

Dcyl=d2+w2

D=d2+h2+w2

For each emission frequency, the reference angular steps  ϕ and Δθ in radians are calculated as [2]

Δϕ=λ/Dcyl

Δθ=λ/D

In the results presented in this contribution, Δϕ=Δθ is assumed for simplicity.

Figure 1 - Spherical coordinates and device dimensions, depth (d), width(w) and height (h).

Table I – The four array configurations used in the results.

| Array configuration | Number of elements | Frequency (GHz) | Element spacing (mm) | D (mm) | Δθ (deg) |
| --- | --- | --- | --- | --- | --- |
| 1 | 2x2 | 28 | 5.9 | 25.8 | 23.8 |
| 2 | 4x4 | 28 | 5.9 | 41.5 | 14.8 |
| 3 | 8x8 | 28 | 5.9 | 74.2 | 8.3 |
| 4 | 16x4 | 28 | 6 | 105.2 | 5.8 |

Figures 2-5 shows radiation pattern of 4 different array configurations with broadside beams, plotted with 1 degree resolution and with their respective reference angular step for comparison. The array configurations are summarized in Table I.

| Figure 2 – The radiation pattern of array #1, (left) sampled with 1 degree, and (right) with reference angular step |  |
| --- | --- |

| Figure 3 – The radiation pattern of array #2, (left) sampled with 1 degree, and (right) with reference angular step |  |
| --- | --- |

| Figure 4 – The radiation pattern of array #3, (left) sampled with 1 degree, and (right) with reference angular step |  |
| --- | --- |

| Figure 5 – The radiation pattern of array #4, (left) sampled with 1 degree, and (right) with reference angular step |  |
| --- | --- |

Observation 1: Fine details of the radiation pattern are not detected if it is sampled with the reference angular spacing.

Figure 6 shows the TRP error of the four array configurations calculated with different angular steps. The reference angular step is shown with the vertical dashed line. Different levels of correlation between the array elements are considered to show the effect of correlation on the TRP error.

| Array #1 | Array #2 |
| --- | --- |
| Array #3 | Array #4 |
| Figure 6 - The TRP error of different array configurations with different correlation levels vs. the angular step. The reference angular step is shown with the vertical dashed line. |  |

Observation 2: TRP can be accurately calculated with the reference angular step. Increasing the density of the measurement grid will not result in improved TRP but will only increase the measurement time.

Observation 3: The electrical size of the radiation source determines the reference angular step. The level of correlation has no effect on the reference angular step. As a result we do not need to find the positions peaks of the emissions

# Conclusion

Using a grid with reference angular step is enough for accurate TRP estimation based on power flux density measurements. The reference angular step is dependent only on the electrical size of the radiation source (i.e., physical size and the emission frequency) and is not affected by the correlation level. When sampling with the reference angular step, fine details of the radiation pattern are not observed, but the TRP value is not affected.

# References

[1]	R4-1805107, “Overview of In-band TRP uncertainty versus sampling grid”, Apr. 2018

[2] 	J. Fridén, A. Razavi and A. Stjernman, "Angular sampling, Test Signal, and Near Field Aspects for Over-the-Air Total Radiated Power Assessment in Anechoic Chambers," Submitted to a Journal, 2018. Available: https://arxiv.org/abs/1803.10993

[3]	R4-1805047, “TRP Test Method for Unwanted Emissions for eAAS BS”, Apr. 201