3GPP TSG-RAN WG4 #87	R4-1807537

Busan, Republic of South Korea, 21st – 25th May 2018

Source:	MVG Industries

Title:	TP to Draft CR 37.843 - OTA TAE measurement in a Near Field Test Range

Agenda item:	6.28.3.1.2

Document for:	Approval

# Introduction

During 3GPP RAN4 #86-bis, the skeleton for TR 37.843 clause 10 conformance testing aspect was agreed. A number of contributions have been then approved for inclusion of specific testing methodologies in clause 10. This contribution is providing the measurement and calibration procedures along with MU assessment for OTA TAE in Near Field test Range.

# Discussion

Near Field is a testing methodology which can be used for radiated transmit power, EIRP measurement [1]. OTA TAE is the time difference between two signals, CRS0, and CRS1 carried on the same beam. It is a directional requirement so that the calibration of the system setup is not taken into account for determining the MU associated with the measurement. Calibration is only needed to set a proper SNR at the measurement equipment.

# Proposal

<< Unchanged sections omitted >>

<< Start of proposal >>

#### 10.2.6.4	Near Field

##### 10.2.6.4.1	General

The system is depicted in section 10.2.2.4.1. In case of OTA Frequency Error type of measurements, NF to FF transform is not needed. TAE is measured in Near Field for the declared direction.

##### 10.2.4.4.2 	Calibration

Stage 1 – Calibration:

Calibration shall be done with the procedure shown in section 10.2.2.4.2 to ensure that the SNR at the measurement equipment input is appropriate and the received signal level at the measurement equipment is within the dynamic range of measurement equipment.

##### 10.2.4.4.3 	Procedure

Stage 2 - Measurement:

The testing procedure consists of the following steps:

- Align the AAS BS with (Theta,Phi) angles corresponding to the declared beam direction to be measured

- Configure TX branch and carrier at a time according to the manufacturer's declared rated output power

- Set the AAS BS to transmit the test signal according to clause 6.6.3.4.2 of 3GPP TS 37.145-2 for UTRA and to clause 6.6.3.4.3 of 3GPP TS 37.145-2 for E-UTRA

- Measure Time alignment Error of each carrier arriving at the measurement equipment (such as a spectrum analyzer or equivalent instrument).

- Repeat steps 3-4 for all conformance test beam direction pairs as described in 3GPP TS 37.145-2 [24], subclause 6.2.

In order to guarantee a proper SNR level at the measurements receiver, for conformance tests TAE shall be measured at maximum power setting.

##### 10.2.4.4.4 	MU assessment

###### 10.2.4.4.4.1 	MU Budget

Same uncertainty budget format as for EVM type of measurements.

###### 10.2.4.4.4.2 	MU Value

Table 10.2.4.4.4.2-1: Near Field Test Range Uncertainty assessment for eAAS OTA TAE measurement

| UID | Uncertainty source |  | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≦ 4.2 GHz |  | Distribution of the probability |  | Divisor based on distribution shape |  | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Axes Intersection |  | 0 | 0 |  | Gaussian |  | 1.00 |  | 1 | FFS | FFS |  |
| 2 | Axes Orthogonality |  | 0 | 0 |  | Gaussian |  | 1.00 |  | 1 | FFS | FFS |  |
| 3 | Horizontal Pointing |  | 0 | 0 |  | Gaussian |  | 1.00 |  | 1 | FFS | FFS |  |
| 4 | Probe Vertical Position |  | 0 | 0 |  | Gaussian |  | 1.00 |  | 1 | FFS | FFS |  |
| 5 | Probe H/V pointing |  | 0 | 0 |  | Gaussian |  | 1.00 |  | 1 | FFS | FFS |  |
| 6 | Measurement Distance |  | 0 | 0 |  | Gaussian |  | 1.00 |  | 1 | FFS | FFS |  |
| 7 | Amplitude and Phase Drift |  | 0 | 0 |  | Gaussian |  | 1.00 |  | 1 | FFS | FFS |  |
| 8 | Amplitude and Phase Noise |  | 0.02 | 0.02 |  | Gaussian |  | 1.00 |  | 1 | 0.02 | 0.02 |  |
| 9 | Leakage and Crosstalk |  | 0 | 0 |  | Gaussian |  | 1.00 |  | 1 | 0 | 0 |  |
| 10 | Amplitude Non-Linearity |  | 0.04 | 0.04 |  | Gaussian |  | 1.00 |  | 1 | 0.04 | 0.04 |  |
| 11 | Amplitude and Phase Shift in rotary joints |  | 0 | 0 |  | Gaussian |  | 1.00 |  | 1 | 0 | 0 |  |
| 12 | Channel Balance Amplitude and Phase |  | 0 | 0 |  | Gaussian |  | 1.00 |  | 1 | 0 | 0 |  |
| 13 | Probe Polarization Amplitude and Phase |  | 0.0001 | 0.0001 |  | Gaussian |  | 1.00 |  | 1 | 0 | 0 |  |
| 14 | Probe Pattern Knowledge |  | 0 | 0 |  | Gaussian |  | 1.00 |  | 1 | 0 | 0 |  |
| 15 | Multiple Reflections |  | 0 | 0 |  | Gaussian |  | 1.00 |  | 1 | 0 | 0 |  |
| 16 | Room Scattering |  | 0.09 | 0.09 |  | Gaussian |  | 1.00 |  | 1 | 0.09 | 0.09 |  |
| 17 | DUT support Scattering |  | 0 | 0 |  | Gaussian |  | 1.00 |  | 1 | 0 | 0 |  |
| 18 | Positioning |  | 0.03 | 0.03 |  | Rectangular |  | 1.73 |  | 1 | 0.02 | 0.02 |  |
| 19 | Probe Array Uniformity |  | 0.055 | 0.055 |  | Gaussian |  | 1.00 |  | 1 | 0.06 | 0.06 |  |
| 20 | Uncertainty of the absolute gain of the probe antenna |  | 0 | 0 |  | Gaussian |  | 1.00 |  | 1 | 0 | 0 |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  |  |  |  | 0.12 |  | 0.12 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  |  |  |  | 0.23 |  | 0.23 |

The Near Field MU budget is carried out without consideration of the measurement equipment as this MU is given in μs. The way this uncertainty converts to dB is FFS.

<< end of proposal >>

# Conclusion

Text proposal to TR 37.843 is presented in order to add the test procedure and MU for OTA TAE measurement in a Near Field test range.

# Reference

[1] 3GPP TR 37.842 v 13.2.0, March 2017