3GPP TSG RAN WG4 Meeting #87	R4-1807660

Busan, South Korea, 21 – 25 May 2018

Source:		Rohde & Schwarz

Title:	Discussion on path delay tolerance for channel modelling

Agenda Item:		7.13.4.2

Document for:	Approval

# Introduction

At the last RAN4 meeting in Melbourne a discussion document on the tolerance for path delays in TDL channel models [1] was discussed. In this paper we continue the discussion and propose a solution to the previously discussed issues.

# Discussion

In our contribution [1] in the last RAN4 meeting we discussed the requirement of having a tolerance in the path delays when implementing the channel models in the test equipment.

Based on the feedback during online and offline discussions, it was made clear that it was not desirable to have a flexible delay grid  n*∆T with n∈N0 and

∆T≤1BW ,

where BW is the bandwidth of the faded signal, since it may lead to different channel models for different bandwidths.

Based on this feedback we have analysed the contributions in the last by other companies that discussed the performance requirements with regards to the channel bandwidth. An overview over the proposals [2] – [7] from the different companies can be seen in Table 1.

| Company | FR1 | FR2 |
| --- | --- | --- |
| Samsung | 50 MHz | 100 MHZ |
| NTT DOCOMO | 10/20 MHz; 50/100 MHz | 100/200 MHz |
| Qualcomm | 20 MHz; 40MHz | 100 MHz |
| Intel | 10/15/20 MHz |  |
| Ericsson | 20/40/60/80/100 MHz | 100/150/200 MHz |

Table 1 Overview of channel bandwidths for demodulation and CSI requirements

From the values in Table 1 it can be seen that the maximum channel bandwidth for demodulation and CSI requirements in FR1 is 100 MHz and for FR2 it is 200 MHz. Taking this into account  it is sufficient to require an equidistant delay  grid n*∆T with n∈N0 and

∆T≤1200 MHz .

Proposal 1: An equidistant delay grid n*∆T with n∈N0 and ∆T≤1200 MHz  is sufficient for emulating the channel models in the TE.

As already shown in the previous paper [1], this may lead to multiple taps in a TDL model to end up with the same delay. In case this occurs it is proposed to combine the powers of those taps into a single path by adding the respective powers, an example is shown in Figure 1.

Figure 1 Example of combining paths with the same delay [1]

Proposal 2: Paths that end up with the same delay will be combined into a single path by adding their respective powers.

# Proposals

# In this paper we continued to discuss the delay tolerance for channel modelling and propose feasible approach to implement those channel models in the TE. RAN4 is asked to endorse proposals 1 and 2 below.

Proposal 1: An equidistant delay grid n*∆T with n∈N0 and ∆T≤1200 MHz  is sufficient for emulating the channel models in the TE.

Proposal 2: Paths that end up with the same delay will be combined into a single path by adding their respective powers.

# References

- R4-1804845: Channel model: Tolerance for path delays; Rohde & Schwarz; RAN4#86bis Melbourne; April 2018

- R4-1805533: Summary of view on NR UE demodulation and CSI requirements, Ericsson; RAN4#86bis Melbourne; April 2018

- R4-1803843: Overview of UE performance requirements; Samsung; RAN4#86bis Melbourne; April 2018

- R4-1804006:	Proposals on NR UE performance test scenarios; Ericsson; RAN4#86bis Melbourne; April 2018

- R4-1804085:	Framework for Single-Carrier Demodulation Tests; Qualcomm; RAN4#86bis Melbourne; April 2018

- R4-1804172:	Views on NR UE performance requirements; Intel; RAN4#86bis Melbourne; April 2018

- R4-1804981:	Discussion on general assumption for UE demodulation requirements; NTT DOCOMO; RAN4#86bis Melbourne; April 2018