3GPP TSG-RAN WG4 Meeting #87													       R4- 1807704

Busan, Korea (Republic of), 21 - 25 May 2018

Source: 	Huawei, HiSilicon

Title: 					n50 A-MPR

Agenda Item: 			7.2.2.1.1

Document for:	Approval

# 1	Introduction

As discussed during the last RAN4 meeting and approved, the contribution [1] provided all technical definitions to be able to make the A-MPR studies to protect the MSService. Thus, A-MPR evaluation for n50 is conducted as defined just below:

- Proposal 1:

  - MSS protection level : “-0.8dBm/MHz EIRP in 1518-1520MHz” + “-30dBm/MHz EIRP in 1520-1559MHz”

- Proposal 2: A-MPR evaluation for n50 is conducted as

  - Definition of the study => Study the Tx only in the 1432-1517MHz band to protect MSS which is in 1518-1520-1559MHz band

  - Tx CBW [MHz] => 5, 10, 15, 20, 40, 50, 60

  - Protected Band [MHz] => 1518-1559MHz

This contribution presents the simulation results for the above scenarios.

# 2	Discussion

## 2.1	Simulation assumptions

A-MPR study for EESS protection has been defined as:

- Band range:

  - start:1432-1517MHz BW:5, 10, 15, 20, 40, 50, 60

- SCS:60M 30k, 15k for other BW

- SE limitation: MSS: “-0.8dBm/MHz EIRP in 1518-1520MHz” + “-30dBm/MHz EIRP in 1520-1559MHz”

- ACLR: EUTRA limit 30dB

SEM:

MPR:

Table 6.2.2-1 Maximum power reduction (MPR) for power class [2] and 3

| Modulation | MPR (dB) |  |
| --- | --- | --- |
|  | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM PI/2 BPSK | ≤ 0.5 | 0 |
| DFT-s-OFDM QPSK | ≤ 1 | 0 |
| DFT-s-OFDM 16 QAM | ≤ 2 | ≤ 1 |
| DFT-s-OFDM 64 QAM | ≤ 2.5 |  |
| DFT-s-OFDM 256 QAM | 4.5 |  |
| CP-OFDM QPSK | ≤ 3 | ≤ 1.5 |
| CP-OFDM 16 QAM | ≤ 3 | ≤ 2 |
| CP-OFDM 64 QAM | ≤ 3.5 |  |
| CP-OFDM 256 QAM | ≤ 6.5 |  |

## 2.2 	EESS protection

# 2	AMPR results for MSS

| BW | AMPR |  |  |  |
| --- | --- | --- | --- | --- |
|  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  |
|  | Outer | inner | Outer | Inner |
| 5MHz | 0 | 0.5 | 0 | -0.5 |
| 10MHz | 3 | 4 | 3.5 | 5 |
| 15MHz | 3 | 4 | 3.5 | 5 |
| 20MHz | 3 | 4.5 | 3.5 | 5.5 |
| 40MHz | 3.5 | 4.5 | 3.5 | 5 |
| 50MHz | 3 | 5 | 3 | 4.5 |
| 60MHz | 2.5 | 4 | 3 | 5 |

# 2	Figures for MSS

# Below, two A-MPR modulations are studied. At the end of each paragraph, there is a table to summary the A-MPR which need to implement to protect MSService.

## 2.3 	DFT-S-OFDM

Just below, the table 1 is the summary of the A-MPR needed to protect MSService in the case of DFT-S-OFDM modulation.

Table 2: A-MPR for "NS_5" for DFT-S-OFDM case

| Channel bandwidth [MHz] | Parameters | Region A | Region B | Region C |
| --- | --- | --- | --- | --- |
| 5 | RBstart | 0 – 4 | 5 – 11 | 12 – 25 |
|  | LCRB [RBs] | 5 – 25 | 10 – 25 | 2 – 25 |
|  | A-MPR [dB] | ≤ 0.5 | ≤ 0.5 | ≤ 0.5 |
| 10 | RBstart | 0 – 11 | 12 - 15 |  |
|  | LCRB [RBs] | 0 – 27 | 0 - 30 |  |
|  | A-MPR [dB] | ≤ 4 | ≤ 4 |  |
| 15 | RBstart | 0 – 20 | 21 – 30 |  |
|  | LCRB [RBs] | 0 – 25 | 0 – 40 |  |
|  | A-MPR [dB] | ≤ 4 | ≤ 4 |  |
| 20 | RBstart | 27 – 35 |  |  |
|  | LCRB [RBs] | 0 – 40 |  |  |
|  | A-MPR [dB] | ≤ 4 |  |  |
| 40 | RBstart | 70 - 80 |  |  |
|  | LCRB [RBs] | 0 - 90 |  |  |
|  | A-MPR [dB] | ≤ 4.5 |  |  |
| 50 | RBstart | 75 - 110 | 111 - 125 |  |
|  | LCRB [RBs] | 0 – 120 | 0 – 140 |  |
|  | A-MPR [dB] | ≤ 4.5 | ≤ 4.5 |  |
| 60 | RBstart | 55 – 75 |  |  |
|  | LCRB [RBs] | 0 – 70 |  |  |
|  | A-MPR [dB] | ≤ 4 |  |  |

## 2.3 	CP-OFDM

Just below, the table 2 is the summary of the A-MPR needed to protect MSService in the case of the CP-OFDM nodulation.

Table : A-MPR for "NS_6" for CP-OFDM case

| Channel bandwidth [MHz] | Parameters | Region A | Region B | Region C |
| --- | --- | --- | --- | --- |
| 5 | RBstart |  |  |  |
|  | LCRB [RBs] |  |  |  |
|  | A-MPR [dB] |  |  |  |
| 10 | RBstart | 0 – 11 | 12 – 20 |  |
|  | LCRB [RBs] | 0 | 0 – 20 |  |
|  | A-MPR [dB] | NA | ≤ 5 |  |
| 15 | RBstart | 0 – 20 | 21 – 30 |  |
|  | LCRB [RBs] | 0 | 0 – 25 |  |
|  | A-MPR [dB] | NA | ≤ 5 |  |
| 20 | RBstart | 0 – 29 | 30 – 45 |  |
|  | LCRB [RBs] | 0 | 0 – 38 |  |
|  | A-MPR [dB] | NA | ≤ 6 |  |
| 40 | RBstart | 0 – 69 | 70 – 90 |  |
|  | LCRB [RBs] | 0 | 0 - 70 |  |
|  | A-MPR [dB] | NA | ≤ 6 |  |
| 50 | RBstart | 0 – 89 | 90 – 110 |  |
|  | LCRB [RBs] | 0 | 0 - 120 |  |
|  | A-MPR [dB] | NA | ≤ 5 |  |
| 60 | RBstart | 0 – 54 | 55 – 65 | 65 – 70 |
|  | LCRB [RBs] | 0 | 0 – 55 | 0 – 90 |
|  | A-MPR [dB] | NA | ≤ 5 | ≤ 5 |

# 3	Conclusion

In this contribution we have presented NR A-MPR back off for Band n50. As the difference between DFT-S-OFDM modulation and CP-OFDM modulation is small and that the worst case is the DFT-S-OFDM modulation excluding the region A and the channel bandwidth 60MHz, thus, it is proposed:

Proposal 1: take into account only the DFT-S-OFDM modulation and the worst case for the region A and the channel 60MHz,

Proposal 2: The tables 3 & 4 below propose protection A-MPR levels for MSS. These tables can be the reference tables for A-MPR parameters in the TS to protect MSS in the band 1518-1559MHz:

Table 3: A-MPR for "NS_6"

| Channel bandwidth [MHz] | Parameters | Region A | Region B | Region C |
| --- | --- | --- | --- | --- |
| 5 | RBstart | 0 – 4 | 5 – 11 | 12 – 25 |
|  | LCRB [RBs] | 0 | 0 | 2 – 25 |
|  | A-MPR [dB] | NA | NA | ≤ 0.5 |
| 10 | RBstart | 0 – 11 | 12 – 20 |  |
|  | LCRB [RBs] | 0 | 0 |  |
|  | A-MPR [dB] | NA | NA |  |
| 15 | RBstart | 0 – 20 | 21 – 30 |  |
|  | LCRB [RBs] | 0 | 0 – 40 |  |
|  | A-MPR [dB] | NA | ≤ 4 |  |
| 20 | RBstart | 0 – 26 | 27 – 35 |  |
|  | LCRB [RBs] | 0 | 0 – 40 |  |
|  | A-MPR [dB] | NA | ≤ 4 |  |
| 40 | RBstart | 0 – 69 | 70 - 80 |  |
|  | LCRB [RBs] | 0 | 0 - 90 |  |
|  | A-MPR [dB] | NA | ≤ 4.5 |  |
| 50 | RBstart | 0 – 89 | 90 - 110 | 111 - 125 |
|  | LCRB [RBs] | 0 | 0 – 120 | 0 – 140 |
|  | A-MPR [dB] | NA | ≤ 4.5 | ≤ 4.5 |
| 60 | RBstart | 0 – 54 | 55 – 65 | 65 – 75 |
|  | LCRB [RBs] | 0 | 0 – 55 | 0 – 90 |
|  | A-MPR [dB] | NA | ≤ 5 | ≤ 5 |

Table 4:  A-MPR result for EESS

| BW | AMPR |  |  |  |
| --- | --- | --- | --- | --- |
|  | DFT-s-OFDM QPSK |  | CP-OFDM QPSK |  |
|  | Outer | inner | Outer | Inner |
| 5MHz | 0 | 0.5 | 0 | -0.5 |
| 10MHz | 3 | 4 | 3.5 | 5 |
| 15MHz | 3 | 4 | 3.5 | 5 |
| 20MHz | 3 | 4.5 | 3.5 | 5.5 |
| 40MHz | 3.5 | 4.5 | 3.5 | 5 |
| 50MHz | 3 | 5 | 3 | 4.5 |
| 60MHz | 2.5 | 4 | 3 | 5 |

# 4	Reference

[1] R4-1805727, WF on NR A-MPR evaluation scenarios for n50, n51, and n74, NTT DOCOMO, INC., Huawei, Etisalat, RAN4#86bis