3GPP TSG-RAN WG4 Meeting #87	R4-1807724

## Busan, Korea, 21 - 25 May, 2018

Source: 	Huawei

Title:	Consideration of the OTA REFSENS sensitivity in the Rel-15 OTA AAS BS declarations

Agenda Item:	6.26.3.1.8 (eAAS BS: Declarations)

Document for:	Discussion and Approval

# Introduction

In this contribution the OTA sensitivity and OTA REFSENS sensitivity terminology is discussed with the solution for the related declarations proposed.

This contribution is a resubmission from RAN4#86bis (R4-1804910, not treated).

# Discussion

In Rel-13, EIS sensitivity requirements was introduced based on set of the manufacturers declarations. This sensitivity is based on the declared values of the Minimum EIS (D10.6), which incorporated the antenna array characteristics and is not direct OTA translation of the legacy conducted sensitivity requirements.

Set of EIS requirement related declarations is presented below for reference.

Table 1: OTA sensitivity related declarations [TS 37.145-2, Rel-13/14]

| Declaration identifier | Declaration |
| --- | --- |
| D10.1 | OSDD identifier |
| D10.2 | OSDD operating band support |
| D10.3 | OSDD RAT support |
| D10.4 | OTA sensitivity E-UTRA supported channel BW |
| D10.5 | Redirection of receiver target support |
| D10.6 | Minimum EIS |
| D10.7 | Receiver target reference direction Sensitivity Range of Angle of Arrival |
| D10.8 | Receiver target redirection range |
| D10.9 | Receiver target reference direction |
| D10.10 | Conformance test directions sensitivity RoAoA |
| D10.11 | Conformance test directions |

In Rel-15, additional sensitivity requirement was introduced for OTA AAS BS, i.e. OTA REFSENS sensitivity level.

The following was observed for OTA REFSENS requirement based on TS 37.105 and TR 37.843 text:

| Specification text extract | Observation |
| --- | --- |
| TS37.105: “The OTA REFSENS requirement is intended to ensure the OTA reference sensitivity level for a declared OTA REFSENS RoAoA.” | OTA REFSENS RoAoA declaration needed |
| TS 37.843: “Conformance testing for OTA reference sensitivity is performed for the following five directions. - The receiver target reference direction. - The direction determined by the maximum  value achievable inside the OTA REFSENS RoAoA maintaining the receiver target reference direction  value. - The direction determined by the minimum  value achievable inside the OTA REFSENS RoAoA maintaining the receiver target reference direction  value. - The direction determined by the maximum  value achievable inside the OTA REFSENS RoAoA maintaining the receiver target reference direction  value. - The direction determined by the minimum  value achievable inside the OTA REFSENS RoAoA maintaining the receiver target reference direction  value. The manufacturer declares the five directions for conformance testing.” | Receiver target reference direction declaration needed also for OTA RENSENS Four conformance test directions declaration needed also for OTA RENSENS RoAoA As the above two are already existing for the OTA sensitivity requirement, it is proposed to call new OTA RENSENS related declarations as: - OTA REFSENS receiver target reference direction - OTA REFSENS conformance test directions |

# Conclusion

Based on the above analysis, the following is proposed:

Proposal 1: Agree on the introduction of the new declaration for OTA REFSENS requirement:

- OTA REFSENS RoAoA

- OTA REFSENS receiver target reference direction

- OTA REFSENS conformance test directions

# References

[1]	R4-1802931	New manufacturer declarations for Rel-15 OTA AAS BS	Huawei

[2]	R4-1804914	Draft CR to TS 37.145-2: manufacturers declarations (4.10), Rel-15