3GPP TSG-RAN WG4 Meeting #87	R4-1807728

## Busan, Korea, 21 - 25 May, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-2 | CR | CRNum | rev | - | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 37.145-2: manufacturers declarations (4.10), Rel-15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-05-05 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This Cat. B CR introduces Rel-15 AAS BS declarations based on the Rel-13/14 baseline. For Rel-15 specification the OTA AAS BS terminology was considered and clarification text is introduced in subclause 4.10, based on discussion in R4-1804910, R4-1804911, R4-1804912. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarifiaction text in 4.10 added on the relations among the hybrid AAS BS and OTA AAS BS declarations. - New table 4.10-2 added for the new Rel-15 OTA AAS BS specific declarations. - D9.8: the EIRP accuracy directions set(s) used in the Rel-13/14 specifications was renamed into “OTA peak directions set(s)” as they are now used also for other requirements. Refer to TR 37.843 for background. - Addition of declarations based on R4-1804910 for OTA REFSENS: Dx.29 – Dx.31 - Addition of declarations based on R4-1804911: Dx.1 – Dx.6 - Addition of declarations based on R4-1804912: D.7 – Dx.28 Additional revisions were introduced (with new ID) on top of the Draft CR version submitted to RAN4#86bis meeting: - New declaration for “AAS BS requirements set” for OTA AAS BS - Few Rel-15 declaratrion proposals were removed based on discussion in separate contribution, e.g. Dx.2, Dx.4, Dx.5 - Dx.9 – Dx.12, Dx.26 – Dx.28: text aligned with the Rel-13 CR agreed during RAN4#86bis |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No conducted manufacturer’s declarations for Rel-15 OTA AAS BS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

## 4.10	Manufacturer declarations

The AAS BS declarations categories D9.x and D10.x listed in table 4.10-1 are required to be provided by the manufacturer for the radiated requirements testing of the hybrid AAS BS or the OTA AAS BS.

For the hybrid AAS BS declarations required for the conducted requirements testing, refer to 3GPP TS 37.145-1 [x], subclause 4.10.

For OTA AAS BS additional declarations are introduced from Rel-15 in table 4.10-2, in order to easily distinguish from the Rel-13/14 OTA declarations which are also applicable for hybrid AAS BS. Declarations in table 4.10-2 are applicable to OTA AAS BS only.

Table 4.10-1: Hybrid AAS BS and OTA AAS BS manufacturers declarations for radiated test requirements

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D9.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the AAS BS enclosure. |
| D9.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the AAS BS enclosure. |
| D9.3 | Beam identifier | A unique title to identify a beam. e.g. a, b, c or 1,2,3. The vendor may declare any number of beams, the minimum requirement to declare for conformance are the beams with the highest intend EIRP for each of the beams widths below: 1) Narrowest intended BeWθ, narrowest intended BeWϕ (possible when narrowest intended BeWθ) at the reference beam direction. 2) Narrowest intended BeWϕ, narrowest intended BeWθ (possible when narrowest intended BeWϕ) at the reference beam direction. 3) Widest intended BeWθ , widest intended BeWϕ (possible when widest intended BeWθ at the reference beam direction. 4) Widest intended BeWϕ – widest intended BeWθ (possible when widest intended BeWϕ) at the reference beam direction. 5) BeWθ and BeWϕ which provide highest intended EIRP of all possible beams at the reference beam direction. NOTE 1: Depending on the capability of the system some of these beams may be the same. When selecting the above five beam widths for declaration, all beams that the AAS BS is intended to produce shall be considered, including beams that during operation may be identified by any kind of cell or UE specific reference signals, with the exception of any type of beam that is created from a group of transmitters that are not all phase synchronised. |
| D9.4 | Beam operating band support | UTRA and/or E-UTRA operating band(s) supported by the beam, declared for every beam identified in D9.3. |
| D9.5 | Beam RAT support | RAT(s) supported by each beam for each supported operating band, declared for every beam identified in D9.3. |
| D9.6 | Beam E-UTRA channel band width support | E-UTRA channel bandwidth supported for each beam and supported operating band identified in D9.3. |
| D9.7 | Reference beam direction pair | The beam direction pair, describing the reference beam peak direction and the reference beam centre direction. Declared for every beam |
| D9.8 | OTA peak directions set | The OTA peak directions set for each beam. Declared for every beam identified in D9.3. NOTE 1a: In Rel-13/14 version of this specification, this declaration was called EIRP accuracy directions set. |
| D9.9 | Maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam identified in D9.3. |
| D9.10 | Rated beam EIRP | The rated EIRP level per carrier at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D9.9), as well as the reference beam direction pair (D9.7). Declared for every beam identified in D9.3. |
| D9.11 | Beamwidth | The beamwidth for the reference beam direction pair and the four maximum steering directions. Declared for every beam identified in D9.3. |
| D9.12 | Equivalent beams | List of beams which are declared to be equivalent. Equivalent beams imply that the beams are expected to have identical OTA peak directions sets and intended to have identical spatial properties at all steering directions within the OTA peak directions set when presented with identical signals. All declarations (D9.4-D9.11) made for the beams are identical and the transmitter unit and associated TAB connector(s), RDN and antenna array responsible for generating the beam are of identical design. |
| D9.13 | Parallel beams | List of beams which have been declared equivalent (D9.12) and can be generated in parallel using independent RF power resources. Independent power resources means that the beams are transmitted from mutually exclusive transmitter units. |
| D9.14 | Number of carriers at maximum EIRP | The number of carriers per operating band the AAS BS is capable of generating at maximum EIRP declared each RAT (and multi-RAT) for every beam identified in D9.3. |
| D9.15 | Multi-band transceiver units | Declared if an operating band is generated using Transceiver units supporting operation in multiple operating bands through common active electronic components(s). |
| D9.16 | Operating bands with multi-band dependencies | List operating bands which are generated by multi-band transceiver units. Declared for each operating band for which multi-band transceiver units (D9.15) have been declared, |
| D9.15 | Maximum radiated Base Station RF Bandwidth | Largest Base Station RF Bandwidth in the operating band, declared for each supported operating band identified in D9.4. |
| D9.18 | Maximum radiated Base Station RF Bandwidth for contiguous operation. | Largest Base Station RF Bandwidth for contiguous spectrum operation, declared for each supported operating band identified in D9.4. |
| D9.19 | Maximum radiated Base Station RF Bandwidth for non- contiguous operation. | Largest Base Station RF Bandwidth for non-contiguous spectrum operation, declared for each supported operating band identified in D9.4. |
| D9.20 | Inter-band CA bands | Declared inter-band CA bands supported per operating band identified in D9.4. |
| D9.21 | CA only operation | Declared per operating band identified in D9.4. |
| D9.22 | Multi-carrier HSPA only operation | Declared per each supported UTRA operating band, if declared in D9.4. |
| D9.23 | Reduced number of supported carriers at maximum EIRP in multi-RAT operations | Declared for each supported operating band identified in D9.4. |
| D9.24 | Reduced maximum EIRP at the total number of supported carriers in multi-RAT operations | Declared for each supported operating band identified in D9.4. |
| D9.25 |  |  |
| D9.26 | Maximum Radio Bandwidth of the operating band with multi-band dependencies | Largest Radio Bandwidth that can be supported by the operating bands with multi-band dependencies. Declared for each supported operating band which has multi-band dependencies (D9.16) |
| D9.27 | Total number of supported carriers for operating bands with multi-band dependencies | Total number of supported carriers for operating bands declared to have multi-band dependencies (D9.16). |
| D9.28 | OTA contiguous or non-contiguous spectrum support | Ability of AAS BS to support contiguous or non-contiguous (or both) frequency distribution of carriers when operating multi-carrier in an operating band. |
| D9.29 | OTA contiguous and non-contiguous parameters identical | Contiguous and non-contiguous operation is possible then parameters are the same. |
| D10.1 | OSDD: identifier | A unique identifier for the OSDD. |
| D10.2 | OSDD :operating band support | Operating band supported by the OSDD, declared for every OSDD identified in D10.1. NOTE 2: As each identified OSDD has a declared minimum EIS value (D10.6), multiple operating band can be only be declared if they have the same minimum EIS declaration. |
| D10.3 | OSDD :RAT support | RAT(s) supported by the OSDD for each supported operating band, declared for every OSDD identified in D10.1. NOTE 3: If the OSDD supports multiple RAT's with different minimum EIS value (D10.6) if all other parameters are the same then different EIS values for different RATS and signal BW's may be declared for an OSDD. |
| D10.4 | OTA sensitivity E-UTRA supported channel BW | The E-UTRA channel BW's supported by each OSDD. |
| D10.5 | Redirection of receiver target support | Ability to redirect the receiver target related to the OSDD |
| D10.6 | Minimum EIS | The minimum EIS requirement (i.e. maximum allowable EIS value) applicable to all sensitivity RoAoA per OSDD. Declared for every supported RAT and E-UTRA supported channel BW for the OSDD. Declared per OSDD identified in D10.1. The lowest EIS value for all the declared OSDD’s is called minSENS, while its related range of angles of arrival is called minSENS RoAoA. NOTE 4: If the AAS BS is not capable of redirecting the receiver target related to the OSDD then there is only one RoAoA applicable to the OSDD. |
| D10.7 | Receiver target reference direction Sensitivity Range of Angle of Arrival | The sensitivity RoAoA associated with the receiver target reference direction (D10.9) for each OSDD. |
| D10.8 | Receiver target redirection range | For each OSDD the associated union of all the sensitivity RoAoA achievable through redirecting the receiver target related to the OSDD |
| D10.9 | Receiver target reference direction | For each OSDD an associated direction inside the receiver target redirection range (D10.8). NOTE 5: For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA. |
| D10.10 | Conformance test directions sensitivity RoAoA | For each OSDD that includes a receiver target redirection range, four sensitivity RoAoA comprising the conformance test directions (D10.11). |
| D10.11 | Conformance test directions | For each OSDD four conformance test directions. If the OSDD includes a receiver target redirection range the following four directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. If an OSDD does not include a receiver target redirection range the following 4 directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. |

Table 4.10-2: OTA AAS BS manufacturers declarations for radiated test requirements

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| Dx.1 | The rated carrier OTA BS power, Pmax,c,TRP | Pmax,c,TRP, is declared as TRP OTA power per carrier, declared per supported operating band, per supported RAT. |
| Dx.2 | AAS BS requirements set | Declaration of either hybrid AAS BS architecture conforming to the hybrid requirement set, or OTA AAS BS architecture conforming to the OTA requirement set. |
| Dx.3 | OTA coverage range | Declared as OTA AAS BS common range of directions within which selected TX OTA requirements are intended to be met. [add description based on Conformance test directions declaration] NOTE: OTA coverage range is used for conformance testing of such OTA Tx requirements as occupied bandwidth, frequency error, TAE or EVM. |
| Dx.6 | Equivalent passive antenna gain | Declared value of the equivalent gain (to the OTA AAS BS) of the passive antenna system, used for translation of regional regulatory requirements for unwanted emissions (i.e. EIRP value using an assumed passive antenna gain). |
| Dx.0 | AAS BS capability sets (CSA) | [to be the same as the D6.12 CSA declaration in Rel15 version of TS37.145-1 for the hybrid AAS BS] |
| Dx.7 | Spurious emission category | [To be defined based on its conduced version in D6.3] |
| Dx.8 | Geographic area support | [To be defined based on its conduced version in D6.4] |
| Dx.9 | Band 20 or Band XX support, operating in geographical areas allocated to broadcasting (DTT) | [To be defined based on its conduced version in D6.5] |
| Dx.10 | Band 20 or Band XX support, emission level for channel N (PEM,N) | [To be defined based on its conduced version in D6.6] |
| Dx.11 | Band 20 or Band XX support, Maximum output Power in 10 MHz (P10MHz) | [To be defined based on its conduced version in D6.7] |
| Dx.12 | Band 32 or Band XXXII support, Declared emission level in Band 32/XXXII (PEM,B32,ind) | [To be defined based on its conduced version in D6.8] |
| Dx.13 | Co-existence with other systems | [To be defined based on its conduced version in D6.10] |
| Dx.14 | Co-location with other base stations | [To be defined based on its conduced version in D6.11] |
| Dx.15 | Single-band RIB or multi-band RIB | [To be defined based on its conduced version in D6.13] |
| Dx.16 | Single or multiple carrier | [To be defined based on its conduced version in D6.23] |
| Dx.17 | Maximum number of supported carriers per band | [To be defined based on its conduced version in D6.24] |
| Dx.18 | Total maximum number of supported carriers | [To be defined based on its conduced version in D6.25] |
| Dx.19 | Other band combination multi-band restrictions | [To be defined based on its conduced version in D6.28] |
| Dx.20 | Ncells | [To be defined based on its conduced version in D6.38] |
| Dx.21 | Maximum supported power difference between carriers | [To be defined based on its conduced version in D6.39] |
| Dx.22 | Maximum supported power difference between carriers is different operating bands | [To be defined based on its conduced version in D6.40] |
| Dx.23 | OTA AAS BS operating band combination support | [To be defined based on its conduced version in D6.41] |
| Dx.24 | UTRA FDD MIMO support | [To be defined based on its conduced version in D6.51] |
| Dx.25 | UTRA Inner loop power control dynamic range | [To be defined based on its conduced version in D6.57] |
| Dx.26 | Inter-band CA or HSDPA bands | [To be defined based on its conduced version in D6.59] |
| Dx.27 | Intra-band contiguous CA or HSDPA | [To be defined based on its conduced version in D6.60] |
| Dx.28 | Intra-band non-contiguous CA or HSDPA | [To be defined based on its conduced version in D6.61] |
| Dx.29 | OTA REFSENS RoAoA |  |
| Dx.30 | OTA REFSENS receiver target reference direction | [To be defined based on D10.9] |
| Dx.31 | OTA REFSENS conformance test directions | [To be defined based on D10.11] |