3GPP TSG-RAN WG4 Meeting #87	R4-1807741

## Busan, Korea, 21 - 25 May, 2018

Source: 	Huawei

Title:	TP to TS 38.141-1: conducted manufacturers declarations for NR BS (4.6)

Agenda Item:	7.8.3 (Conducted conformance testing - 38.141-1)

Document for:	Approval

# Introduction

Based on discussion in [2], in this contribution we are providing TP to the TS 38.141-1 for the NR BS manufacturer declarations for conducted test requirements.

# Conclusions

Based on the above introduction, the following proposal is formulated:

Proposal 1:  approve the attached TP to TS 38.141-1 for the NR BS manufacturer declarations for conducted test requirements.

# References

[1] 3GPP TS 38.141-1, shared on RAN4 reflector after RAN4#86bis

[2] R4-1807740	Discussion on manufacturer’s declarations for NR BS conducted testing

# TP to TS 38.141-1

------------------------------ Modified section ------------------------------

## 4.6	Manufacturer declarations

The following NR BS declarations listed in table 4.10-1 are required to be provided by the manufacturer for the conducted requirements testing of the BS type 1-C and BS type 1-H.

For the BS type 1-H declarations required for the radiated requirements testing, refer to 3GPP TS 38.141-2 [x].

Table 4.6-1 Manufacturer declarations for NR BS conducted test requirements

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1-C | 1-H |
| D6.1 | NR operating bands | NR operating bands supported by each single band connector or multi-band connector. | x | x |
| D6.2 | BS requirements set | Declaration of one of the NR base station requirement’s set as defined for BS type 1-C, BS type 1-H, BS type 1-O, and BS type 2-O. | x | x |
| D6.3 | Spurious emission category | Declare the BS spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [x]. | x | x |
| D6.4 | Geographic area support | The manufacturer shall declare the regions the BS may operate in. e.g. CEPT. | x | x |
| D6.5 | [Band n20 support, operating in geographical areas allocated to broadcasting (DTT)] | Editor’s note: this declaration is subject to technical discussion on the applicability of the DTT protection from NR BS. [If the BS has single band connector(s) or multi-band connector(s) declared to support Band n20 the manufacturer shall declare if the BS may operate in geographical areas allocated to broadcasting (DTT).] | [x] | [x] |
| D6.6 | [Band n20 support, emission level for channel N (PEM,N)] | Editor’s note: this declaration is subject to technical discussion on the applicability of the DTT protection from NR BS. [If the BS has sin single band connector(s) or multi-band connector(s) declared to support Band n20 and has been declared to operate in geographical areas allocated to broadcasting (DTT), the emission level for channel N (Annex G of 3GPP TS 36.104 [x]) shall be declared.] | [x] | [x] |
| D6.7 | [Band n20 support, Maximum output Power in [10] MHz (P10MHz)] | Editor’s note: this declaration is subject to technical discussion on the applicability of the DTT protection from NR BS. [If the BS has single band connector(s) or multi-band connector(s) declared to support Band n20 and has been declared to operate in geographical areas allocated to broadcasting (DTT), the maximum output power in [10] MHz (Annex G of 3GPP TS 36.104 [11]) shall be declared.] | [x] | [x] |
| D6.10 | Co-existence with other systems | The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA, PHS and/or NR operating in another band are deployed. | x | x |
| D6.11 | Co-location with other base stations | The manufacturer shall declare whether the BS under test is intended to operate co-located with Base Stations of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or NR operating in another band. | x | x |
| D6.12 | Conducted NR capability set (CSNR) | The manufacturer shall declare the supported NR capability set(s) according to table [conducted NR capability sets in TS38.141-1] for all single band connector(s) or multi-band connector(s) and supported operating band. | x | x |
| D6.13 | Single band connector or multi-band connector | Declaration of the single band or multi-band capability of single band connector(s) or multi-band connector(s), declared for every connector. | x | x |
| D6.14 | Contiguous or non-contiguous spectrum | Editor’s note: MSR related Ability to support contiguous or non-contiguous (or both) frequency distribution of carriers when operating multi-carrier, per single band connector or multi-band connector, per operating band, per RAT. | x | x |
| D6.15 | Contiguous and non-contiguous parameters identical | If contiguous and non-contiguous operation is possible then parameters are the same. | x | x |
| D6.16 | Maximum Radio Bandwidth of the multi-band connector | Largest radio bandwidth that can be supported by the multi-band connector. May be different for transmit and receive. Declared for each supported operating band and operating band combination (D6.41) supported for every multi-band connector. | x | x |
| D6.17 | Maximum Base Station RF Bandwidth | Largest Base Station RF Bandwidth in the operating band, declared for each supported operating band for every single band connector or multi-band connector. | x | x |
| D6.18 | Maximum Base Station RF Bandwidth for contiguous operation | Largest Base Station RF Bandwidth for contiguous spectrum operation, declared for each supported operating band and CSNR for every single band connector or multi-band connector. | x | x |
| D6.19 | Maximum Base Station RF Bandwidth for non- contiguous operation | Largest Base Station RF Bandwidth for non-contiguous spectrum operation, declared for each supported operating band and CSNR for every single band connector or multi-band connector. | x | x |
| D6.20 | NR supported channel bandwidths and SCS | NR channel bandwidth and SCS supported for each supported operating band for every single band connector or multi-band connector. | x | x |
| D6.21 | single band connector / multi-band connector supported operating bands | List of operating bands and band combinations supported by each single band connector or multi-band connector. | x | x |
| D6.22 | CA only operation | Declaration of CA-only operation, declared per single band connector or multi-band connector. | x | x |
| D6.23 | Single or multiple carrier | Capable of operating with a single carrier (only) or multiple carriers per operating band, per RAT, for all single band connector(s) or multi-band connector(s). | x | x |
| D6.24 | Maximum number of supported carriers per operating band | Maximum number of supported carriers per supported operation band, declared per operating band, per RAT, for all single band connector(s) or multi-band connector(s). | x | x |
| D6.25 | Total maximum number of supported carriers | Maximum number of supported carriers for all supported operating bands, declared for all single band connector(s) or multi-band connector(s). | x | x |
| D6.26 | Reduced number of supported carriers at the rated total output power in multi-RAT operations | Editor’s note: MSR related Declared for each supported operating band for all single band connector(s) or multi-band connector(s). | x | x |
| D6.27 | Reduced total output power at the total number of supported carriers in multi-RAT operations | Editor’s note: MSR related Declared for each supported operating band for all single band connector(s) or multi-band connector(s). | x | x |
| D6.28 | Other band combination multi-band restrictions | Declare any other limitations under simultaneous operation in the declared band combinations (D6.41) for each multi-band connector which have any impact on the test configuration generation. Declared for every multi-band connector. | x | x |
| D6.30 | Rated carrier output power (Prated,c,AC, or Prated,c,TABC) | Editor’s note: MSR related Conducted rated carrier output power, per single band connector or multi-band connector. Declared per supported operating band, per supported RAT, for all single band connector(s) or multi-band connector(s). | x | x |
| D6.31 | Rated carrier output power for contiguous spectrum operation | Conducted rated carrier output power for contiguous spectrum operation, per single band connector or multi-band connector. Declared for each supported operating band and CSNR, for all single band connector(s) or multi-band connector(s). | x | x |
| D6.32 | Rated carrier output power for non-contiguous spectrum operation | Conducted rated carrier output power for non-contiguous spectrum operation, per single band connector or multi-band connector. Declared for each supported operating band and CS, for all single band connector(s) or multi-band connector(s). | x | x |
| D6.33 | [Rated output power per RAT (Prated,RAT,TABC)] | Editor’s note: MSR related; leave FFS to see it “per RAT” rated power declarations and definitions (Prated,RAT,AC, Prated,RAT,TABC) is necessary. [PRated,RAT,TABC is declared per supported operating band, per supported RAT for all single band connector(s) or multi-band connector(s).] | [x] | [x] |
| D6.34 | Rated total output power (Prated,t,AC, or Prated,t,TABC) | Conducted total rated output power, per single band connector or multi-band connector. Declared for supported operating band, for all single band connector(s) or multi-band connector(s). For multi-band connectors declared for each supported operating band in each supported band combination. | x | x |
| D6.35 | Rated total output power for contiguous spectrum operation | Conducted total rated output power for contiguous spectrum operation, per single band connector or multi-band connector. Declared for each supported operating band and CSNR, for all single band connector(s) or multi-band connector(s). | x | x |
| D6.36 | Rated total output power for non-contiguous spectrum operation | Conducted total rated output power for non-contiguous spectrum operation, per single band connector or multi-band connector. Declared for each supported operating band and CSNR, for all single band connector(s) or multi-band connector(s). | x | x |
| D6.37 | [Rated multi-band total output power, Prated,MB,TABC] | Editor’s note: FFS until the needs for Prated,MB,TABC in the specification is confirmed. [Conducted multi-band rated total output power, per multi-band connector. Declared for all declared operating band combinations for every multi-band connector.] | [x] | [x] |
| D6.38 | Ncells | Number corresponding to the minimum number of cells that can be transmitted by a BS in a particular operating band with transmission on all TAB connectors supporting the operating band. |  | x |
| D6.39 | Maximum supported power difference between carriers | Maximum supported power difference between carriers in each supported operating band, declared for all single band connector(s) or multi-band connector(s). | x | x |
| D6.40 | Maximum supported power difference between carriers is different operating bands | Supported power difference between any two carriers in any two different supported operating bands, for all declared operating band combination, declared for every multi-band connector. | x | x |
| D6.41 | Operating band combination support | List of operational bands combinations supported by the BS. | x | x |
| D6.42 | Total number of supported carriers for the declared band combinations of BS | Total number of supported carriers for the declared band combinations (D6.41) of the BS. | x | x |
| D6.43 | Intra-system interfering signal declaration list | List of single band connector(s) or multi-band connector(s) for which an intra-system interfering signal level is required to be declared. Declaration is required if the intra-system interfering signal level is larger than the co-location interfering signal level. | x | x |
| D6.44 | Intra-system interfering signal level | The interfering signal level in dBm per single band connector(s) or multi-band connector(s) declared for each supported operating band, for all single band connector(s) or multi-band connector(s) covered by D6.43. | x | x |
| D6.54 | [DL RS transmission groups] | [Groups of single band connector(s) or multi-band connector(s) which are declared to transmit DL RS. Declared per operating band.] | x | x |
| D6.57 |  |  |  |  |
| D6.58 | TAE groups | Set of declared TAB connector beam forming groups on which the TAE requirements apply. All TAB connectors belong to at least one TAB connector beam forming group (even if it's a TAB connector beam forming group consisting of one connector). The smallest possible number of TAB connector beam forming groups need to be declared such that there is no TAB connector not contained in at least one of the declared TAB connector beam forming groups. Declared for each supported RAT and operating band. |  | x |
| D6.59 | Inter-band CA | Band combinations declared to support inter-band CA (per CA capable multi-band connector(s), as in D6.22). Declared for every multi-band connector which support CA. | x | x |
| D6.60 | Intra-band contiguous CA | Bands declared to support intra-band contiguous CA (per CA capable single band connector(s) or multi-band connector(s), as in D6.22). Declared for every single band connector(s) or multi-band connector(s). | x | x |
| D6.61 | Intra-band non-contiguous | Bands declared to support intra-band non-contiguous CA (per CA capable single band connector(s) or multi-band connector(s), as in D6.22). Declared for every single band connector(s) or multi-band connector(s). | x | x |
| D6.70 | Equivalent connectors | List of single band connector(s) or multi-band connector(s), which have been declared equivalent. Equivalent connectors imply that the single band connector(s) or multi-band connector(s), are expected to behave in the same way when presented with identical signals under the same operating conditions. All declarations made for the single band connector(s) or multi-band connector(s) are identical and the transmitter unit and/or receiver unit driving the single band connector/ multi-band connector are of identical design. | x | x |
| D6.71 | BS class | BS class of the BS, declared as Wide Area BS, Medium Range BS, or Local Area BS. | x | x |
| D6.72 | TAB connector RX min cell group | Declared as a group of TAB connectors to which RX requirements are applied. This declaration corresponds to group of TAB connectors which are responsible for receiving a cell when the BS type 1-H setting corresponding to the declared minimum number of cells (Ncells) with transmission on all TAB connectors supporting an operating band. |  | x |
| D6.73 | TAB connector TX min cell group | Declared group of TAB connectors to which TX requirements are applied. This declaration corresponds to group of TAB connectors which are responsible for transmitting a cell when the BS type 1-H setting corresponding to the declared minimum number of cells (Ncells) with transmission on all TAB connectors supporting an operating band. |  | x |

------------------------------ End of modified section ------------------------------