3GPP TSG-RAN WG4 Meeting #87                 	R4-1807783

Busan, Korea (Republic of), 21-25 May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BWP switching delay requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2017-05-14 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Requirements for BWP switching delay need to be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New section 8.x is added to define BWP switching delay requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 8.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

##### 8.x BWP Switching Delay

8.x.1 Introduction

This section defines requirements for the delay within which the UE shall be able to complete BWP switching on PCell or any activated SCell in standalone NR, and PSCell or any activated SCell in SCG in EN-DC.

8.x.2 BWP Switching Delay Requirement

The requirements in this section apply for a UE supporting EN-DC and configured with at least PSCell, and for a UE supporting standalone NR and configured with at least PCell.

When uplink and/or downlink BWP is switched on PCell, PSCell or any of the activated SCells, the UE shall be capable to complete BWP switching upon the specified conditions.

After receiving BWP switching command in slot n, the UE shall be capable to be scheduled in the new BWP no later than in slot n + TBWP_switching + Tchannel_estimation, where:

TBWP_switching is the BWP switching delay.

For Type 1 capable UE, TBWP_switching is up to:

  - A1 slots, if BWP switching involves changing the center frequency and/or bandwidth of the BWP, with or without changing the SCS,

  - A2 slots, if BWP switching involves changing only the SCS.

For Type 2 capable UE, TBWP_switching is up to:

  - B1 slots, if BWP switching involves changing the center frequency and/or bandwidth of the BWP, with or without changing the SCS,

  - B2 slots, if BWP switching involves changing only the SCS.

Table 8.x.2-1 BWP switching delay A1, A2, B3 and B4

|  | NR Slot length (ms) | Interruption length (slots) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | A1 | A2 | B1 | B2 |
| 0 | 1 | 1 | 1 | 2 | 1 |
| 1 | 0.5 | 2 | 1 | 4 | 2 |
| 2 | 0.25 | 3 | 2 | 8 | 4 |
| 3 | 0.125 | 5 | 4 | 16 | 8 |

Tchannel_estimation is the time it takes for the UE for channel estimation on the new BWP after BWP switching. If center frequency of the old BWP and the new BWP is the same, the value for Tchannel_estimation is 0, If center frequency of the old BWP and the new BWP is different, Tchannel_estimation is [TBD].