3GPP TSG-RAN WG4 Meeting #87 	R4-1807805

# Busan, Korea, 21st – 25th May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 37.843: TRP measurement grids |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-05-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Capturing and providing background information of the agreements in [1]. [1] R4-1805865, WF on TRP measurement grids, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |
| Summary of change: |  | TRP related terms, symbols and definitions including background information are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Agreements on TRP measurement grids will not be captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2 and 10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | N | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

# 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

AAS BS receiver: composite receiver function of an AAS BS receiving in an operating band

active antenna system base station: base station system which combines an Antenna Array with an Active transceiver unit array and a Radio Distribution Network

band category: group of operating bands for which the same MSR scenarios apply

Base Station RF Bandwidth: bandwidth in which a base station transmits and/or receives single or multiple carrier(s) and/or RATs simultaneously within a supported operating band

NOTE:	In single carrier operation, the Base Station RF Bandwidth is equal to the channel bandwidth.

Base Station RF Bandwidth edge: frequency of one of the edges of the Base Station RF Bandwidth

beam: main lobe of a radiation pattern from an AAS BS

NOTE:	For certain AAS antenna array, there may be more than one beam.

beam centre direction: direction equal to the geometric centre of the -3 dB EIRP contour of the beam

beam direction pair: data set consisting of  the beam centre direction and the related beam peak direction

beam peak direction: direction where the maximum EIRP is supposed to be found

beamwidth: angles describing the major and minor axes of an ellipsoid closest fit to an essentially elliptic half-power contour of a beam

carrier: modulated waveform conveying the physical channels

carrier aggregation: aggregation of two or more E-UTRA component carriers in order to support wider transmission bandwidths

channel bandwidth: RF bandwidth supporting a single RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell

NOTE 1:	The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

NOTE 2:	For UTRA FDD, the channel bandwidth is the nominal channel spacing specified in 3GPP TS 25.104 [2], For UTRA TDD 1,28 Mcps, the channel bandwidth is the nominal channel spacing specified in 3GPP TS 25.105 [3].

NOTE 3:	For E-UTRA, the channel bandwidths are specified in 3GPP TS 36.104 [4].

contiguous spectrum: spectrum consisting of a contiguous block of spectrum with no sub-block gap(s)

downlink operating band: part of the (FDD) operating band designated for downlink

EIRP accuracy directions set: beam peak directions for which the EIRP accuracy requirement is intended to be met. The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set

equivalent isotropic radiated power: equivalent power radiated from an isotropic directivity device producing the same field intensity at a point of observation as the field intensity radiated in the direction of the same point of observation by the discussed device

NOTE:	Isotropic directivity is equal in all directions (0 dBi).

equivalent isotropic sensitivity: sensitivity for an isotropic directivity device equivalent to the sensitivity of the discussed device exposed to an incoming wave from a defined AoA

NOTE 1:	The sensitivity is the minimum received power level at which a RAT specific requirement is met.

NOTE 2:	 Isotropic directivity is equal in all directions (0 dBi).

hybrid AAS BS:	AAS BS which has both a conducted RF interface and a radiated RF interface in the far field and conforms to a hybrid requirements set

hybrid requirements set: complete set of requirements applied to a hybrid AAS BS with both conducted and radiated  requirements

Inter RF Bandwidth gap: frequency gap between two consecutive Base Station RF Bandwidths that are placed within two supported operating bands

minSENS RoAoA: the reference RoAoA associated with the OSDD with the lowest declared EIS value.

non-AAS BS: BS conforming to one of  the RF requirement specifications 3GPP TS 25.104 [2], 3GPP TS 25.105 [3], 3GPP TS 36.104 [4] or 3GPP TS 37.104 [5]

non-contiguous spectrum: spectrum consisting of two or more sub-blocks separated by sub-block gap(s)

OTA AAS BS: AAS BS which has ≥8 transceiver units for E-UTRA or MSR and ≥4 transceiver units for UTRA per cell and has a radiated RF interface only and conforms to the OTA requirements set.

OTA REFSENS RoAoA: Is the RoAoA determined by the contour defined by the points at which the achieved EIS is 3dB higher than the achieved EIS in the reference direction.

NOTE: This contour will be related to the average element/sub-array radiation pattern 3dB beam width.

OTA requirements set:	 complete set of OTA requirements applied to an OTA AAS BS.

OTA sensitivity directions declaration: set of manufacturer declarations comprising one or more EIS values (with related RAT and channel bandwidth), and the directions where it (they) applies

NOTE:	All the directions apply to all the EIS values in an OSDD.

Radio Bandwidth: frequency difference between the upper edge of the highest used carrier and the lower edge of the lowest used carrier.

radio distribution network: passive network which distributes radio signals generated by the transceiver unit array to the antenna array, and/or distributes the radio signals collected by the antenna array to the transceiver unit array

NOTE:	In the case when the active transceiver units are physically integrated with the array elements of the antenna array, the radio distribution network is a one-to-one mapping.

rated beam EIRP: EIRP that is declared as being achieved in the beam peak direction associated with a particular beam direction pair

receiver target: angles of arrival in which reception is performed

receiver target redirection range: union of all the sensitivity RoAoA achievable through redirecting the receiver target related to the OSDD

receiver target reference direction: direction inside the receiver target redirection range declared by the manufacturer for conformance testing.

NOTE	For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA.

reference beam direction: declared beam direction pair, including reference beam centre direction and reference beam peak direction where the reference beam peak direction is the direction for the intended maximum EIRP within the EIRP accuracy compliance directions set

reference RoAoA: the sensitivity RoAoA associated with the receiver target reference direction for each OSDD.

sensitivity RoAoA: RoAoA within which the declared EIS(s) of an OSDD is intended to be achieved at any instance of time for a specific AAS BS direction setting

sub-block: one contiguous allocated block of spectrum for use by the same Base Station

NOTE	There may be multiple instances of sub-blocks within an Base StationRF Bandwidth.

sub-block gap: frequency gap between two consecutive sub-blocks within an Base Station RF Bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation

TAB connector: transceiver array boundary connector

throughput: number of payload bits successfully received per second for a reference measurement channel in a specified reference condition

total radiated power: is the total power radiated by the antenna.

NOTE 1:	The total radiated power is the power radiating in all direction for two orthogonal polarizations.

NOTE 2:	total radiated power is defined in both the near-field region and the far-field region.

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna

transmission bandwidth: bandwidth of an instantaneous E-UTRA transmission from a UE or BS, measured in Resource Block units

TRP systematic error: the relative difference between theoretical TRP and numerically estimated TRP

uplink operating band: part of the (FDD) operating band designated for uplink

<End of modified section>

<Start of modified section>

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

BeWθ 	The Beam width in θ

BeWϕ	The Beam width in ϕ

BeWθ,REFSENS 	The beamwidth equivalent to the OTA REFSENS RoAoA in the θ-axis in degrees.

BeWφ,REFSENS	The beamwidth equivalent to the OTA REFSENS RoAoA in the φ-axis in degrees.

BWChannel	Channel bandwidth (for E-UTRA)

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz. BWChannel_CA= Fedge_high- Fedge_low.

BWConfig	Transmission bandwidth configuration (for E-UTRA), expressed in MHz, where BWConfig = NRB x 180 kHz in the uplink and BWConfig = 15 kHz + NRB x 180 kHz in the downlink.

DwPTS	Downlink part of the special subframe (for E-UTRA TDD operation

f	Frequency

f	Separation between the Base Station RF bandwidth edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency

fmax 	The largest value of f used for defining the requirement

ΔminSENS 	Difference between conducted reference sensitivity and EISminsens

ΔOTAREFSENS 	Difference between conducted reference sensitivity and OTA REFSENS

EISminsens		The EIS declared for the minSENS RoAoA

FC	Carrier centre frequency

Ffilter	Filter centre frequency

f_offset 	Separation between the Base Station RF bandwidth edge frequency and the centre of the measuring filter

f_offsetmax 	The maximum value of f_offset used for defining the requirement

FBW RF,high 	Upper RF bandwidth edge, where FBW RF,high = FC,high + Foffset, RAT

FBW RF,low 	Lower RF bandwidth edge, where FBW RF,low = FC,low - Foffset, RAT

FC,high		Centre frequency of the highest transmitted/received carrier.

FC,low		Centre frequency of the lowest transmitted/received carrier.

Fedge_low 	The lower edge of aggregated channel bandwidth, expressed in MHz. Fedge_low = FC_low - Foffset.

Fedge_high 	The upper edge of aggregated channel bandwidth, expressed in MHz. Fedge_high = FC_high + Foffset. Foffset, RAT	Frequency offset from the centre frequency of the highest transmitted/received carrier to the upper RF bandwidth edge, sub-block edge or inter-RF bandwidth edge, or from the centre frequency of the lowest transmitted/received carrier to the lower RF bandwidth edge, sub-block edge or inter-RF bandwidth edge for a specific RAT.

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Fuw	Frequency offset of unwanted signal

Ncells	The declared number corresponding to the minimum number of cells that can be transmitted by an AAS BS in a particular band with transmission on all transceiver units supporting the operating band.

NRB	Transmission bandwidth configuration, expressed in units of resource blocks (for E-UTRA)

Pmax,c,EIRP 	The maximum carrier EIRP when the AAS BS is configured at the rated carrier output TRP (PRated,c,TRP)

Pmax,c,TRP 	The maximum carrier output TRP per cell

Pmax,t,TRP 	The maximum total output power per cell

PRated,c,EIRP 	The rated carrier output EIRP when the AAS BS is configured at the rated carrier output TRP (PRated,c,TRP)

PRated,c,TRP 	The rated carrier output TRP

Wgap	Sub-block gap size or Inter RF Bandwidth gap size

The angle in the reference coordinate system between the projection of the x/y plane and the radiation vector defined between -90° and 90°. 0° represents the direction perpendicular to the y/z plane. The angle is aligned with the down-tilt angle.

The angle in the reference coordinate system between the x-axis and the projection of the radiation vector onto the x/y plane defined between -180° and 180°.

PREFSENS	Conducted reference Sensitivity power level

TRPEstimate	Numerically approximated TRP

TRPReference	Theoretical TRP

<End of modified section>

<Start of next modified section>

## 10.9	TRP measurement grids

### 10.9.1	General

TRPReference is the total power radiated by the antenna of a transmitting device, when non-idealities such as mismatch and losses in the antenna are taken into account. In theory, TRPReference is defined as the integral of the antenna’s far field radiation pattern over a spherical surface, that is

TRPReference= θ=0πϕ=02πUθ,ϕ sinθdθdϕ

where U(θ,ϕ) is the radiation intensity at each angle in watts per steradian.

The radiation intensity can be defined as

Uθ,ϕ=EIRP(θ,ϕ)4π

Thus the definite integral for TRPReference becomes

TRPReference= 14π θ=0πϕ=02πEIRPθ,ϕ sinθdθdϕ

In practice, discrete samples of EIRP are measured at different directions (θ,ϕ) over the entire sphere, which are used to numerically approximate the surface TRPReference integral.  The distribution of sampling points on the spherical surface depends the type of sampling grids applied. In following subclauses, several spherical sampling grids are described.

### 10.9.2	Spherical equal angle grids

### With the spherical equal angle grid, the grid spacing is uniform in the θ and ϕ directions. The range of θ angles from 0 to π is divided into N equally spaced subintervals and the range of ϕ angles from 0 to 2π is divided into M equally spaced subintervals. An EIRP sample is measured at a given angle (θn, ϕm), where n and m are the indices used to denote the nth θ and mth ϕ angles, respectively. The definite integral for TRPReference  can simply be approximated as a spatial average of EIRP data samples, which is

### TRPEstimate=π2NMn=1N-1m=0M-1EIRP(θn,ϕm)sinθn

### The total number of angular sampling points is equal to (N-1)×M.

### Note TRPEstimate = TRPReference as N and M approach ∞. The relative error as a result of the approximation is referred to as the TRP systematic error, which is defined as

### TRP systematic error =TRPReferenceTRPEstimate

### There is a trade-off between the accuracy of the TRPEstimate and the total number of sampling points. A large number of sampling points leads to long measurement time. Thus it is important to achieve short measurement time and fulfilling the minimum TRP systematic error.

### The equal angle grid points are not uniformly distributed on the sphere but are clustered towards the poles.

### 10.9.3	Spherical equal area grids

With the spherical equal area sampling grid, the spherical surface is partitioned into N equal area regions. Let n be the index for the nth region and there is one point (θn,ϕn) located in the center of each region. The definite integral for TRPReference can be approximated as

### TRPEstimate=1Nn=1NEIRP(θn,ϕn)

### The equal area grid points are distributed uniformly on the sphere but the pattern of θ and ϕ angles is irregular. The total number angular sampling points is N. Unlike the spherical equal angle grid, the TRPEstimate equation is not weighted by sin θ.

### 10.9.4	Spherical Fibonacci grids

### The Fibonacci grid points are arranged along a generative spiral on the spherical surface. Similar to the equal area sampling grid, the Fibonacci grid generates points that are uniformly spaced in an isotropic way. Assume there are I points in the Fibonacci sampling grid, then the definite integral for TRPReference can be approximated as

### TRPEstimate=1Ii=1IEIRP(θi,ϕi)

where i = 0 .. I-1

θi=cos-11+2i+1I  and

ϕi=2πiΨ,	where 	Ψ=1+52

The total number of angular sampling points is I. Like the spherical equal area grid, the TRPEstimate equation is not weighted by sin θ.

<End of modified section>