3GPP TSG-RAN WG4 Meeting #87	    R4-1807994

Busan, Korean, May 21th-25th, 2018

Source: 	Samsung

Title: 	Ad-hoc minutes for NR UE demodulation and CSI

Agenda Item:	7.12.1

Document for:	Approval

Ad-hoc attendees: Samsung, Intel, Huawei, Qualcomm, Ericsson, CMCC, China Telecomm, CATT, NTT DoCoMo, ANRITSU,R&S, AT&T

Test coverage

| NTT DoCoMo R4-1807439 | Test coverage in Rel-15 performance requirements: Proposal 1: At least the mandatory features without capability signaling summarized Table 1 and Table 2 shall be sufficiently verified in the RAN4 performance requirements. - FFS: Treatment of mandatory features with capability - FFS: Optional features Table 1. Mandatory L1 features without capability in RAN1 feature list [3] # in RAN1 feature list; Feature group; Components. 0-1; CP-OFDM waveform for DL and UL; 1) CP-OFDM for DL. 2) CP -OFDM for UL. 0-3; DL modulation scheme; 1) QPSK modulation. 2) 16QAM modulation. 3) 64QAM modulation for FR1. 1-1; Basic initial access channels and procedures; 1) RACH preamble format. 2) SS block based RRM measurement. [3) RMSI/broadcast OSI reception]. [4] Paging]. 2-1; Basic PDSCH reception; 1. Data RE mapping. 2. Single layer transmission. 2-5; Basic downlink DMRS for scheduling type A; 1. Support 1 symbol FL DMRS without additional symbol(s). 2. Support 1 symbol FL DMRS and 1 additional DMRS symbol. 3. Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port.. 2-6; Basic downlink DMRS for scheduling type B; 1. Support 1 symbol FL DMRS without additional symbol(s). 2. Support 1 symbol FL DMRS and 1 additional DMRS symbol. 2-32; Basic CSI feedback; 1. Type I single panel codebook based PMI (further discuss which mode or both to be supported as mandatory). 2. 2Tx codebook for FR1 and FR2. 3. 4Tx codebook for FR1. 4. 8Tx codebook for FR1 when configured as wideband CSI report. 5. p-CSI on PUCCH formats over 1 – 2 OFDM symbols once per slot (or piggybacked on a PUSCH). 6. p-CSI report on PUCCH formats over 4 – 14 OFDM symbols once per slot (or piggybacked on a PUSCH). 7. a-CSI on PUSCH (at least Z value >= 14 symbols, detail processing time to be discussed separately). further check a-CSI on p-CSI-RS and/or SP-CSI-RS from component-7. 2-50; Basic TRS; 1. Support of TRS (mandatory). 2. All the periodicity are supported.. FFS: TRS bandwidth configuration =. [Atl.1 BWP;. Alt.2: min(52,BWP). Alt.3: Both]. 3-1; Basic DL control channel; 1) One UE-specific configured CORESET per BWP per cell in addition to CORESET0. - CORESET resource allocation of 6RB bit-map and duration of 1 – 3 OFDM symbols for FR1. - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSSs, CORESET resource allocation of 6RB bit-map and duration 1-3 OFDM symbols for FR2. - For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 1-2 OFDM symbols for FR2. - REG-bundle sizes of 2/3 RBs or 6 RBs. - Interleaved and non-interleaved CCE-to-REG mapping. - Precoder-granularity of REG-bundle size. - PDCCH DMRS scrambling determination. - Single TCI state for a CORESET configuration. 2) CSS and USS configurations for unicast PDCCH transmission per BWP per cell. - PDCCH aggregation levels 1, 2, 4, 8, 16. - For type 1 with dedicated RRC configuration, type 3, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot. - For type 1 without dedicated RRC configuration and for type 0, 0A, and 2, the monitoring occasion can be any OFDM symbol(s) of a slot. 3) Monitoring DCI formats 0_0, 1_0, 0_1, 1_1. 4) Number of PDCCH blind decodes per slot with a given SCS follows Case 1-1 table. 5) Processing one unicast DCI scheduling DLand one unicast DCI scheduling UL per slot per scheduled CC. 6) Processing one of RA-RNTI or SI-RNTI or P-RNTI or C-RNTI in a slot per scheduled CC. 5-1; Basic scheduling/HARQ operation; 1) Frequency-domain resource allocation. - RA Type 0 only and Type 1 only for PDSCH without interleaving. - RA Type 1 for PUSCH without interleaving. 2) Time-domain resource allocation. - 1-14 OFDM symbols for PUSCH once per slot. - Starting symbol, and duration are determined by using the DCI. - PDSCH mapping type A with 7-14 OFDM symbols. - PUSCH mapping type A and type B. - For type 1 without dedicated RRC configuration and for type 0, 0A, and 2, PDSCH mapping type A with {4-14} OFDM symbols and type B with {2, 4, 7} OFDM symbols. 3) TBS determination. 4) Nominal UE processing time for N1 and N2 (Capability #1). 5) HARQ process operation with configurable number of DL HARQ processes of up to 16. 6) Cell specific RRC configured UL/DL assignment. 7) Dynamic UL/DL determination based on L1 scheduling DCI with/without cell specific RRC configured UL/DL assignment. 8) Intra-slot frequency-hopping for PUSCH scheduled by Type 1 before RRC connection. 7-1; Channel coding; 1) LDPC encoding and associated functions for data on DL and UL. 2) Polar encoding and associated functions for PBCH, DCI, and UCI. 3) Coding for very small blocks. - Note #1: Mandatory or optional is still under discussion in RAN1 and RAN4. - Note #2: Gray components are only for UL channel and yellow components are still under discussion in RAN1 Table 2. Mandatory L1 features without capability in RAN4 feature list [4] # in RAN4 feature list; Feature group; Components. 1-2; 64QAM modulation for FR2 PDSCH; 1) 64QAM modulation for FR2 PDSCH. |
| --- | --- |

Agreements:

First focused on the mandatory features

FFS for handling mandatory features with and without capability signalling in same priority or not

General: common parameters &Scenarios

Configurations of Channel bandwidth and SCS (Channel Bandwidth +SCS) (30 minutes)

Proposals from companies

| Companies | Proposals |
| --- | --- |
| Ericsson R4-1806118 | Demod: - FR1: 20, 40, 60, 80, 100 MHz +30kHz - FR2: 100MHz+120kHz CSI: - FR1: 20,100MHz+30kHz - FR2: 100,200MHz+120kHz |
| Intel R4-1806299 (Demod) | FR1: - 10 MHz CBW + 15kHz SCS - 20 MHZ CBW + 30kHz SCS - 40 MHZ CBW + 30kHz SCS FR2: - 50 MHZ CBW + 60kHz SCS - 100 MHZ CBW + 120kHz SCS |
| Qualcomm R4-1806366 (Demod) | FR1: - TDD: 40MHz CHBW+30kHz SCS - FDD: 20 MHZ CBW + 15kHz SCS FR2: - TDD: 100MHz +120kHz |
| CTC R4-1806370 | For FR1, not preclude any of the sub-carrier spacing for PDSCH/PDCCH at the beginning. (15kHz, 30kHz, 60kHz) |
| CATT R4-1806713 | Frequency Range; SCS and UE CBW combinations; Applicable NR bands (for example). FR1; 15 KHz- 5 MHz; n1, n2, n3…. 15 KHz- 10MHz; n1, n2, n3…. 30 KHz- 20MHz; n1, n41, n77…. 30KHz- 50MHz; n41, n77, n78, n79…. FR2; 60 KHz-100 MHz; n257…. 120 KHz -100 MHz; n257…. |
| CMCC R4-1806892 | FR1: - 20 MHZ CBW + 15kHz SCS - 100 MHZ CBW + 30kHz SCS FR2: - 200MHz +60KHz SCS - 200MHz +120kHz SCS |
| NTT DoCoMo R4-1807438 | SCS: - FR1: 15kHz for 2Rx UE, 30kHz for 4Rx UE - FR2: 120kHz for 2Rx UE - Besides such default values, additional dedicated test cases for other SCS required CHBW: - FR1 - Group 1: 10MHz for bands where max CBW is lower than 30MHz(e.g. almost LTE-referming bands except for band n41/n50) - Group 2: 50MHz for bands where max CBW between 30Mz and 100MHz (e.g. n41, n50, n77-n79) - FR2: Group 3: 100MHz or 200MHz |
| Samsung R4-1806442 | Default set-up - FR1: 20MHz +30kHz - FR2: 100MHz +60kHz Additional dedicated test cases - FR1: 5MHz +15kHz, 20MHz +60kHz, 40MHz + 30kHz - FR2: 200MHz +120kHz |
| Huawei R4-1807910 (Demod) | - Define the demodulation performance requirements for each supported maximum bandwidths for each SCS - FR1: ? - 15kHz: 5MHz, 20MHz, 25MHz, 30MHz, 40MHz, 50MHz - 30KHz: 20MHz, 25MHz, 30MHz, 40MHz, 80MHz and 100MHz - 60KHz: 20MHz, 25MHz, 30MHz, 40MHz, 80MHz and 100MHz - FR2: ? - 200 MHz+ 60kHz - 200MHz + 120kHz |

Previous RAN4 agreements:

For supported numerologies, such agreements agreed in previous RAN4 meeting:

- FR1: 15kHz, 30kHz for sub 1GHz; 15kHz, 30kHz and 60kHz (optional) for above 1GHz

- FR2: 60kHz, 120kHz

Regarding channel bandwidths supported by UE, all the channel bandwidths combos of FR1 in current specification 38.101-1 15.1.0 mandatory supported by UE and for FR2, upper to 200MHz mandatory supported by UE as specified in 38.101-2 15.1.0. The supported combinations of channel bandwidth and SCS for each band specified in Table 5.3.5-1 of TS38.101-1/-2.

Key factors need to be considered: Test coverage, test effort, number of test cases, feasible for supporting band agonistic manner

Q1: Do we need default configuration of {Channel bandwidth, SCS} for UE performance requirements?

Agreement:

Introduce performance requirements in band agnostic manner per frequency range at least for FR1

Selected some combination(s) of {Channel bandwidth, SCS} as starting point for early stage simulation alignment purpose

- Candidate options for FR1:

- 20MHz + 30kHz

- 10MHz+ 15kHz

- 40MHz +30kHz

- Candidate options for FR2:

- 100 MHz + 120 kHz (baseline for alignment simulation purpose)

- 100 MHz + 60 kHz

- 200 MHz + 120 kHz]

Additional test cases for other specific combinations can be further discussed

Q2: Do we need additional dedicated test cases for several combinations of {Channel bandwidths, SCS} besides of default test set-up?

Q3: Do we need to differentiate TDD /FDD with different default config.(FR1) ?

Q4: Do we need to cover all SCSs?

TDD DL/UL configuration (30 minutes)

Summary proposals:

| Companies | FR1 |  |  | FR2 |  |
| --- | --- | --- | --- | --- | --- |
|  | 15kHz | 30kHz | 60kHz | 60kHz | 120kHz |
| Ericsson R4-1806118 |  | DDSU, S=13D+1Gp |  |  | DDSU,S=12D+2Gp |
| Intel R4-1806299 | Aligned with LTE config#2 DDDSU, S = 12D+1Gp+1U | 7D1S2U S = 10D + 2Gp + 2U | Opt1: 7D+S+12U, S = 6D+4Gp+4U Opt2: 7D1S2U,S = 10D + 2Gp + 2U | DDDSU, S = 12D+1Gp+1U | 7D1S2U, S = 10D + 2Gp + 2U |
| Qualcomm R4-1806365 |  | DSSU, S1 = 10D+2Gp+2U, S2 = 12D+2Gp |  |  | DDSU, S = 12D+2Gp |
| CTC R4-1806370 | Select one UL/DL configuration resulting in larger number of HARQ process. |  |  |  |  |
| CMCC R4-1806893 |  | Option 1: DDDSU, S (D, GP, U) = [8,4,2] Option 2: DDSU, S (D, GP, U) = [10,4,0] Option 3: DDDSSDDSUU, S1(D,GP,U)=[12, 2, 0], S2(D,GP,U)= [0,2,12], S3(D, GP, U) = [10,4,0] |  |  |  |
| NTT DoCoMo R4-1807438 |  | Multiple Scenarios Option 1: 7D1S2U, S = 6D + 4 Gp + 4U Option 2: SU, S = 12D+2Gp Option 3: DS1DS1DS1DS2UU, S1=10D+2Gp+2U, S2=6D + 4 Gp + 4U |  |  | Multiple Scenarios Option 1: DDDSU, S = 12D+2Gp Option 2: DSUU, S = 12D+2Gp Option 3: DSDSU, S = 10D+2Gp+2U |
| Huawei R4-1807912 | LTE config#2, DDDSU | DDDSU S = 8~10 D + 4~2 Gp + 2UL |  | DDDSU S = 8~10 D + 4~2 Gp + 2UL | DDDSU S = 8~10 D + 4~2 Gp + 2UL |
| CATT R4-1806713 | LTE config#1 |  |  |  |  |

Q1: Do we need to introduce several configs. covering different scenarios? i.e. DL heavy, UL heavy, increasing UL opportunity (URLLC)

Introduce multiple configurations covering different scenarios (30 kHz), FFS for detailed configurations.

- FFS for other numerologies

Configuration for different SCSs will be decided separately

Q2: Do we need default config. if multiple configs. Introduced?

FR1: (Test set-up)

- 15 kHz: DDDSU(aligned with LTE config#2 with 5ms periodicity)

  - S = 12D+1Gp+1U

Candidate options for initial simulation alignment purpose:

- FR1 15 kHz:

  - DDDSU(aligned with LTE config#2 with 5ms periodicity)

    - S = 12D+1Gp+1U

- (FR1 30 kHz):

  - DDDSU

  - 7D1S2U

  - DSSU

- FR2 60 kHz:

  - DDDSU

- FR2 120 kHz:

  - DDDSU

  - DSDSU

TDD configurations for performance requirements can be different with initial alignment simulations assumption

Others (pending on time)

Issue 1: SA/NSA requirements, CA/DC/SUL requirements

| Companies | Proposals |
| --- | --- |
| Intel R4-1806299 | NR UE performance requirements shall cover SA/NSA requirements - Reuse basic demodulation / CSI requirements for SA/NSA modes - NSA requirements: - Define only NR requirements for normal demodulation / CSI tests - Define both LTE and NR requirements for SDR test cases - FFS on how to select the BCs for NSA performance requirements - FFS for LTE configuration for the NSA performance requirements NR UE performance requirements shall cover both single carrier and CA scenarios. Prioritize the following requirements - Single carrier normal demodulation performance requirements - SDR requirements for single carrier and CA CA normal demodulation requirements are deprioritized |
| China Telecom R4-1806370 | First introduce the NR only requirements that can be applied in SA and NSA scenarios, and then introduce NSA-specific requirements. For NR CA, the demodulation requirements defined for LTE CA can be considered as a starting point, and other requirements shall also be added when necessary. If it is impossible to finalize the CA requirements in Rel-15 timeline, maybe a phased approach can be discussed. |
| Huawei R4-1807908 | Define the EN-DC demodulation performance requirements in terms of single carrier LTE and NR performance. LTE and NR CCs will be transmitted simultaneously and the performance on each CC will be verified separately. For EN-DC, configure TM3 transmission on LTE CCs and keep it unchanged when verify the performance of different features on NR CCs. Define the NR CA demodulation performance requirements in terms of single carrier NR performance. All the NR CCs will be transmitted simultaneously and the performance on each CC will be verified separately. Define the UE demodulation performance requirements for SUL to verify the performance following the HARQ timing for SUL band combination and when the numerologies on the downlink and uplink carriers are different. |

Issue 2: Tx/Rx EVM assumption

| Companies | Proposals |
| --- | --- |
| Intel R4-1806299 | Use the following RF impairments models to define requirements - FR1: TX EVM = 6% for QPSK/16QAM/64QAM and 3% for 256QAM - FR2: [X]% TX EVM (AWGN based) and explicitly modelled TX/RX phase noise |
| Huawei R4-1807908 | It is proposed not to model Rx EVM for the simulation and follow the traditional LTE approach to derive the NR demodulation performance requirements. |

Issue3: other proposals

| Companies | Proposals |
| --- | --- |
| Intel R4-1806299 | - Deployment Scenarios Prioritize work on Single TRP noise-limited scenarios without inter-cell interference. Consider multi-TRP DPS scenarios with the 2nd priority. Consider inter-cell interference scenario with 2nd priority to verify IRC functionality for FR1. Reuse LTE assumptions on the typical interference power profiles. - Number of Rx ports Further discuss the methodology to define the 2RX/4RX requirements for FR1 and applicability rules for UEs supporting different number of RX chains in different bands. For FR2 define the performance requirements under assumption of single active RX antenna panel |
| Samsung R4-1806440 | Work Plan (Phase approach) Phase 0: Overall test scope, generic common test set-up and channel modelling, specification skeleton Phase 1: Completion 1st bullet of test cases (RAN4 July Ad-hoc, RAN4#88) - Demod: - PDSCH: Normal test cases with single carrier - PDCCH - CSI: CQI, PMI test cases Phase 2: Completion 2nd bullet of test cases (RAN4#88bis, RAN4#89) - Demod: - PDSCH: SDR test, CA/DC/SUL test cases if any - PBCH - CSI: RI, L1-RSRP, CRI test cases if any |
| Ericsson R4-1806118 | Work plan and scope Proposal 1: NR UE performance test scope includes 3 categories of tests. - Demodulation tests - Basic performance tests for essential NR baseband functionality - Bandwidth, Subcarrier spacing, DMRS demodulation, etc. - Additonal performance tests for certain NR features - QCL, time/freq tracking, etc. - CSI tests - Basic performance tests for essential NR baseband functionality - Bandwidth, Subcarrier spacing, CSI-RS for CSI reporting, etc. - Additonal performance tests for certain NR features - QCL, precoding for more Tx ports for beamingforming, etc. - Functional tests related to DL performance - Right beam is tracked/used, SRS coherency, etc. Proposal 2: Rel-15 focus on basic performance tests of critical NR features, e.g. CBW, SCS, DMRS for demodulation and CSI-RS for CSI reporting, considering the phase approach for the timeline. |

PDSCH (30 minutes)

- Performance metrics

- FRCs

- HARQ parameters and RV sequence

- Resource allocation

- PRB bundling parameters

- DRMS configuration

- CSI-RS for TRS

- PTRS

- QCL

- Reference receiver assumptions

Number of Tx ports

| NTT DoCoMo | FR1: 1, 2, 4 and 8Tx ports (up to 8 NZP/ZP CSI-RS ports) FR2: 1, 2 and 4Tx ports (up to 4 NZP/ZP CSI-RS ports) |
| --- | --- |
| Qualcomm | 1 layer 1Tx, 2 layer 2Tx, 4layer 4Tx |
| Intel | 2Tx |
| Ericsson | FR1 8Tx FR2 2Tx |
| Samsung | 1 layer 1Tx, 2 layer 2Tx/4Tx, 4layer 4Tx |
| Huawei | 1Tx, 2Tx, 4Tx Layer :1 layer, 2 layers, 3 layers, 4layers |
| CMCC | TDD: 8Tx |

Potential Agreements:

MIMO configurations

For initial alignment simulation purpose:

FR1:

- 1 layer test: 2Tx 2Rx, 2Tx4Rx

- 2 layer test: 2Tx 2Rx, 2Tx4Rx

- 4 layer test:  4Tx4Rx,8Tx4Rx

- 3 layer test: 4Tx4Rx

FR2:

- 1 layer test: 1Tx 2Rx

- 2 layer test: 2Tx 2Rx

4RX demodulation requirements in FR2 not excluded

Performance metrics

| Intel | Reuse LTE metric as throughput vs. SNR (70%) |
| --- | --- |
| Qualcomm | Reuse LTE metric as throughput vs. SNR (70%) |
| CATT | Reuse LTE metric as throughput vs. SNR (70%) |
| Huawei | Reuse LTE metric as throughput vs. SNR (70%, 30%) |
| CTC | Reuse LTE metric as throughput vs. SNR For URLLC, BLER vs SNR (10-5) |
| AT&T | Spectral efficiency vs SNR |

At least for FR1: Reuse LTE metric as throughput vs. SNR

  - Test point: 70% (normal demodulation test cases)

  - FFS for other test point(s) of some specific test cases

FFS for test metric for URLLC specific test cases if any

FRCs

| Intel | MCS5 (Table 1), MCS13 (Table 1), MCS24 (Table 1) and MCS24 (Table 2) |
| --- | --- |
| CATT | QPSK 1/3, 16QAM 1/2, 64QAM 3/4, 256QAM 3/4 |
| Huawei | QPSK 1/3, 16QAM 1/2, 64QAM 1/2, 64QAM 3/4, 256QAM 0.6, 256QAM 0.77 |
| Qualcomm | FR1, Single layer: MCS5, MCS13, MCS24 FR1, Two layers: MCS13, MCS19 FR1, Four layers: MCS14, MCS18 FR2, Single layer: MCS5, MCS13, MCS19 FR2, Two layers: MCS13, MCS18 |
| Samsung | QPSK 1/3, 16QAM 1/2, 64QAM 3/4, 256QAM 4/5 |

At least covering below MCS levels:

QPSK 1/3, 16QAM 1/2, 64QAM 3/4, 256QAM 4/5 (FR1 only)

HARQ parameters and RV sequence (Maximum number of HARQ processes, RV sequence, TB-based and CBG-based HARQ)

| Intel | FDD : Max number HARQ 4 {0,1,2,3} for QPSK and 16QAM and {0,0,1,2} for 64QAM and 256QAM TB-Based |
| --- | --- |
| Ericsson | {0,1,2,3} |
| Huawei | {0,1,2,3} for QPSK and 16QAM and {0,0,1,2} for 64QAM and 256QAM Both TB and CBG-Based |
| Qualcomm | FDD : Max number HARQ 4 {0,2,31} |

FDD: Max number HARQ transmission 4

RV sequence for alignment simulation purpose:

Option 1: {0, 1 ,2,3} for QPSK and 16QAM and {0,0,1,2} for 64QAM and 256QAM

Option 2: {0, 2, 3,1} for all the modulation orders

PDSCH scheduling (Resource allocation, PDSCH mapping type, PRB bundling)

| Intel | Full PRB allocation Type A PRB bundling size 2 |
| --- | --- |
| Ericsson | Full PRB allocation PRB bundling size 2 and WB |
| Huawei | Full PRB allocation PRB bundling size 2 |
| CTC | Both Type A and Type B |
| AT&T | Type A and Type B |
| Qualcomm | PRB bundling size 2 and WB |

Initial alignment simulation purpose:

- Full PRB allocation with channel bandwidth

- PDSCH mapping type: both type A and type B

- Further discussion for candidate options this week for PRB bundling size

DMRS configuration

| Intel | Type 1 or 2, 1 symbol FL DMRS without additional symbol Type 1 or 2, 1 symbol FL DMRS with 1 additional symbol Type 1 or 2, 1 symbol FL DMRS with 2 additional symbols |
| --- | --- |
| Ericsson | Type 1, 1 symbol FL DMRS with 1 or 2 additional symbol |
| Huawei | Type 1 or 2, 1 symbol FL DMRS without additional symbol; Type 1 or 2, 1 symbol FL DMRS with 1 additional symbol; Type 1 or 2, 1 symbol FL DMRS with 2 additional symbols. |
| CATT | 1 symbol FL DMRS without additional symbol(s) 1 symbol FL DMRS and 1 additional DMRS symbol. |
| Qualcomm | Type 1, 1 symbol FL DMRS with 1 additional symbol |

PTRS, TRS

| Intel | PTRS configure for FR2 |
| --- | --- |
| Ericsson | PTRS configure for FR2 |
| Huawei | Not configured PTRS Configure TRS |
| AT&T | PTRS configure for FR2 |
| Qualcomm | PTRS configure for FR2 |

Reference Receiver

| Intel | LMMSE-IRC and R-ML for SU-MIMO, LMMSE-IRC for MU-MIMO and inter-cell interference |
| --- | --- |
| CTC | Not consider interference-aware receiver in Rel-15 timeline. |
| Huawei | Use MMSE as the reference receiver. |

Others

QCL

Transmission scheme

SDR Test

Intel [R4-1806284]: Further discuss which combinations of BW and numerologies will be used for FR1 and FR2 SDR tests.

Huawei [R4-1807909]: Design the SDR test to verify whether UE has the simultaneous buffering and can share the soft buffer among different HARQ processes such that the peak data rate can be achieved and performance can be guaranteed.

Control channel PDCCH/PBCH (pending on time)

- Test metrics

- Aggregation level

- CORESET configuration

- DCI format (number of bits)

- Reference receiver assumptions

Test Metric

| Intel | Pm-dsg |
| --- | --- |
| CATT | SNR @ 1% Pm-dsg |

Aggregation level

| Intel | 2, 4, 8 |
| --- | --- |
| CATT | 4,8 |
| Huawei | Based on test coverage. Use higher AL for 2 Rx and less for 4Rx cases |

CORESET configuration

| Intel | CORESET BW: (FR1) 24,48,96; (FR2): 24,60; CORESET duration: 1,2 symbols; CCE-to-REG Mapping: Interleaved, non-interleaved |
| --- | --- |
| Qualcomm | Contiguous PDCCH CORESET; 2 symbol CORESET (sym 0,1); REG bundle precoder granularity |
| Huawei | CORESET BW: Max RBs for corresponding BW CORESET duration: 2, 1 symbols; CCE-to-REG Mapping: Interleaved and/or non-interleaved REG bundle size precoder granularity |
| CATT | CORESET BW: (FR1) 52, 51; (FR2): 132; [Allocated resource blocks] CORESET duration: 2 symbols; CCE-to-REG Mapping: Interleaved and/or non-interleaved REG bundle size precoder granularity |

DCI format (information bits)

| Intel | 1_0, 1_1 |
| --- | --- |
| CATT | 1_0 Number of bits: FR1: 39; FR2: 42 |
| Huawei | ~ 40 bis |

Others

Reference receiver assumptions

- Intel [R4-1806324]: LMMSE

- CATT [R4-1806714]: MMSE

Antenna Configuration

- Intel [R4-1806324]: 2x2, 2x4; Low correlation

- CATT [R4-1806714]: 2x2; Low correlation

PBCH

| Intel | Introduce requirements to test key PBCH features in different conditions with different SSB periodicity |
| --- | --- |
| Huawei | De-prioritize PBCH test cases |
| CTC | Introduce test cases |

CSI (30 minutes)

Overall Test Configuration &Scope

Issue 1: CSI configuration

- CSI reporting  type

- CSI-RS resource setting

- CSI-RS resource type

- Number of CSI report and BW

| Company | Proposals |
| --- | --- |
| Samsung | P1: Introducing CSI performance requirements focused on the mandatory and fundamental features of NR CSI framework under Rel-15 timeframe. P2: In Rel-15, NR CSI test cases focused on below configurations: - CSI report types: periodic CSI report and aperiodic CSI report. - CSI-RS resources setting: 1 NZP CSI-RS resource and 1 CSI-IM resource configuration for CSI report. CSI-IM used for interference measurement. - Number of CSI report and BWP: single BWP/CC with 1 CSI reporting configured. |
| Ericsson | NZP CSI-RS for channel measurement CSI-IM for interference measurement FR1: Periodic CSI-RS for FR1, Aperiodic CSI-RS for FR2 |
| Intel | - NZP CSI-RS for channel measurement - CSI-IM for interference measurement |

First priority focused on:

NZP CSI-RS for channel measurement
CSI-IM for interference measurement

CSI report types: periodic CSI report and aperiodic CSI report

CSI-RS resources types: periodic and aperiodic

Issue 2: Test scope

- L1-RSRP

- CRI

- LI

| Company | Proposals |
| --- | --- |
| Samsung | L1-RSRP belong to NR CSI framework, new requirements need to be introduced under CSI performance requirements agenda in Rel-15. |
| Huawei | Introduce CRI tests as one part of NR FR1 CSI tests. Introduce LI tests to verify the reporting accuracy in RAN4 CSI tests. FFS whether to introduce L1-RSRP test cases in Demod part or RRM part. |
| Intel | CRI and LI second priority L1-RSRP align with RRM discussion |
| AT&T | RAN4 should define CSI reporting requirements for CQI, PMI, RI, CRI, LI and L1-RSRP |

Issue 3: Channel bandwidth and SCS

| Company | Proposals |
| --- | --- |
| Samsung | FR1 20MHz 30kHz FR2 100MHz,60kHz |
| Ericsson | FR1 20MHz,100MHz /30kHz FR2 100MHz,200MHz/120kHz |
| Intel | FR1 10MHz/15KHz 20MHz/30KHz FR2 100MHz/120kHz |

CQI reporting

-CQI tables

-WB &SB CQI test

-Test metric

-MIMO configuration

| Company | Proposals |
| --- | --- |
| Samsung | FR1 (CQI table 2) FR2 (CQI table 1) Static CQI definition test, wideband fading CQI test and sub-band frequency selective test need to be introduced For static CQI test, single CQI reporting, test metrics (CQI distribution and BLER requirements) as existing LTE test can be reused as starting point FR1: 2Tx & (2Rx / 4Rx); FR2: 2Tx & 2Rx |
| Intel | Wideband CQI, prioritize reporting test with Table 1/2 |
| Ericsson | Wideband CQI CQI table 2 (FR1) CQI table 1 (FR2) 16Tx port with 2/4 Rx ports for FR1 2Tx 2Rx for FR2 |
| Huawei, HiSilicon | WB/SB CQI |

PMI reporting

- Codebook Type

- Number of Tx ports

- PMI reporting WB &SB

- Test metric

| Company | Proposals |
| --- | --- |
| Samsung [R4-1806444] | Type-I single, Type-I multi, Type-II, Type-II port FR1: single PMI 8Tx & (2Rx / 4Rx); multiple PMI 4Tx & (2Rx / 4Rx) |
| Intel | Wideband PMI, Type-I single, FR1: 4Tx & 8Tx; FR2: 2Tx & 4Tx |
| Ericsson | Wideband PMI, Type-I single, FR1: 16Tx & 32Tx; FR2: 2Tx |
| Huawei, HiSilicon | Type-I single |

In rel-15 focused on Type-I single panel codebook

RI reporting

- Number of Rank

- Test metric

| Company | Proposals |
| --- | --- |
| Intel | FR1: up to rank4; FR2: up to rank2; |
| Ericsson | FR1: up to rank4; FR2: up to rank2; |
| Huawei, HiSilicon | Reuse test metric  and as the starting point |

Other issues

AT&T (R4-1807415):  RAN4 should send  an LS to RAN1 about the DMRS assumption for computing CSI

Channel model (Pending on time)

-Channel models (TDL or simplified TDL)

-Doppler shift

-Delay spread

Intel [R4-1806325]:

For FR1 we have the following proposals:

Proposal #1: For FR1, simplify TDL channel models by choosing strongest paths that contribute to [95]% of total power

Proposal #2: For FR1 choose desired delay spread based on short / nominal / long delay spread condition for NR UE performance requirements

Proposal #3: Prioritize defining test requirements under low antenna correlation. The medium and high antenna correlation coefficients are FFS for FR1

For channel models for FR2 we have the following observations and proposals:

Proposal#4: For generation of TDL from CDL the following parameters are FFS: Antenna array configuration, Antenna pattern, Scaling of angles, Base CDL channel model

Proposal#5: The methodology for Option 2 Doppler shift modelling based on spatial parameters is FFS and needs to be further clarified

Proposal #6: The down selection of methodology to model Doppler shall consider impact to UE performance

Proposal #7: It is FFS if channel model simplification by eliminating non-significant paths is required in FR2 after TDL channel re-generation

Ericsson [R4-1806768]:

Proposal 1: RAN4 defines the propagation channel model for FR1 by simplifying the path delay profile specified in TR 38.901, TDL-A, TDL-B, and TDL-C.

Proposal 2: For simplification, RAN4 set the tap delay resolution up to 5ns.

Proposal 3: For TDL propagation channel model, RAN4 sets the realistic delay spread factors according to the measurement results in the field. For FR1, we should set 30ns to 300ns, considering macro cell, micro cell or indoor.

Proposal 4: For TDL propagation channel model, RAN4 considers trimming the weak channel paths.

Proposal 5: For FR2, RAN4 consider using TDL channel models with small delay spread but need wait for the outcome of testability SI.

Huawei [R4-1807479]:

Proposal 1: Select TDL-B and TDL-C for NR FR1 performance requirements.

Proposal 2: Select 30ns and 300ns as the scaling parameters for NR FR1 performance requirements with 15kHz SCS and scale accordingly for other SCS.

Propose 3: Define Doppler spread value of 10Hz, 100Hz and 500Hz for 15kHz SCS and scale according for other SCS.

Propose 4: Remove small power taps to simplify TDL channels.

Propose 5: Reuse LTE methodology and parameters for MIMO correlation in the current stage.

- Annex

- T-doc list

| TDoc | Title | Source |
| --- | --- | --- |
| R4-1806118 | NR UE performance test scenarios and test lists | Ericsson |
| R4-1806119 | TDD UL/DL configuration for UE REFSENS and performance tests | Ericsson |
| R4-1806120 | Consideration for the performance spec structure | Ericsson |
| R4-1806272 | Discussion on NR UE CSI feedback performance requirements | Intel Corporation |
| R4-1806284 | NR PDSCH UE demodulation requirements | Intel Corporation |
| R4-1806298 | Views on TS 38.101-4 NR UE performance requirements specification structure | Intel Corporation |
| R4-1806299 | Views on NR UE Demodulation and CSI reporting requirements | Intel Corporation |
| R4-1806300 | Views on NR UE soft buffer requirements | Intel Corporation |
| R4-1806324 | NR DL Control UE demodulation requirements | Intel Corporation |
| R4-1806325 | Propagation channels for NR demodulation requirements | Intel Corporation |
| R4-1806365 | TDD Slot Patterns for Demodulation Performance Tests | Qualcomm Incorporated |
| R4-1806366 | BW-SCS combinations for demodulation performance tests | Qualcomm Incorporated |
| R4-1806370 | General views on NR UE demodulation requirements | China Telecom |
| R4-1806440 | Work plan for UE performance requirements | Samsung |
| R4-1806441 | Proposals on 38.101-4 specification structure | Samsung |
| R4-1806442 | Overview of UE demodulation performance requirements | Samsung |
| R4-1806443 | Draft skeleton of 38.101-4 | Samsung |
| R4-1806444 | Overview of UE CSI performance requirements | Samsung |
| R4-1806578 | Test Cases for Single-Carrier PDSCH Demodulation Performance | Qualcomm Incorporated |
| R4-1806618 | High level requirements for PDCCH demodulation performance tests | Qualcomm Incorporated |
| R4-1806713 | Discussion on NR UE demodulation requirements | CATT |
| R4-1806714 | Discussion on NR PDCCH demodulation requirement | CATT |
| R4-1806768 | Propagation channel model for demodulation requirements | Ericsson |
| R4-1806892 | Discussion on SCS, CBW and Rx ports for UE demodulation performance | CMCC |
| R4-1806893 | Discussion on UL/DL configurations for NR TDD demodulation performance | CMCC |
| R4-1807406 | Parameters and assumptions for UE demodulation performance and CSI reporting requirements | AT&T |
| R4-1807411 | PDSCH demodulation requirements for NR | AT&T |
| R4-1807413 | Performance requirement for 1 symbol PDCCH for LTE-NR co-existence scenarios | AT&T |
| R4-1807415 | DMRS assumptions for CSI reporting | AT&T |
| R4-1807438 | Views on UE demodulation requirements | DOCOMO Communications Lab. |
| R4-1807439 | Forward compatible issue for UE demodulation requirements | DOCOMO Communications Lab. |
| R4-1807476 | Discussion on PDCCH demodulation performance requirements | Huawei, HiSilicon |
| R4-1807477 | Discussion on PBCH demodulation performance requirements | Huawei, HiSilicon |
| R4-1807478 | Discussion on NR CSI requirements | Huawei, HiSilicon |
| R4-1807479 | Discussion on channel model for UE demodulation performance requirements | Huawei, HiSilicon |
| R4-1807888 | Testing Code Block Groups | Huawei Telecommunication India, HiSilicon |
| R4-1807908 | Discusson on scenarios and common parameters for NR UE demodulation performance requirements | Huawei, HiSilicon |
| R4-1807909 | Verification of peak data rate and soft buffer | Huawei, HiSilicon |
| R4-1807910 | Discussion on PDSCH demodulation performance requirements | Huawei, HiSilicon |
| R4-1807911 | Further discussion on the specification structure for 38.101-4 | Huawei, HiSilicon |
| R4-1807912 | UL-DL configuration for NR demodulation performance requirements | Huawei, HiSilicon |

- Channel bandwidth and SCS per band in TS38.101-1/-2

Table 5.3.5-1 Channel Bandwidths for Each NR band in 38.101-1

| NR band / SCS / UE Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 101,2 MHz | 152 MHz | 202 MHz | 252 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
| n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
| n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
| n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
| n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
| n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |
| n50 | 15 | Yes | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes3 |  |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes3 |  |
| n51 | 15 | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |
| n70 | 15 | Yes | Yes | Yes | Yes3 | Yes3 |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes3 | Yes3 |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes3 | Yes3 |  |  |  |  |  |  |
| n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n74 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
| n75 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
| n76 | 15 | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |
| n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |
| n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |
|  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes |
| n80 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
| n81 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n82 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n83 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n84 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
| NOTE 1: 90% spectrum utilization may not be achieved for 30kHz SCS. NOTE 2: 90% spectrum utilization may not be achieved for 60kHz SCS. NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.3.5-1: Channel bandwidths for each NR band from TS38.101-1

| NR band / SCS / UE Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| n258 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| n260 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| n261 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |

- Proposals from companies for TDD UL/DL configuration

| Companies | Proposals |
| --- | --- |
| Ericsson R4-1806118 | FR1: Phase 1: Aligned with LTE configu#2 with 5ms periodicity Phase 2: Consider more configs. FR2: Use 3DL 1UL for first round simulation alignment 30kHz FR1 :DDSU, S=13D+1Gp+1U 120kHz FR2: DDSU, S=13D+1Gp |
| Intel R4-1806299 | Reuse DL/UL configurations defined for REFSENS for base NR demodulation requirements. Proposal 1: Use the following configurations for TDD FR1 REFSENS tests RRC Configuration; 15 kHz SCS; 30 kHz SCS; 60 kHz SCS (Option 1); 60 kHz SCS (Option 2). UL-DL-configuration-common; referenceSubcarrierSpacing; 15 kHz; 15 kHz; 15 kHz; 30 kHz. dl-UL-TransmissionPeriodicity; ms5; ms5; ms5; ms2p5. nrofDownlinkSlots; 3; 3; 1; 3. nrofDownlinkSymbols; 12; 12; 12; 12. nrofUplinkSlots; 1; 1; 1; 1. nrofUplinkSymbols; 1; 1; 3; 1. PDSCH-ServingCellConfig; nrofHARQ-ProcessesForPDSCH; 6; 8; 8; 8. PUCCH-Config; dl-DataToUL-ACK; [2, 3, 4, 6]; [2, 4, 6, 8]; [8]; [2, 4, 6, 8]. Proposal 2: Use the following configurations for TDD FR2 REFSENS tests RRC Configuration; 60 kHz SCS; 120 kHz SCS. UL-DL-configuration-common; referenceSubcarrierSpacing; 60 kHz; 60 kHz. dl-UL-TransmissionPeriodicity; ms1p25; ms1p25. nrofDownlinkSlots; 3; 3. nrofDownlinkSymbols; 12; 12. nrofUplinkSlots; 1; 1. nrofUplinkSymbols; 1; 1. PDSCH-ServingCellConfig; nrofHARQ-ProcessesForPDSCH; 6; 8. PUCCH-Config; dl-DataToUL-ACK; [2, 3, 4, 6]; [2, 4, 6, 8]. |
| Qualcomm R4-1806365 | FR1: FR2: |
| CTC R4-1806370 | Select one UL/DL configuration resulting in larger number of HARQ process. |
| CMCC R4-1806893 | FR1 (30kHz) - Option 1: DDDSU, S (D, GP, U) = [8,4,2] - Option 2: DDSU, S (D, GP, U) = [10,4,0] - Option 3: DDDSSDDSUU, S1(D,GP,U)=[12, 2, 0], S2(D,GP,U)= [0,2,12], S3(D, GP, U) = [10,4,0] Note: Slightly prefer option 3 Op1 Op2 Op3 |
| NTT DOCOMO R4-1807438 | For FR1: For FR2: Proposal: Discuss the impact of 16HARQ process on the demodulation performance in HARQ retransmission scenario (soft buffer, etc). |
| CATT R4-1806713 | LTE config#1, DDSUU |
| Huawei R4-1807912 | Proposal 1: Use the slot format that is aligned with LTE config#2 with 5ms periodicity for NR demodulation performance with SCS 15kHz. Proposal 2: Use slot format {D D D S U} with S slot format: nrofDownlinkSymbols = 8~10, nrofUplinkSymbols = 2 for NR FR1 with SCS 30kHz demodulation performance requirements with dl-UL-TransmissionPeriodicity 2.5ms. Proposal 3: Use slot format {D D D S U} with S slot: nrofDownlinkSymbols = 8~10, nrofUplinkSymbols = 2 for NR FR2 with SCS 60kHz and SCS 120kHz demodulation performance requirements with dl-UL-TransmissionPeriodicity 1.25ms and 0.625ms respectively. |