3GPP TSG-RAN WG4 Meeting #88	R4-1809699

Gothenburg, Sweden, 20 – 24 August 2018

Agenda Item:	7.9.4.2.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on MU for NR FR1 OTA receiver dynamic range requirement

Document for:	Approval

# 1.	Introduction

The MU for eAAS and NR FR1 OTA receiver in-band directional requirements were discussed in RAN4 AH-1807 and the way forward were agreed [1], where it was agreed that ‘If no further analysis on NR FR1 according to the agreed timeline, the same MU budget as eAAS (3GHz < f ≦ 4.2GHz) can be reused for NR FR1 (3GHz < f ≦ 6GHz).’

This contribution provides our proposal on the MU for NR FR1 OTA receiver dynamic range requirement, according to the agreed way forward in [1] and our proposals in [2].

# 2.	Discussion

It was agreed in [1] that the same MU of 0.3dB as for the conducted case can be assumed for eAAS and NR FR1 OTA receiver dynamic range requirement for operating bands below 4.2GHz.

For operating bands above 4.2GHz, we propose in [2] that:

1.	For the MU variables independent of the radiated measurement methods, use the values specified in TS 36.141 [3] for 4.2GHz < f ≦ 6GHz.

2.	For the MU variables dependent of the radiated measurement methods, use the values included in [4] for 4.2GHz < f ≦ 6GHz, and reuse the 3GHz < f ≦ 4.2GHz values in TR 37.842 [5] for 4.2GHz < f ≦ 6GHz for those not included in [4], unless other values are proposed in this meeting.

Using the above proposals, the same MU of 0.3dB as for the conducted case can be assumed for NR FR1 OTA receiver dynamic range requirement for operating bands above 4.2GHz, as the 0.3dB MU is specified for all operating bands (including Band 46 which is above 4.2GHz) in [3].

Consequently, the test system specific measurement uncertainty values for NR FR1 OTA receiver dynamic range test can be obtained in Table 1 below.

# Table 1: Test system specific measurement uncertainty values for NR FR1 OTA receiver dynamic range test

|  | Expanded uncertainty ue [dB] |  |  |
| --- | --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2GHz | 4.2GHz < f ≦ 6GHz |
| Indoor Anechoic Chamber | 0.3 | 0.3 | 0.3 |
| Compact Antenna Test Range | 0.3 | 0.3 | 0.3 |
| Common maximum accepted test system uncertainty | 0.3 | 0.3 | 0.3 |

# 3.	Conclusion

This contribution has provided our proposal on the MU for NR FR1 OTA receiver dynamic range requirement, according to the agreed way forward in [1] and our proposals in [2].

Proposal:

Adopt the following table as the MU for NR FR1 OTA receiver dynamic range test.

# Table 1: Test system specific measurement uncertainty values for NR FR1 OTA receiver dynamic range test

|  | Expanded uncertainty ue [dB] |  |  |
| --- | --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2GHz | 4.2GHz < f ≦ 6GHz |
| Indoor Anechoic Chamber | 0.3 | 0.3 | 0.3 |
| Compact Antenna Test Range | 0.3 | 0.3 | 0.3 |
| Common maximum accepted test system uncertainty | 0.3 | 0.3 | 0.3 |

# References

[1]	R4-1809428, “WF on MU of receiver directional requirements”, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, NTT DOCOMO, INC.

[2]	R4-1809697, “Proposals on MU for NR FR1 OTA receiver requirement for operating bands above 4.2GHz”, Nokia, Nokia Shanghai Bell.

[3]	3GPP TS 36.141 v15.3.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing”.

[4]	R4-1805879, “Test Equipment uncertainty values for FR1”, Keysight Technologies.