3GPP TSG-RAN WG4 Meeting #88	R4-1809704

Gothenburg, Sweden, 20 – 24 August 2018

Agenda Item:	7.9.4.2.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on MU for NR FR1 OTA receiver out-of-band blocking requirement

Document for:	Approval

# 1.	Introduction

The MU for eAAS and NR FR1 OTA receiver in-band directional requirements were discussed in RAN4 AH-1807 and the way forward were agreed [1], while the MU for OTA receiver out-of-band blocking requirement was discussed without conclusion [2].

This contribution provides our proposal on the MU for NR FR1 OTA receiver out-of-band blocking requirement, according to the agreed way forward in [1] and comments received during discussion on [2].

# 2.	Discussion

It was agreed in [1] that the MU for eAAS and NR FR1 OTA receiver in-band directional requirements should be calculated as:

MU=MUconductedwanted2+MUconductedint2+MUEIS2-MUTestEquipment2-2*MUmatching2+ACLR effect

And we propose in [2] that since the interfering signal for the OTA receiver out-of-band blocking requirement is an out-of-band CW signal, the impact of interferer broadband noise should be considered instead of ACLR effect as stated in TS 36.141 [3]. Hence, the equation above should be modified for the OTA receiver out-of-band blocking requirement as:

MU=MUconductedwanted2+MUconductedint2+MUEIS2-MUTestEquipment2-2*MUmatching2+broadband noise effect

For operating bands above 4.2GHz, we propose in [4] that:

1.	For the MU variables independent of the radiated measurement methods, use the values specified in [3] for 4.2GHz < f ≦ 6GHz.

2.	For the MU variables dependent of the radiated measurement methods, use the values included in [5] for 4.2GHz < f ≦ 6GHz, and reuse the 3GHz < f ≦ 4.2GHz values in TR 37.842 [6] for 4.2GHz < f ≦ 6GHz for those not included in [5], unless other values are proposed in this meeting.

Using the above proposals, the MU for NR FR1 OTA receiver out-of-band blocking requirement can be calculated as shown in Tables 1 and 2 below for Indoor Anechoic Chamber and Compact Antenna Test Range, respectively.

Table 1: MU for Indoor Anechoic Chamber

| Test System Uncertainty | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2GHz | Standard uncertainty ui [dB] 4.2GHz < f ≦ 6GHz (Wanted signal) 4.2Hz < f ≦ 12.75GHz (Interferer signal) |
| --- | --- | --- | --- |
| 1.96σ MUconductedwanted (Wanted signal level error in [3]) | 0.7 | 1 | 1.5 |
| 1.96σ MUconductedint (Interferer signal level error in [3]) | 1 | 1.2 | 3 |
| MUEIS (Combined standard uncertainty in [5, 6]) | 0.62 | 0.64 | 0.73 |
| MUTestEquipment (Uncertainty of the RF signal generator in [5, 6]) | 0.46 | 0.46 | 0.58 |
| MUmatching (Impedance mismatch in the transmitting chain in [5, 6]) | 0.1 | 0.16 | 0.16 |
| 1.96σ Broadband noise effect (Impact of interferer broadband noise in [3]) | 0.1 | 0.1 | 0.1 |
| Combined standard uncertainty (1σ) | 0.79 | 0.94 | 1.80 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) | 1.54 | 1.83 | 3.54 |

Table 2: MU for Compact Antenna Test Range

| Test System Uncertainty | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2GHz | Standard uncertainty ui [dB] 4.2GHz < f ≦ 6GHz (Wanted signal) 4.2GHz < f ≦ 12.75GHz (Interferer signal) |
| --- | --- | --- | --- |
| 1.96σ MUconductedwanted (Wanted signal level error in [3]) | 0.7 | 1 | 1.5 |
| 1.96σ MUconductedint (Interferer signal level error in [3]) | 1 | 1.2 | 3 |
| MUEIS (Combined standard uncertainty in [5, 6]) | 0.68 | 0.71 | 0.79 |
| MUTestEquipment (Uncertainty of the RF signal generator in [5, 6]) | 0.46 | 0.46 | 0.58 |
| MUmatching (Standing wave between DUT and test range antenna in [5, 6]) | 0.15 | 0.15 | 0.15 |
| 1.96σ Broadband noise effect (Impact of interferer broadband noise in [3]) | 0.1 | 0.1 | 0.1 |
| Combined standard uncertainty (1σ) | 0.82 | 0.99 | 1.83 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) | 1.61 | 1.94 | 3.59 |

During the discussion on [2], it was questioned where the 3 dB MUconductedint for frequency above 3GHz in TS 37.145-1 [7] came from, and it was clarified that this value was taken from TS 37.141 [8] considering multi-RAT deployment. Considering TS 38.141-2 [9] is specified for single-RAT NR deployment, the values in [3] are more appropriate and thus are used here. Consequently, the test system specific measurement uncertainty values for NR FR1 OTA receiver out-of-band blocking test can be obtained in Table 3 below.

# Table 3: Test system specific measurement uncertainty values for NR FR1 OTA receiver out-of-band blocking test

|  | Expanded uncertainty ue [dB] |  |  |
| --- | --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2GHz | 4.2GHz < f ≦ 6GHz (Wanted signal) 4.2GHz < f ≦ 12.75GHz (Interferer signal) |
| Indoor Anechoic Chamber | 1.54 | 1.83 | 3.54 |
| Compact Antenna Test Range | 1.61 | 1.94 | 3.59 |
| Common maximum accepted test system uncertainty | 1.6 | 1.9 | 3.6 |

# 3.	Conclusion

This contribution has provided our proposal on the MU for NR FR1 OTA receiver out-of-band blocking requirement, according to the agreed way forward in [1] and comments received during discussion on [2].

Proposal:

Adopt the following table as the MU for NR FR1 OTA receiver out-of-band blocking test.

# Table 3: Test system specific measurement uncertainty values for NR FR1 OTA receiver out-of-band blocking test

|  | Expanded uncertainty ue [dB] |  |  |
| --- | --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2GHz | 4.2GHz < f ≦ 6GHz (Wanted signal) 4.2GHz < f ≦ 12.75GHz (Interferer signal) |
| Indoor Anechoic Chamber | 1.54 | 1.83 | 3.54 |
| Compact Antenna Test Range | 1.61 | 1.94 | 3.59 |
| Common maximum accepted test system uncertainty | 1.6 | 1.9 | 3.6 |

# References

[1]	R4-1809428, “WF on MU of receiver directional requirements”, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, NTT DOCOMO, INC.

[2]	R4-1808671, “Proposals on MU for OTA receiver out-of-band blocking requirement”, Nokia, Nokia Shanghai Bell.

[3]	3GPP TS 36.141 v15.3.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing”.

[4]	R4-1809697, “Proposals on MU for NR FR1 OTA receiver requirement for operating bands above 4.2GHz”, Nokia, Nokia Shanghai Bell.

[5]	R4-1805879, “Test Equipment uncertainty values for FR1”, Keysight Technologies.

[6]		3GPP TR 37.842 v13.2.0, “E-UTRA and UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)”.

[7]	3GPP TS 37.145-1 v15.0.0, “Active Antenna System (AAS) Base Station (BS) conformance testing; Part 1: Conducted conformance testing”.

[8]	3GPP TS 37.141 v15.3.0, “E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing”.

[9]	R4-1810576, “Draft TS 38.141-2 v0.3.0”, Huawei, HiSilicon.