3GPP TSG-RAN WG4 Meeting #88	R4-1809851

Gothenburg, SE, 20th – 24th Aug, 2018

Agenda Item:	6.6.4.1

Source: 	Intel Corporation

Title: 	PDSCH Demodulation requirements under fading conditions with 1024QAM

Document for:	Discussion

# Introduction

In RAN4 #87 the way forward on demodulation and CSI requirements for 1024QAM was approved [1]. For requirements with 1024QAM in fading channels there were a few remaining issues. In this contribution we provide our inputs on the remaining issues for requirements under fading conditions with 1024QAM.

# Discussion

In RAN4#87 the following parameters were agreed for 1024QAM requirements in fading channel conditions:

Table 1: Common Parameters for 1024 Demodulation requirements in fading conditions

| Parameters | Value |
| --- | --- |
| Bandwidth | 10MHz |
| Channel model | EPA5 |
| CFI (OFDM symbol for control channel) | 1 |
| Duplex mode | FDD |
| Tx EVM | 2% |

Table 2: Test cases for 1024 Demodulation requirements in fading conditions

| No. | TM | PMI feedback (pre-coding) | MIMO layer | MCS | Antenna config. |
| --- | --- | --- | --- | --- | --- |
| 1 | TM4 | Wideband PMI Feedback (PUSCH 3-1, Reporting interval =1) | 1-layer | MCS#23 (with 52752 bits) | 4x2 Low |
| 2 | TM4 | Wideband PMI feedback (PUSCH 3-1, Reporting interval =1) | 2-layer | MCS#23 (with 52752 bits) | 4x4 Low |
| 3 | TM9 | Option 1: Wideband PMI Feedback Option 2: Per-PRG Random Precoder | 1-layer | MCS#23 (with 52752 bits) | 4x2 Low |
| 4 | TM9 | Option 1: Wideband PMI Feedback Option 2: Per-PRG Random Precoder | 2-layer | MCS#23 (with 52752 bits) | 4x4 Low |

## Test Cases in TM4

For the TM4 test cases agreed we provide testcase parameters and simulation results:

Table 3: Testcase parameters for 1,2 layer WB PMI test in TM4

| Parameter |  |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | -6 | -6 |
|  |  |  | dB | -6 (Note 1) | -6 (Note 1) |
|  |  |  | dB | 0 | 3 |
| at antenna port |  |  | dBm/15kHz | -98 | -98 |
| Precoding granularity |  |  | PRB | 50 | 50 |
| PMI delay (Note 2) |  |  | ms | 8 | 8 |
| Reporting interval |  |  | ms | 1 | 1 |
| Reporting mode |  |  |  | PUSCH 3-1 | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  |  | 0000000000000000000000000000000011111111111111110000000000000000 | 0000000000000000000000000000000011111111111111110000000000000000 |
| PDSCH transmission mode |  |  |  | 4 |  |
| Number of OFDM symbols for PDCCH per component carrier |  |  | OFDM symbol | 1 | 1 |
| Band-width |  |  | MHz | 10 | 10 |
| Propagation condition |  |  |  | EPA5 | EPA5 |
| Correlation matrix and antenna config. |  |  |  | 4x2 Low | 4x4 Low |
| Number of Layers |  |  |  | 1 | 2 |
|  | Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |  |

Simulation results for TM4

| Test 1 | Test 2 |
| --- | --- |

Figure 1: TM4 WB PMI Simulation results

70% of Max TP is achieved at 21.7 dB for Test1 and at 24.5dB for Test2 with no impairments.

Observation #1: With no impairments for TM4 tests with 1024QAM 70% of max TP is achieved at 21.7 dB and 24.5dB for Test1 and Test2 respectively.

## Test Cases in TM9

For TM9 the test parameters were discussed and the options to downselect from are:

Option 1: Wideband PMI Feedback

Option 2: Per-PRG Random Precoder

In TM9 the testcases are defined with random precoder rather than with feedback PMI. For 1024QAM tests in TM9 we recommend using random precoder to define demodulation requirements.

| Test 1 | Test 2 |
| --- | --- |

Figure 2: TM9 Simulation results with random precoder

| Test 1 | Test 2 |
| --- | --- |

Figure 3: TM9 Simulation results with WB PMI

Proposal #1: For 1024QAM tests in fading channel, define tests in TM9 with random precoder

# Conclusion

In this paper we provide simulation results for 1024QAM tests in fading conditions for TM4 and TM9. We have the following observations and proposals:

Observation #1: With no impairments for TM4 tests with 1024QAM 70% of max TP is achieved at 21.7 dB and 24.5dB for Test1 and Test2 respectively.

Proposal #1: For 1024QAM tests in fading channel, define tests in TM9 with random precoder

# References

- R4-1807966, Way forward on demodulation and CSI requirements for high capacity stationary wireless link and 1024QAM, Huawei, HiSilicon