3GPP TSG-RAN4 Meeting #88	R4-1809899

# Gothenburg, Sweden, 20th Aug – 24th Aug, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test cases for interruptions at transitions in EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-8-6 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no test cases for interruptions at transitions in EN-DC |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add three test cases: 1. E-UTRAN FDD – NR FR1 FDD interruptions at transitions between active and non-active during DRX in synchronous EN-DC 2. E-UTRAN TDD – NR FR1 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC 3. E-UTRAN TDD – NR FR2 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases for interruptions at transitions in EN-DC will be missed in TS38.133. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section A.8.2.1, A.8.2.2 and A.8.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change #1>>

A.8.2.1	E-UTRAN FDD – NR FR1 FDD interruptions at transitions between active and non-active during DRX in synchronous EN-DC

A.8.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that when LTE PCell is in DRX and NR PSCell is in non-DRX, NR PSCell interruptions due to transitions from active to non-active and from non-active to active during LTE PCell DRX the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for NR PSCell in EN-DC specified in TS38.133 section 8. 2.1.2.

The test parameters are given in Table A.8.2.1.1-1, A. 8.2.1.1-2 and A. 8.2.1.1-3 below. In the test there are two cells: Cell1 and Cell2. Cell1 is LTE PCell on the FDD primary component (RF channel 1) and Cell2 is NR PSCell on the FR1 FDD secondary component (RF channel 2).. The test consists of one time period, with duration of T1. During T1, NR PSCell is continuously scheduled in DL while LTE PCell is not scheduled and has DRX configured. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell1 and Cell2. Cell1 shall be configured as LTE PCell and Cell2 shall be configured as NR PSCell. Prior to start of T1 the DRX inactivity timer for the LTE PCell have already expired. During T1 the UE shall be continuously scheduled on NR PSCell while not scheduled on LTE PCell. PDCCH indicating a new transmission on PSCell shall be sent continuously during the whole time duration to ensure UE would not enter DRX state on PSCell.

Table A.8.2.1.1-1: General test parameters for E-UTRAN FDD – NR FR1 FDD interruptions at transitions between active and non-active during DRX in synchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | One is E-UTRAN RF channel and the other is NR RF channel |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| SCS for NR PSCell | KHz | 15 |  |
| SSB periodicity | ms | 20 |  |
| CP length |  | Normal | Applicable to cell1 and cell 2 |
| DRX |  | ON | DRX related parameters are defined in Table A. 8.2.1.1-3 |
| Measurement gap pattern Id |  | OFF |  |
| T1 | s | 10 |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.x.x.x. Note 2: A UE capable of both synchronous and asynchronous EN-DC operations is only required to pass this test case in accordance with the principle defined in section A.x.x. |  |  |  |

Table A.8.2.1.1-2: Cell specific test parameters for E-UTRAN FDD – NR FR1 FDD interruptions at transitions between active and non-active during DRX in synchronous EN-DC

| Parameter | Unit | Cell1 | Cell2 |
| --- | --- | --- | --- |
|  |  | T1 | T1 |
| RF Channel Number |  | 1 | 2 |
| BWchannel | MHz | 10MHz: NRB,c = 50 | 10MHz: NRB,c = 50 |
| PSCell PDSCH parameters: DL Reference Measurement Channel |  | 10MHz: R.3 FDD | 10MHz: X.X FDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 10MHz: R.6 FDD | 10MHz: X.X FDD |
| OCNG Patterns |  | 10MHz: OP.10 FDD | 10MHz: X.X FDD |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_PB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_PB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Note 2 | dBm/15 KHz | -101 | -101 |
|  | dB | 19 | 19 |
|  | dB | 19 | 19 |
| RSRP Note 3 | dBm/15 KHz | -82 | -82 |
| SCH_RP Note 3 | dBm/15 KHz | -82 | -82 |
| Io Note 3 | dBm/Ch BW | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low |
| Time offset to cell1 Note 4 | s | - | 33 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |

Table A.8.2.1.1-3: DRX-Configuration for E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC

| Field | Cell2 | Comment |
| --- | --- | --- |
|  | Value |  |
| onDurationTimer | ms1 | Specified in clause 6.3.2 in TS 38.331 |
| drx-InactivityTimerNote 1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 |  |
| drx-RetransmissionTimerUL | sl1 |  |
| longDRX-CycleStartOffset | ms160 |  |
| shortDRX | disable |  |

A.8.2.1.2	Test Requirements

The UE shall be continuously scheduled in NR PSCell during the entire length of T1. UE shall not be scheduled in LTE PCell during T1. During the time duration T1 the UE shall transmit at least 99% of ACK/NACK on NR PSCell.

Interruption on NR  PSCell shall not exceed X slot as defined in table 8.2.1.2-1.

Table 8.2.1.2-1 Interruption length X at transition between active and non-active during DRX

|  | NR Slot length (ms) | Interruption length X |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 3 |  |
| 3 | 0.125 | 5 |  |

The rate of correct events observed during repeated tests shall be at least 90%.

<< End of Change #1>>

<< Start of Change #2>>

A.8.2.2	E-UTRAN TDD – NR FR1 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC

A.8.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that when LTE PCell is in DRX and NR PSCell is in non-DRX, NR PSCell interruptions due to transitions from active to non-active and from non-active to active during LTE PCell DRX the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for NR PSCell in EN-DC specified in TS38.133 section 8. 2.1.2.

The test parameters are given in Table A.8.2.2.1-1, A. 8.2.2.1-2 and A. 8.2.2.1-3 below. In the test there are two cells: Cell1 and Cell2. Cell1 is LTE PCell on the TDD primary component (RF channel 1) and Cell2 is NR PSCell on the FR1 TDD secondary component (RF channel 2). The test consists of one time period, with duration of T1. During T1, NR PSCell is continuously scheduled in DL while LTE PCell is not scheduled and has DRX configured. Prior to the start of the time duration T1, Cell1 shall be configured as LTE PCell and Cell2 shall be configured as NR PSCell. Prior to start of T1 the DRX inactivity timer for the LTE PCell have already expired. During T1 the UE shall be continuously scheduled on NR PSCell while not scheduled on LTE PCell. PDCCH indicating a new transmission on PSCell shall be sent continuously during the whole time duration to ensure UE would not enter DRX state on PSCell.

Table A.8.2.2.1-1: General test parameters for E-UTRAN TDD – NR FR1 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | One is E-UTRAN RF channel and the other is NR RF channel |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| Slot format |  | 46 | As specified in table 4.3.2-3 in TS 38.211. Applicable to cell 2 |
| SCS for NR PSCell | KHz | 15 | Applicable to cell 2 |
| SSB periodicity | ms | 20 |  |
| CP length |  | Normal | Applicable to cell1 and cell 2 |
| DRX |  | ON | DRX related parameters are defined in Table A. 8.2.1.1-3 |
| Measurement gap pattern Id |  | OFF |  |
| T1 | s | 10 |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.x.x.x. Note 2: A UE capable of both synchronous and asynchronous EN-DC operations is only required to pass this test case in accordance with the principle defined in section A.x.x. |  |  |  |

Table A.8.2.2.1-2: Cell specific test parameters for E-UTRAN TDD – NR FR1 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC

| Parameter | Unit | Cell1 | Cell2 |
| --- | --- | --- | --- |
|  |  | T1 | T1 |
| RF Channel Number |  | 1 | 2 |
| BWchannel | MHz | 10MHz: NRB,c = 50 | 10MHz: NRB,c = 50 |
| PSCell PDSCH parameters: DL Reference Measurement Channel |  | 10MHz: R.0 TDD | 10MHz: X.X TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 10MHz: R.6 TDD | 10MHz: X.X TDD |
| OCNG Patterns |  | 10MHz: OP.1 TDD | 10MHz: X.X TDD |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_PB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_PB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Note 2 | dBm/15 KHz | -101 | -101 |
|  | dB | 19 | 19 |
|  | dB | 19 | 19 |
| RSRP Note 3 | dBm/15 KHz | -82 | -82 |
| SCH_RP Note 3 | dBm/15 KHz | -82 | -82 |
| Io Note 3 | dBm/Ch BW | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low |
| Time offset to cell1 Note 4 | s | - | 500 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |

Table A.8.2.2.1-3: DRX-Configuration for E-UTRAN TDD – NR FR1 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC

| Field | Cell2 | Comment |
| --- | --- | --- |
|  | Value |  |
| onDurationTimer | ms1 | Specified in clause 6.3.2 in TS 38.331 |
| drx-InactivityTimerNote 1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 |  |
| drx-RetransmissionTimerUL | sl1 |  |
| longDRX-CycleStartOffset | ms160 |  |
| shortDRX | disable |  |

A.8.2.2.2	Test Requirements

The UE shall be continuously scheduled in NR PSCell during the entire length of T1. UE shall not be scheduled in LTE PCell during T1. During the time duration T1 the UE shall transmit at least 99% of ACK/NACK on NR PSCell.

Interruption on NR  PSCell shall not exceed X slot as defined in table 8.2.1.2-1.

Table 8.2.2.2-1 Interruption length X at transition between active and non-active during DRX

|  | NR Slot length (ms) | Interruption length X |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 3 |  |
| 3 | 0.125 | 5 |  |

The rate of correct events observed during repeated tests shall be at least 90%.

<< End of Change #2>>

<< Start of Change #3>>

A.8.2.3	E-UTRAN TDD – NR FR2 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC

A.8.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that when LTE PCell is in DRX and NR PSCell is in non-DRX, NR PSCell interruptions due to transitions from active to non-active and from non-active to active during LTE PCell DRX the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for NR PSCell in EN-DC specified in TS38.133 section 8. 2.1.2.

The test parameters are given in Table A.8.2.3.1-1, A. 8.2.3.1-2 and A. 8.2.3.1-3 below. In the test there are two cells: Cell1 and Cell2. Cell1 is LTE PCell on the TDD primary component (RF channel 1) and Cell2 is NR PSCell on the FR2 TDD secondary component (RF channel 2). The test consists of one time period, with duration of T1. During T1, NR PSCell is continuously scheduled in DL while LTE PCell is not scheduled and has DRX configured. Prior to the start of the time duration T1, Cell1 shall be configured as LTE PCell and Cell2 shall be configured as NR PSCell. Prior to start of T1 the DRX inactivity timer for the LTE PCell have already expired. During T1 the UE shall be continuously scheduled on NR PSCell while not scheduled on LTE PCell. PDCCH indicating a new transmission on PSCell shall be sent continuously during the whole time duration to ensure UE would not enter DRX state on PSCell.

Table A.8.2.3.1-1: General test parameters for E-UTRAN TDD – NR FR2 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | One is E-UTRAN RF channel and the other is NR RF channel |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| Slot format |  | 46 | As specified in table 4.3.2-3 in TS 38.211. Applicable to cell 2 |
| SCS for NR PSCell | KHz | 120 | Applicable to cell 2 |
| SSB periodicity | ms | 20 |  |
| CP length |  | Normal | Applicable to cell1 and cell 2 |
| DRX |  | ON | DRX related parameters are defined in Table A. 8.2.1.1-3 |
| Measurement gap pattern Id |  | OFF |  |
| T1 | s | 10 |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.x.x.x. Note 2: A UE capable of both synchronous and asynchronous EN-DC operations is only required to pass this test case in accordance with the principle defined in section A.x.x. |  |  |  |

Table A.8.2.3.1-2: Cell specific test parameters for E-UTRAN TDD – NR FR2 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC

| Parameter | Unit | Cell1 | Cell2 |
| --- | --- | --- | --- |
|  |  | T1 | T1 |
| RF Channel Number |  | 1 | 2 |
| BWchannel | MHz | 10MHz: NRB,c = 50 | 100MHz |
| PSCell PDSCH parameters: DL Reference Measurement Channel |  | 10MHz: R.0 TDD | 100MHz: X.X TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 10MHz: R.6 TDD | 100MHz: X.X TDD |
| OCNG Patterns |  | 10MHz: OP.1 TDD | 100MHz: X.X TDD |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_PB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_PB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Note 2 | dBm/15 KHz | -101 | -101 |
|  | dB | 19 | 19 |
|  | dB | 19 | 19 |
| RSRP Note 3 | dBm/15 KHz | -82 | -82 |
| SCH_RP Note 3 | dBm/15 KHz | -82 | -82 |
| Io Note 3 | dBm/Ch BW | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low |
| Time offset to cell1 Note 4 | s | - | 500 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |

Table A.8.2.3.1-3: DRX-Configuration for E-UTRAN TDD – NR FR2 TDD interruptions at transitions between active and non-active during DRX in asynchronous EN-DC

| Field | Cell2 | Comment |
| --- | --- | --- |
|  | Value |  |
| onDurationTimer | ms1 | Specified in clause 6.3.2 in TS 38.331 |
| drx-InactivityTimerNote 1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 |  |
| drx-RetransmissionTimerUL | sl1 |  |
| longDRX-CycleStartOffset | ms160 |  |
| shortDRX | disable |  |

A.8.2.3.2	Test Requirements

The UE shall be continuously scheduled in NR PSCell during the entire length of T1. UE shall not be scheduled in LTE PCell during T1. During the time duration T1 the UE shall transmit at least 99% of ACK/NACK on NR PSCell.

Interruption on NR  PSCell shall not exceed X slot as defined in table 8.2.1.2-1.

Table 8.2.3.2-1 Interruption length X at transition between active and non-active during DRX

|  | NR Slot length (ms) | Interruption length X |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 3 |  |
| 3 | 0.125 | 5 |  |

The rate of correct events observed during repeated tests shall be at least 90%.

<< End of Change #3>>