3GPP TSG-RAN WG4 Meeting #88                	R4-1809948

Gothenburg, Sweden, 20 Aug – 24 Aug 2018

Agenda item:	6.5.7.1

Source:	Samsung

Title:	Initial simulation results for eFeMTC

Document for:	Discussion

# 1	Introduction

During RAN4#87 Busan meeting, WF about the new BS performance requirements for eFeMTC was agreed as [1]:

  - RAN4 will introduce new PUSCH demodulation requirements with subPRB transmission

    - RAN4 will specify the demodulation requirements with 2 of 3 sub-carriers pi/2 BPSK

      - FFS: CE Mode A and/or CE Mode B

    - FFS: 3/6 subcarriers with QPSK demodulation requirements

    - RAN4 will wait for RAN1 conclustion on the detailed specfication

  - RAN4 will introduce new PUSCH demodulation requirements for CE Mode A with the following condition. Interested companies are encouraged provide the simulation results for alignment.

    - 8 PUSCH repetitions

    - FRC A3-2 (QPAK1/3, TBS =600bit, 6PRB)

    - Bandwidth 3, 5, 10, 15, and 20MHz

    - ETU200

    - Antenna configuration: 1x2

In this contribution, the simulation results of PUSCH CE Mode A with high Doppler spread was provided for initial alignment purpose.

# 2	Simulation results

In this section, we provide our initial result for CE Mode A. The target SNR with 70% throughput is selected as the performance metric. For the simulation assumption, we just capture the agreed WF as Table 1.

Table 1: Simulation Assumption for eFeMTC PUSCH with CE Mode A

| Parameter | Value |
| --- | --- |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 2 |
| Propagation condition | ETU200 |
| Cyclic prefix | Normal |
| Reference receiver | MRC |
| Channel bandwidth | 3, 5, 10, 15 and 20MHz |
| Maximum number of HARQ transmissions | 4 |
| Number of PUSCH repetitions | 8 |
| Frequency hopping interval | 4 |
| FRC | A3-2 in 361.04 |

The simulation results are shown in Table 2.

Table 2: Simulation results for PUCSH, 10MHz channel bandwidth, CE Mode A, 1Tx

| BW | SNR(dB) (70%TP) |
| --- | --- |
| 3 | -8.97 |
| 5 | -8.96 |
| 10 | -8.98 |
| 15 | -8.99 |
| 20 | -8.97 |

The BLER performance are shown in Figure 1

Figure 1 BLER Performance with PUSCH CE Mode A

From the results, we can observe that there is no obvious frequency diversity gain achieved due to the frequency hopping compared with the performance of different bandwidth, due to high Doppler frequency spread.

# 3	Conclusion

In this contribution, the initial simulation results of PUSCH with CE Mode A under high Doppler frequency spread for alignment are presented.

# References

R4-1807976, WF on BS demodulation performance requirement for eFeMTC , Ericsson