3GPP TSG-RAN WG4 Meeting #88	R4-1810012

# Gothenburg, Sweden, Aug. 20th – 24th, 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on RRC Re-establishment Requirements in NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-07-06 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | 1. To supplement the requirements when the serving cell Ês/Iot is less than the threshold [X]dB. - UE will only monitor the configured frequency which SMTC periodicity is equal or less than 20ms. - The target cell will always be unknown cell. 2. To clarify the definition of SMTC and Nfreq when serving cell signal quality is less than the threshold [X]dB. 3. To simplify the expression in the specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Update the requirement table when serving cell signal quality is less than threshold [X]dB in the requirement table. 2. Add the definition of intra-frequency SMTC. 3. Update the definition of inter-frequency SMTCi and Nfreq to monitor UE’s behaviour when serving cell signal quality is less than the threshold [X]dB. 4. Simplify the SMTC and Nfreq defintion |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1.The requirement will be incomplete. 2.The defintion of SMTCi and Nfreq will be not accurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGES----------------------------

6.2.1	SA: RRC Re-establishment

6.2.1.1	Introduction

This clause contains requirements on the UE regarding RRC connection re-establishment procedure. RRC connection re-establishment is initiated when a UE in RRC_CONNECTED state loses RRC connection due to any of failure cases, including radio link failure, handover failure, and RRC connection reconfiguration failure. The RRC connection re-establishment procedure is specified in clause 5.3.7 of TS 38.331 [2].

The requirements in this clause are applicable for RRC connection re-establishment to NR cell.

6.2.1.2	Requirements

In RRC connected mode the UE shall be capable of sending RRCConnectionReestablishmentRequest message within Tre-establish_delay seconds from the moment it detects a loss in RRC connection. The total RRC connection delay (Tre-establish_delay) shall be less than:

Tre-establish_delay=TUL_grant +TUE_re-establish_delay     ms

TUL_grant: It is the time required to acquire and process uplink grant from the target PCell. The uplink grant is required to transmit RRCConnectionReestablishmentRequest message.

The UE re-establishment delay (TUE_re-establish_delay) is specified in clause 6.2.1.2.1.

6.2.1.2.1	UE Re-establishment delay requirement

The UE re-establishment delay (TUE_re-establish_delay) is the time between the moments when any of the conditions requiring RRC re-establishment as defined in clause 5.3.7 in TS 38.331 [2] is detected by the UE and when the UE sends PRACH to the target PCell. The UE re-establishment delay (TUE_re-establish_delay) requirement shall be less than:

TUE_re-establish_delay=50+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH     ms

The target NR cell shall be considered detectable when:

-	SS-RSRP related side conditions given in Section 10.x are fulfilled for a corresponding NR Band,

-	SCH_RP and SCH Ês/Iot according to Annex TBD for a corresponding NR Band.

Tidentify_intra_NR: It is the time to identify the target intra-frequency NR cell and it depends on whether the target NR cell is known cell or unknown cell and on the frequency range (FR) of the target NR cell. If the UE is not configured with intra-frequency NR carrier for RRC re-establishment then Tidentify_intra_NR=0; otherwise Tidentify_intra_NR shall not exceed the values defined in table 6.2.1.2.1-1.

Tidentify_inter_NR,i: It is the time to identify the target inter-frequency NR cell on inter-frequency carrier i configured for RRC re-establishment and it depends on whether the target NR cell is known cell or unknown cell and on the frequency range (FR) of the target NR cell. Tidentify_inter_NR,i shall not exceed the values defined in table 6.2.1.2.1-2.

TSMTC: It is the STMC periodicity configured for the intra-frequency carrier when serving cell SCH Ês/Iot is equal or larger than [X]dB. UE assumes the default SMTC periodicity as 20ms when serving cell SCH Ês/Iot is less than [X]dB.

TSMTC,i: It is the SMTC periodicity configured for the inter-frequency carrier i when serving cell SCH Ês/Iot is equal or larger than [X]dB. UE assumes the default SMTC periodicity as 20ms when serving cell SCH Ês/Iot is less than [X]dB.

TSI-NR  = It is the time required for receiving all the relevant system information according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for the target PCell.

TPRACH = It is the delay caused due to the random access procedure when sending random access to the target NR cell. The delay depends on the PRACH configuration defined in Table 6.3.3.2-2 [6] or Table 6.3.3.2-3 [6] for FR1 and in Table 6.3.3.2-4 [6] for FR2.

Nfreq: It is the total number of NR frequency layers to be monitored for RRC re-establishment when serving cell SCH Ês/Iot is equal or larger than [X]dB; it should be the number of NR frequency layers configured with SMTC periodicity equal or less than 20ms when serving cell SCH Ês/Iot is less than [X]dB; Nfreq = 1 if the target PCell is known.

There is no requirement if the target cell does not contain the UE context.

In the requirement, the target FR1 cell is known if it has been meeting the relevant cell identification requirement during the last [5] seconds otherwise it is unknown.

Editor’s note: The current known cell’s definition could be re-visited after the Handover, PScell addition and Scell activiation’s known cell definition is agreed.

Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell SCH Ês/Iot (dB) | Frequency range (FR) of target NR cell | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ X |  | FR1 |  | MAX (200 ms, [5] x TSMTC) |  | MAX (800 ms, [10] x TSMTC) |  |
| ≥ X |  | FR2 |  | MAX (400 ms, K2 x [5] x TSMTC) |  | MAX (1000 ms, (K3 x [10]) x TSMTC)) |  |
| < X | FR1 | 800ms Note 1 |  |
| < X | FR2 | [1600ms or 3200ms] Note 1 |  |
| Note 1: UE assumes the target frequency SMTC periodicity as 20ms. |  |  |  |

Editor’s note: K2 and K3 are FFS and is the number of receiver beam sweeps required to measure/detect NR cell in FR2. The SCH Ês/Iot threshold (i.e, value of X) can be later decided based on RLM test cases.

Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell

| Serving cell SCH Ês/Iot (dB) | Frequency range (FR) of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ X | FR1 | MAX (200 ms, [6] x TSMTC, i) | MAX (800 ms, [13] x TSMTC, i) |
| ≥ X | FR2 | MAX (400 ms, K4 x [6] x TSMTC, i) | MAX (1000 ms, (K5 x [13]) x TSMTC, i)) |
| < X | FR1 | 800ms Note 1 |  |
| < X | FR2 | [1840ms or 3680ms] Note 1 |  |
| Note 1: UE assumes the target frequency SMTC periodicity as 20ms. |  |  |  |

Editor’s note: K4 and K5 are FFS and is the number of receiver beam sweeps required to measure/detect NR cell in FR2. The SCH Ês/Iot threshold (i.e, value of X) can be later decided based on RLM test cases.

----------------------END OF CHANGES----------------------------