3GPP TSG-RAN WG4 Meeting #88                                          R4-1810051

Gothenburg, SE, 20th – 24th August, 2018

Source:	Verizon, Ericsson, Qualcomm, Samsung, Nokia

Title:	TP for TR 38.716-01-01 NR Intra-band n260 CA

Agenda item:			9.1.3

Document for:	Approval

# 1. Introduction

Rel-16 LTE-NR CA combinations are listed in the latest WID [1]. This contribution is a text proposal for 38.716-01-01 to introduce NR intra-band n260 non-contiguous CA.

# 2. Text Proposal

## 8.9	NR intra-band non-contiguous n260 CA

### 8.9.1	Operating bands for NR

….

----- Unchanged sections omitted -----

### 8.9.x	Channel bandwidths per operating band for DC

Table 8.9.x-1: NR CA configurations and bandwidth combination fallback group defined for non-contiguous intra-band CA

|  |  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |
| NR configuration | Uplink CA configurations | SCS | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Fall-back group |
| CA_n260(5A) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  |  | 1000 |  |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  |  | 2000 |  |
| CA_n260(6A) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  | 1200 |  |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 |  |  | 2400 |  |
| CA_n260(7A) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  | 1400 |  |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 |  | 2800 |  |
| CA_n260(8A) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 1600 |  |
|  |  | 120 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 50, 100, 200, 400 | 3200 |  |

----- Unchanged sections omitted -----

----- Unchanged sections omitted -----

### 8.9.x	Channel bandwidths per operating band for DC

Table 8.9.x-1: NR CA configurations and bandwidth combination fallback group defined for non-contiguous intra-band CA

|  |  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |
| NR configuration | Uplink CA configurations | SCS | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Fall-back group |
| CA_n260(2G) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  |  |  | 400 | 3 |
|  |  | 120 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  |  |  | 400 |  |

### Table 8.9.X.-2: NR CA configurations and bandwidth combination fallback group defined for non-contiguous intra-band CA

|  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) | Fall-back group |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  |
| CA_n260(2A-G) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  | 1000 |  |
|  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |
| CA_n260(A-2G) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260(2G) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |  |  |  |  |  | 800 |  |
|  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |  |  |  |
| CA_n260(2A-2G) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |  |  | 1200 |  |
|  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |
| CA_n260(2A-2G-O) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  | 1400 |  |
|  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |
| CA_n260(3A-2G) | - | See CA_n260(3A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |  | 1600 |  |
|  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |  |  | See CA_n260(3A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |
| CA_n260(4A-G) | - | See CA_n260(4A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  | 1800 |  |
|  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  | See CA_n260(4A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |
| CA_n260(4A-2G) | - | See CA_n260(4A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in table above |  |  |  | 2000 |  |
|  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |  |  | See CA_n260(4A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |
| CA_n260(A-G) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  |  |  | 600 |  |
|  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(3A-G) | - | See CA_n260(3A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  | 1400 |  |
|  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  | See CA_n260(3A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |

----- Unchanged sections omitted -----

----- Unchanged sections omitted -----

### 8.9.x	Channel bandwidths per operating band for DC

Table 8.9.x-1: NR CA configurations and bandwidth combination fallback group defined for non-contiguous intra-band CA

|  |  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |
| NR configuration | Uplink CA configurations | SCS | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Fall-back group |
| CA_n260(2H) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  | 600 | 3 |
|  |  | 120 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  | 600 |  |

### Table 8.9.X.-2: NR CA configurations and bandwidth combination fallback group defined for non-contiguous intra-band CA

|  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) | Fall-back group |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) |  |  |
| CA_n260(A-2H) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260(2H) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |  |  |  |  | 1000 |  |
|  |  | See CA_n260(2H) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |  |  |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |
| CA_n260(2A-H) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  | See CA_n260H Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  | 1100 |  |
|  |  | See CA_n260H Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |
| CA_n260(2A-2H) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  | See CA_n260(2H) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |  |  |  | 1400 |  |
|  |  | See CA_n260(2H) Bandwidth Combination Fallback group 3 in table above |  |  |  |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |
| CA_n260(A-H) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260H Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  |  | 700 |  |
|  |  | See CA_n260H Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |  |  |  |  |

----- Unchanged sections omitted -----

----- Unchanged sections omitted -----

### 8.9.x	Channel bandwidths per operating band for DC

Table 8.9.x-1: NR CA configurations and bandwidth combination fallback group defined for non-contiguous intra-band CA

|  |  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |
| NR configuration | Uplink CA configurations | SCS | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Fall-back group |
| CA_n260(2O) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  |  |  | 400 | 4 |
|  |  | 120 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  |  |  | 400 |  |
| CA_n260(3O) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  | 600 | 4 |
|  |  | 120 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  | 600 |  |
| CA_n260(4O) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 800 | 4 |
|  |  | 120 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 800 |  |

### Table 8.9.X.-2: NR CA configurations and bandwidth combination fallback group defined for non-contiguous intra-band CA

|  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) | Fall-back group |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  |  |  | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) |  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  |
| CA_n260(2A-O) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |  |
|  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-2O) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260(2O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |
|  |  | See CA_n260(2O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |
| CA_n260(2G-O) | - | See CA_n260(2G) Bandwidth Combination Fallback group 3 in Table 2 above |  |  |  |  |  |  |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  | 600 |  |
|  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in Table 2 above |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-2O) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  | See CA_n260(2O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  |  |  |  | 1200 |  |
|  |  | See CA_n260(2O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |
| CA_n260(2A-3O) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  | See CA_n260(3O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  |  |  |  | 1400 |  |
|  |  | See CA_n260(3O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  |  |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |
| CA_n260(3A-2O) | - | See CA_n260(3A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  | See CA_n260(2O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  |  |  | 1600 |  |
|  |  | See CA_n260(2O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  | See CA_n260(3A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(4A-O) | - | See CA_n260(4A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  | 1800 |  |
|  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  | See CA_n260(4A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(4A-2O) | - | See CA_n260(4A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  | See CA_n260(2O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  | 2000 |  |
|  |  | See CA_n260(2O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  | See CA_n260(4A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |
| CA_n260(A-O) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 |  |
|  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(G-O) | - | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  | 400 |  |
|  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n260(A-G-O) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  | 800 |  |
|  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-G-O) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  | 1200 |  |
|  |  | See CA_n260G Bandwidth Combination Fallback group 3 in Table 5.5A.1-2 of 38.101-2 |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |
| CA_n260(A-2G-O) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260(2G) Bandwidth Combination Fallback group 3 in Table 2 above |  |  |  |  |  |  |  |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  | 1000 |  |
|  |  | See CA_n260(2G) Bandwidth Combination Fallback group 3 in Table 2 above |  |  |  |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |
| CA_n260(A-3O) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260(3O) Bandwidth Combination Fallback group 4 in Table 2 above |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |  |
|  |  | See CA_n260(3O) Bandwidth Combination Fallback group 4 in Table 2 above |  |  |  |  |  |  |  |  |  |  |  |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |  |
| CA_n260(3A-O) | - | See CA_n260(3A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  | 1400 |  |
|  |  | See CA_n260O Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  | See CA_n260(3A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### Table 8.9.X.-3: NR CA configurations and bandwidth combination fallback group defined for non-contiguous intra-band CA (Max #CC ≤ 12)

|  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) | Fall-back group |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  |
| CA_n260(A-4O) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260(4O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  |  |  |  | 1200 |  |
|  |  | See CA_n260(4O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |  |
| CA_n260(2A-4O) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  | See CA_n260(4O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  |  |  | 1600 |  |
|  |  | See CA_n260(4O) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |

----- Unchanged sections omitted -----

----- Unchanged sections omitted -----

### 8.9.x	Channel bandwidths per operating band for DC

Table 8.9.x-1: NR CA configurations and bandwidth combination fallback group defined for non-contiguous intra-band CA

|  |  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |
| NR configuration | Uplink CA configurations | SCS | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Fall-back group |
| CA_n260(2P) | - | 60 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  | 600 | 4 |
|  |  | 120 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 | 50, 100, 200 |  |  | 600 |  |

### Table 8.9.X.-2: NR CA configurations and bandwidth combination fallback group defined for non-contiguous intra-band CA

|  |  | NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) | Fall-back group |
|  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) |  |  |
| CA_n260(A-P) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260P Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  |  | 700 |  |
|  |  | See CA_n260P Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |
| CA_n260(2A-P) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  | See CA_n260P Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  |  |  |  |  |  |  | 1100 |  |
|  |  | See CA_n260P Bandwidth Combination Fallback group 4 in Table 5.5A.1-2 of 38.101-2 |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |
| CA_n260(A-2P) | - | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 | See CA_n260(2p) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  |  | 1000 |  |
|  |  | See CA_n260(2p) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  | See CA_n260A Bandwidth Combination in Table 5.3A.4-1 of 38.101-2 |  |  |  |  |
| CA_n260(2A-2P) | - | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  | See CA_n260(2p) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  |  |  | 1400 |  |
|  |  | See CA_n260(2p) Bandwidth Combination Fallback group 4 in table above |  |  |  |  |  |  | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |

----- Unchanged sections omitted -----

# Reference

[1]	RP- RP-181441, “New WID on EN-DC for 2 bands DL with 2 bands UL (1 LTE band + 1 NR band)”, NTT DOCOMO, INC.