3GPP TSG-RAN WG4 Meeting #88	R4-1810113

Gothenburg, Sweden, 20th August – 24th August 2018

Source: 	Skyworks Solutions, Inc.

Title: 		Inconsistencies in “Single UL allowed” EN-DC Dual Uplink Operation for NR FR1 (two bands)

Agenda Item:	7.5	System Parameters Maintenance	[NR_newRAT-Core]

Document for:	Approval

# Introduction

In Release 15.2.0 [1] two-band EN-DC combinations which may operate using the “Single Uplink (UL) allowed” scheme are captured in Table 5.2B.4.1-1 “Band combinations for EN-DC (two bands)”, and Table 7.3B.2.3.5.1-1 “Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)”. This contribution analyses the following inconsistencies:

- DC_1A_n77A, DC_3A_n77A, DC_3A_n78A, DC_3C_n78A single uplink operation is correctly captured in Table 5.2B.4.1-1, but not in Table 7.3B.2.3.5.1-1,

- In both Tables, DC_2A_n66A, DC_5A_n66A, DC_2A_n78A EN-DC combinations are not defined as “Single UL allowed” combinations. However, these combinations meet the agreed RAN4 eligibility criteria to allow single UL operation.

- DC_20A_n8A, is defined single UL allowed in Table 7.3B.2.3.5.1-1 but not in Table 5.2B.4.1-1.

This contribution proposes corrections to both Tables and is followed by a draft change request (CR).

# Discussion

## Reminder of RAN4 Single UL Allowed Assumptions [1]

For EN-DC configurations indicated by column “Single Uplink allowed” in Tables of section 5 [1], the UE may indicate capability of not supporting simultaneous dual and triple uplink operation due to possible intermodulation interference to its own downlink band:

- if the intermodulation order is 2 or,

- if the intermodulation order is 3 for the combinations when both operating bands are below 1 GHz or between 1695 MHz – 2690 MHz.

In case for the EN-DC configurations listed in Tables in section 5 [1] for which the intermodulation products caused by the dual and triple uplink operation fall into the receive band but do not interfere with the own downlink transmission as defined in Annex-A the UE is mandated to operate in dual and triple uplink mode. Single Uplink is also allowed for certain band combinations where intermodulation or reverse intermodulations product could create difficulty for meeting emission requirements.

Note that this definition is slightly ambiguous and is the subject of a revisited definition in contribution [2].

## Single UL allowed consistency check

The list of EN-DC combinations with inconsistent single UL allowed definition can be found in Table 1, with following rationale:

DC_1A_n77A, DC_3A_n77A, DC_3A_n78A, DC_3C_n78A

These are EN-DC combinations for which the DL LTE band is victim of IMD2 products leading to large MSD. Therefore, these cases fulfil the “Single UL allowed” criteria.

- For DC_1A_n77A:

According to Table 7.3B.2.3.5.1-1, single UL operation is not allowed for this combination (cf. extracted text in Table 1). This requirement is inconsistent with both conventions of Section 5.2B.1 [1] and Table 5.2B.4.1-1 since   when B1 and n77 UL are operated at 1950 and 4090 MHz respectively (cf. Table 1), IMD 2 products fall at B1 duplex distance (2140 MHz) leading to large LTE MSD.

- For DC_3A_n77A, DC_3A_n78A, DC_3C_n78A:

According to Table 7.3B.2.3.5.1-1, single UL operation is not allowed for these combinations (cf. extracted text in Table 1). This requirement is inconsistent with both conventions of Section 5.2B.1 [1] and Table 5.2B.4.1-1 since when B3 and n77 / n78 UL are operated at 1740 and 3575 MHz respectively (cf. Table 1), IMD 2 products fall at B3 duplex distance (1935MHz) leading to large LTE MSD.

Table 7.3B.2.3.5.1-1 should be corrected accordingly.

Note that for these EN-DC combinations, every LTE Band 1 and Band 3 DL channel can be impacted by a specific n77/n78 UL channel. Single UL operation is only allowed for these specific cases of collision.

DC_2A_n66A, DC_5A_n66A, DC_2A_n78A

These are respectively H-H IMD3 and IMD2 cases leading to large LTE MSD as shown in Table 1. Therefore, these cases fulfill the “Single UL allowed” criteria according to Section 5.2B.1 [1]. However, Table 1 below highlights in yellow that single UL operation is not allowed in both Table 5.2B.4.1-1 and Table 7.3B.2.3.5.1-1.

- For DC_2A_n66A, when B2 and n66 UL are operated according to Table 7.3B.2.3.5.1-1 UL configurations of 1855 and 1775 MHz respectively, IMD 3 products falling at B2 duplex distance (1935 MHz, cf. Table 1) produce LTE MSD.

- For DC_5A_n66A, when B5 and n66 UL are operated according to Table 7.3B.2.3.5.1-1 UL configurations of 838 and 1721 MHz respectively, IMD 2 products falling at B5 duplex distance (883 MHz, cf. Table 1) produce large LTE MSD.

- For DC_2A_n78A, when B2 and n78 UL are operated at Table 7.3B.2.3.5.1-1 UL configurations of 1855 and 3795 MHz respectively, IMD 2 products falling at 1935 MHz (cf. Table 1) exhibit a large overlap of B2 DL channel (1940MHz) and therefore produce a large LTE MSD.

Both Tables 5.2B.4.1-1 and 7.3B.2.3.5.1-1 should be corrected to indicate that single UL operation is allowed.

Note that only a limited set of LTE B2 DL channels are impacted by IMD products. These are located at the bottom of B2 DL band. For example, in the case of 5MHz CBW for both bands in DC_2A_n66A and DC_2A_n78A, B2 DL carrier frequencies of 1932.5 MHz and 1937.5MHz are impacted only when n66 and n78 UL is operated at 1772.5/1777.5 and 3785/3795 MHz respectively.

For DC_5A_n66A, a limited number of LTE B5 DL channels are impacted by specific n66 uplink channels. For example, in the case of 5MHz CBW in both B5 and n66, B5 DL carrier of 826.5 MHz is never impacted by IMD 2 products. Single UL operation is only allowed for the specific cases of collision.

DC_20A_n8A

This is a L-L IMD 3 case leading to large LTE MSD as shown in Table 1, and therefore meets the single UL allowed criteria. This is correctly captured in Table 7.3B.2.3.5.1-1 but not in Table 5.2B.4.1-1, this latter one needs a correction.

Note that a limited number of LTE B20 DL channels are impacted by a specific n8 UL channels. For example, in the case of 5MHz CBW in both B20 and n8, B20 DL carrier of 834.5 MHz is never impacted by IMD 2 products. Single UL operation is only allowed for the specific cases of collision.

Table 1: List of two-band EN-DC combinations with inconsistent “single UL” definition (highlighted in yellow) in [1]: Table 5.2B.4.1-1 vs Table 7.3B.2.3.5.1-1.

| Two band EN-DC combinations |  | Single UL allowed according to [1] | Extracted text from Table 5.2B.4.1-1 [1] |  |  |  | Extracted text from Table 7.3B.2.3.5.1-1 [1] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | EN-DC band range |  | E-UTRA band | NR band | Single UL Allowed |  | IMD order | LTE MSD [dB] | Single UL allowed |
| DC_1A_n77A | H-H | Yes | 1 | n77 | DC_1_n77 | IMD 2 Falls at B1 DL (2140 MHz) |  | 29.8 to 32.54 UL B1 = 1950 MHz UL n77 = 4090 MHz | No |
| DC_2A_n66A | H-H | Yes | 2 | n66 | No | IMD 3 Falls at B2 DL (1935 MHz) |  | 20 UL B2 = 1855 MHz UL n66 = 1775 MHz | No |
| DC_5A_n66A | L-H | Yes | 5 | n66 | No | IMD 2 Falls at B5 DL (883 MHz) |  | 30 UL B5 = 838 MHz UL n66 = 1721 MHz | No |
| DC_2A_n78A | H-H | Yes | 2 | n78 | No | IMD 2 Partial overlap Falls at 1940 MHz |  | 26 to 28.74 UL B2 = 1855 MHz N78 = 3795 MHz | No |
| DC_3A_n77A DC_3A_n78A DC_3C_n78A | H-H | Yes | 3 | n77 | DC_3_n77 DC_3_n78 | IMD 2 Falls at B3 DL (1835 MHz) |  | 26 to 28.74 UL B3 = 1740MHz UL n77 = 3575 MHz | No |
| DC_20A_n8A | L-L | Yes | 20 | n8 | No | IMD 3 |  | 21 | Yes |

Observation 1:

According to RAN 4 agreements:

- DC_1A_n77A, DC_3A_n77A, DC_3A_n78A, DC_3C_n78A combinations fulfill single UL allowed criteria according to section 5.2B.1 [1]. Information captured in Table 7.3B.2.3.5.1-1 is inconsistent with Table 5.2B.4.1-1: large LTE MSD due to IMD 2 is correctly captured in Table 5.2B.4.1-1 but not in Table 7.3B.2.3.5.1-1.

- DC_2A_n66A, DC_5A_n66A, DC_2A_n78A combinations fulfill single UL allowed criteria according to section 5.2B.1 [1]. Neither Table 5.2B.4.1-1 nor Table 7.3B.2.3.5.1-1 declare these EN-DC combinations as “single UL allowed” [1]. Large MSD and reported IMD product order in Table 7.3B.2.3.5.1-1 are inconsistent with single UL not being allowed for these combinations.

- DC_20A_n8A combination fulfills single UL allowed criteria according to section 5.2B.1 [1]. Table 5.2B.4.1-1 is inconsistent with Table 7.3B.2.3.5.1-1 since single UL is not indicated as allowed in [1].

To account for the above observations, we make the following proposals.

Proposal 1: Approve corrections in yellow to Table 5.2B.4.1-1 as listed in Table 2, i.e. replace “No” in column “Single UL allowed” for DC_2_n66, DC_2_n78, DC_5_n66, and DC_20_n8 with “DC_2_n66”, “DC_2_n78”, “DC_5_n66” and “DC_20_n8” respectively.

Table 2 Proposed changes to Table 5.2B.4.1-1 “Band combinations for EN-DC (two bands)”

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1_n28 | 1 | n28 | No |
| DC_1_n40 | 1 | n40 | No |
| DC_1_n51 | 1 | n51 | No |
| DC_1_n773 | 1 | n77 | DC_1_n77 |
| DC_1_n783 | 1 | n78 | No |
| DC_1_n793 | 1 | n79 | No |
| DC_2_n5 | 2 | n5 | No |
| DC_2_n66 | 2 | n66 | No DC_2_n66 |
| DC_2_n71 | 2 | n71 | No |
| DC_2_n78 | 2 | n78 | No DC_2_n78 |
| DC_3_n7 | 3 | n7 | No |
| DC_3_n28 | 3 | n28 | No |
| DC_3_n40 | 3 | n40 | No |
| DC_3_n51 | 3 | n51 | No |
| DC_3_n773 | 3 | n77 | DC_3_n77 |
| DC_3_n783 | 3 | n78 | DC_3_n78 |
| DC_3_n793 | 3 | n79 | No |
| DC_5_n40 | 5 | n40 | No |
| DC_5_n66 | 5 | n66 | No DC_5_n66 |
| DC_5_n783 | 5 | n78 | No |
| DC_7_n28 | 7 | n28 | No |
| DC_7_n51 | 7 | n51 | No |
| DC_7_n783 | 7 | n78 | No |
| DC_8_n40 | 8 | n40 | No |
| DC_8_n773 | 8 | n77 | No |
| DC_8_n783 | 8 | n78 | No |
| DC_8_n793 | 8 | n79 | No |
| DC_11_n773 | 11 | n77 | No |
| DC_11_n783 | 11 | n78 | No |
| DC_11_n793 | 11 | n79 | No |
| DC_12_n5 | 12 | n5 | No |
| DC_12_n66 | 12 | n66 | No |
| DC_18_n773 | 18 | n77 | No |
| DC_18_n783 | 18 | n78 | No |
| DC_18_n793 | 18 | n79 | No |
| DC_19_n773 | 19 | n77 | No |
| DC_19_n783 | 19 | n78 | No |
| DC_19_n793 | 19 | n79 | No |
| DC_20_n8 | 20 | n8 | No DC_20_n8 |
| DC_20_n284 | 20 | n28 | No |
| DC_20_n51 | 20 | n51 | No |
| DC_20_n77 | 20 | n77 | No |
| DC_20_n783 | 20 | n78 | No |
| DC_21_n773 | 21 | n77 | No |
| DC_21_n783 | 21 | n78 | No |
| DC_21_n793 | 21 | n79 | No |
| DC_25_n41 | 25 | n41 | No |
| DC_26_n413 | 26 | n41 | No |
| DC_26_n773 | 26 | n77 | No |
| DC_26_n783 | 26 | n78 | No |
| DC_26_n793 | 26 | n79 | No |
| DC_28_n51 | 28 | n51 | No |
| DC_28_n773 | 28 | n77 | No |
| DC_28_n783 | 28 | n78 | No |
| DC_28_n793 | 28 | n79 | No |
| DC_30_n5 | 30 | n5 | No |
| DC_30_n66 | 30 | n66 | No |
| DC_38_n78 | 38 | n78 | No |
| DC_39_n781,3 | 39 | n78 | No |
| DC_39_n793 | 39 | n79 | No |
| DC_40_n77 | 40 | n77 | No |
| DC_41_n77 | 41 | n77 | No |
| DC_41_n78 | 41 | n78 | No |
| DC_41_n792 | 41 | n79 | No |
| DC_42_n51 | 42 | n51 | No |
| DC_42_n77 | 42 | n77 | No |
| DC_42_n78 | 42 | n78 | No |
| DC_42_n79 | 42 | n79 | No |
| DC_66_n71 | 66 | n71 | No |
| DC_66_n5 | 66 | n5 | No |
| DC_66_n78 | 66 | n78 | No |
| NOTE 1: The frequency range above 3600MHz for Band n78 is not used in this combination. NOTE 2: The frequency range below 2545MHz for Band 41 is not used in this combination. NOTE 3: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. NOTE 4: The frequency range in band 28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |

Proposal 2: Approve corrections to Table 7.3B.2.3.5.1-1 as listed in Table 3, i.e. add “Yes” in column “Single UL allowed” for DC_1A_n77A (B1 UL = 1950 MHz, UL n77 = 4090 MHz), DC_2A_n66A (B2 UL = 1855 MHz, n66 UL = 1775 MHz), DC_2A_n78A (B2 UL = 1855 MHz, n78 UL = 3795 MHz),  DC_3A_n77A, DC_3A_n78A, DC_3C_n78A (B3 UL= 1740 MHz, n77/n78 UL = 3575 MHz) and DC_5A_n66A (B5 UL = 838 MHz, n66 UL = 1721 MHz).

Note that for the sake of simplification and to make [1] consistent with recent conventions adopted in LTE Rel’15, it might be worth considering capturing single UL allowed operation only in Table 5.2B.4.1-1. This proposal is discussed in [3].

Table 3:Proposed changes to Table 7.3B.2.3.5.1-1 “Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)”.

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A_n77A | 1 | 1950 | 5 | 25 | 2140 | 29.8 | FDD | IMD23 | Yes |
|  |  |  |  |  |  | 32.54 |  |  |  |
|  | n77 | 4090 | 10 | 25 | 4090 | N/A | TDD | N/A |  |
| DC_1A_n77A | 1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD43 |  |
|  |  |  |  |  |  | 10.74 |  |  |  |
|  | n77 | 3710 | 10 | 25 | 3710 | N/A | TDD | N/A |  |
| DC_1A_n78A, DC_1A_SUL_n78A-n84A | 1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD43 |  |
|  |  |  |  |  |  | 10.74 |  |  |  |
|  | n78 | 3710 | 10 | 25 | 3710 | N/A | TDD |  |  |
| DC_2A_n66A | 2 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 | Yes |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | TDD | N/A |  |
| DC_2A_n66A | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |  |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | TDD | IMD5 |  |
| DC_2A_n78A | 2 | 1855 | 5 | 25 | 1940 | 26 | FDD | IMD23 | Yes |
|  |  |  |  |  |  | 28.74 |  |  |  |
|  | n78 | 3795 | 10 | 25 | 3795 | N/A | TDD | N/A |  |
| DC_2A_n78A | 2 | 1885 | 5 | 25 | 1955 | 8.0 | FDD | IMD43 |  |
|  |  |  |  |  |  | 10.74 |  |  |  |
|  | n78 | 3700 | 10 | 25 | 3700 | N/A | TDD | N/A |  |
| DC_3A_n7A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |  |
|  | n7 | 2535 | 10 | 52 | 2655 | 10.25 | FDD | IMD4 |  |
| DC_3A_n77A DC_3A_n78A | 3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD23 | Yes |
|  |  |  |  |  |  | 28.74 |  |  |  |
|  | n77, n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |  |
| DC_3A_n77A DC_3A_n78A | 3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD43 |  |
|  |  |  |  |  |  | 10.74 |  |  |  |
|  | n77, n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |  |
| DC_3A_n78A | 3 | 1712.5 | 5 | 25 | 1807.5 | TBD5 | FDD | IMD2 | Yes |
|  | n78 | 3515 | 10 | 50 | 3515 | N/A | TDD | N/A |  |
|  | 3 | 1762.5 | 5 | 25 | 1857.5 | N/A | FDD | N/A | No |
|  | n78 | 3465 | 10 | 50 | 3465 | N/A | TDD | N/A |  |
| DC_3A-SUL_n78A-n80A, DC_66A-SUL_n78A-n86A | 3, 66 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD23 | Yes |
|  |  |  |  |  |  | 28.74 |  |  | Yes |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A | Yes |
| DC_3A_SUL_n78A-n80A, DC_66A-SUL_n78A-n86A | 3, 66 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD43 | No |
|  |  |  |  |  |  | 10.74 |  |  | No |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A | No |
| DC_3C_n78A | 3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 | Yes |
|  |  |  |  |  |  | 28.75 |  |  |  |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |  |
|  | n78 | 3710 | 10 | 25 | 3710 | N/A | TDD | N/A |  |
| DC_3C_n78A | 3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |  |
|  |  |  |  |  |  | 10.75 |  |  |  |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |  |
| DC_5A_n66A | 5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD23 | Yes |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A |  | N/A |  |
| DC_5A_n78A | 5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD4 |  |
|  | n78 | 3421 | 10 | 52 | 3421 | N/A | TDD | N/A |  |
| DC_8A_n77A DC_8A_n78A DC_8A-SUL_n78A-n81A | 8 | 897.5 | 5 | 25 | 942.5 | 8.3 | FDD | IMD4 |  |
|  | n77, n78 | 3635 | 10 | 52 | 3635 | N/A | TDD | H4 |  |
| DC_8A_n79A DC_8A-SUL_n79A-n81A | 8 | 897.5 | 5 | 25 | 942.5 | 4.8 | FDD | IMD5 |  |
|  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | TDD | N/A |  |
| DC_20A_n8A | 20 | 849.5 | 5 | 25 | 808.5 | 21 | FDD | IMD3 | Yes |
|  | n8 | 892.5 | 5 | 25 | 937.5 | 21 | FDD | IMD3 |  |
| DC_20A_n77A | 20 | 850 | 5 | 25 | 810 | 11 | FDD | IMD4 |  |
|  | n77 | 3360 | 10 | 50 | 3360 | N/A | TDD | N/A |  |
|  | 20 | 840 | 5 | 25 | 800 | 6.5 | FDD | IMD5 |  |
|  | n77 | 4160 | 10 | 50 | 4150 | N/A | TDD | N/A |  |
| DC_20A_n78A, DC_20A-SUL_n78A-n82A | 20 | 850 | 5 | 25 | 810 | 21.7 | FDD | IMD44 |  |
|  | n78 | 3360 | 10 | 50 | 3360 | N/A | TDD | N/A |  |
| DC_21A_n79A | 21 | 1457.5 | 5 | 25 | 1505.5 | 18.4 | FDD | IMD3 |  |
|  | n79 | 4420.5 | 40 | 216 | 4420.5 | N/A | TDD | N/A |  |
| DC_26A_n41A | 26 | 839 | 5 | 25 | 884 | 15.6 | FDD | IMD3 |  |
|  | n41 | 2562 | 10 | 52 | 2562 | N/A | TDD | N/A |  |
| DC_28A_n51A | 28 | 725.5 | 20 | 25 | 765.5 | 5 | FDD | IMD 4, 5 | Yes |
|  | n51 | 1429.5 | 5 | 25 | 1429.5 | 5 | TDD | IMD 4, 5 |  |
| DC_26A_n77A DC_26A_n78A | 26 | 836.5 | 5 | 25 | 881.5 | 11.1 | FDD | IMD4 |  |
|  | n77, n78 | 3390 | 10 | 50 | 3390 | N/A | TDD | N/A |  |
| CA_28A_n77A, CA_28A_n78A, DC_28A-SUL_n78A-n83A | 28 | 705.5 | 5 | 25 | 760.5 | 5.5 | FDD | IMD5 |  |
|  | n77, n78 | 3582.5 | 10 | 25 | 3582.5 | N/A | TDD | N/A |  |
| DC_66A_n5A | n5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD23 |  |
|  | 66 | 1721 | 5 | 25 | 2121 | N/A |  | N/A |  |
| DC_66A_n71A | 66 | 1750 | 5 | 25 | 2150 | 5 | FDD | IMD4 |  |
|  | n71 | 675 | 5 | 25 | 629 | N/A |  | N/A |  |
| DC_66A_n78A | 66 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD23 |  |
|  |  |  |  |  |  | 28.74 |  |  |  |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |  |
|  | 66 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD43 |  |
|  |  |  |  |  |  | 10.74 |  |  |  |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |  |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A. In case Single UL is allowed and the UE only indicates support of “Single UL” the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 5: For UEs only indicating support of Single UL, this requirement is verified with non-simultaneous uplink transmissions on the E-UTRA and NR CGs |  |  |  |  |  |  |  |  |  |

# Conclusion

This contribution provides an analysis of the recently agreed two-band EN-DC combinations, for which several inconsistencies with regards to meeting Single UL allowed criteria have been identified:

- DC_1A_n77A, DC_3A_n77A, DC_3A_n78A, DC_3C_n78A combinations fulfill single UL allowed criteria according to section 5.2B.1 [1]. Information captured in Table 7.3B.2.3.5.1-1 is inconsistent with Table 5.2B.4.1-1: large LTE MSD due to IMD 2 is correctly captured in Table 5.2B.4.1-1 but not in Table 7.3B.2.3.5.1-1.

- DC_2A_n66A, DC_5A_n66A, DC_2A_n78A combinations fulfill single UL allowed criteria according to section 5.2B.1 [1]. Neither Table 5.2B.4.1-1 nor Table 7.3B.2.3.5.1-1 declare these EN-DC combinations as “single UL allowed” [1]. Large MSD and reported IMD product order in Table 7.3B.2.3.5.1-1 are inconsistent with single UL not being allowed for these combinations.

- DC_20A_n8A combination fulfills single UL allowed criteria according to section 5.2B.1 [1]. Table 5.2B.4.1-1 is inconsistent with Table 7.3B.2.3.5.1-1 since single UL is not indicated as allowed in [1].

To account for the above observations, we make the following proposals.

Proposal 1: Approve corrections in yellow to Table 5.2B.4.1-1 as listed in Table 2, i.e. replace “No” in column “Single UL allowed” for DC_2_n66, DC_2_n78, DC_5_n66, and DC_20_n8 with “DC_2_n66”, “DC_2_n78”, “DC_5_n66” and “DC_20_n8” respectively.

Proposal 2: Approve corrections to Table 7.3B.2.3.5.1-1 as listed in Table 3, i.e add “Yes” in column “Single UL allowed” for DC_1A_n77A (B1 UL = 1950 MHz, UL n77 = 4090 MHz), DC_2A_n66A (B2 UL = 1855 MHz, n66 UL = 1775 MHz), DC_2A_n78A (B2 UL = 1855 MHz, n78 UL = 3795 MHz),  DC_3A_n77A, DC_3A_n78A, DC_3C_n78A (B3 UL= 1740 MHz, n77/n78 UL = 3575 MHz) and DC_5A_n66A (B5 UL = 838 MHz, n66 UL = 1721 MHz).

Note that for the sake of simplification and to make [1] consistent with recent conventions adopted in LTE Release 15, it might be worth considering capturing single UL allowed operation only in Table 5.2B.4.1-1. This proposal is discussed in [3].

# References

[1]	38101-3-f20, RAN#80, July 9th 2018.

[2] R4-1810108 “Corrections to single UL allowed criteria for EN-DC operation”, Skyworks Solutions Inc., RAN4#88.

[3] R4-1810902 Proposal to remove Single UL allowed column from Table 7.3B.2.3.5.1-1 in 38101-3, Skyworks Solutions Inc., RAN4#88.