3GPP TSG-RAN WG4 Meeting 88	R4-1810150

Göteborg, Sweden, 20 – 24 August 2018

Agenda Item:	6.7.7.2.1

Source:	Ericsson

Title:	CQI reporting tests for CRS-based PDSCH

Document for:	Discussion

# 1	Introduction

RAN4#AH1807 agreed with the way forward on sTTI demodulation and CQI reporting test [1]. In this contribution, we discuss the CQI to MCS table and show the simulation results.

- No PDSCH allocation on subframe 0 and subframe 5 for all CQI reporting test cases - No DMRS sharing in TM9 CQI reporting test - Companies are encouraged to provide: - CQI2MCS mapping table - Simulation results of CQI reporting - Proposal of test points - Simulation assumption: R4-1805493

This contribution provides the update simulation results based on the agreements.

# 2	Discussion

## 2.1	CQI to MCS table

This section discusses the CQI to MCS table used for CRS-based wideband CQI reporting test based on the test parameter in Table 1. Table 2 also shows the SPDCCH parameters agreed in AH1807.

Table 1	Parameters for wideband CQI reporting test.

|  | Test 1 |
| --- | --- |
| Bandwidth | 10MHz |
| TTI length | Slot-based / Subslot-based |
| Number of PDCCH symbols | 2 |
| Cyclic prefix | Normal |
| Subframe configuration | Non-MBSFN |
| Propagation channel | EPA5 |
| Antenna configuration | High (2x2) |
| Transmission modes | TM4 |
| CodeBookSubsetRestrction bitmap | 000001 |
| Reporting interval | 5ms |
| CQI delay | n+4 slots for slot-based n+4 or n+6 subslots for subslot-based |
| Reporting mode | PUSCH 1-1 |
| PDSCH transmission bandwidth | 50 RB |
| Number of HARQ transmissions | 1 |
| Duplex mode | FDD / TDD |

Table 2	SPDCCH parameters.

|  | SPDCCH parameters |
| --- | --- |
| Reference symbol | CRS |
| Transmission mode | Localized |
| Number of OFDM symbols | 1 |
| Number of PRBs | 16 |
| Allocated PRBs | {0, 1, …, 15} |
| Rate matching mode | Mode 1 |
| SPDCCH L1 Reuse Indication | Not configured |
| Total number of SCCEs | 4 (16 SREGs) |

Based on the parameters above, Table 3 shows the CQI to MCS table used for the wideband CQI. We calculate the available resource elements based on the assumption:

- SF#0 and SF#5 are DTX as same as the existing wideband CQI reporting test. In this case, we only need to consider the subframes without PSS/SSS/PBCH and SI transmission.

- Reduce the REs used for SPDCCH transmission (AL4 = 16 RBs with one OFDM symbol)

- CFI=2

- 2 CRS ports

Table 3	CQI-to-MCS table for CRS-based Slot/Subslot-PDSCH

|  |  |  | Slot-PDSCH |  | Subslot-PDSCH |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Slot 0 | Slot 1 | Subslot 1 | Subslot 2 | Subslot 3 | Subslot 4 | Subslot 5 |
|  |  | PRB | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
|  |  | Available REs | 2800 | 3672 | 1408 | 1008 | 872 | 1008 | 1472 |
| CQI index | Code rate | Modulation |  |  |  |  |  |  |  |
| 1 | 0.08 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0.12 | 2 | 0 | 1 | 1 | 0 | 0 | 0 | 1 |
| 3 | 0.19 | 2 | 2 | 3 | 3 | 2 | 1 | 2 | 3 |
| 4 | 0.3 | 2 | 4 | 5 | 6 | 4 | 3 | 4 | 6 |
| 5 | 0.44 | 2 | 6 | 7 | 8 | 6 | 5 | 6 | 9 |
| 6 | 0.59 | 2 | 7 | 9 | 9 | 8 | 7 | 8 | 9 |
| 7 | 0.37 | 4 | 10 | 13 | 14 | 11 | 10 | 11 | 14 |
| 8 | 0.48 | 4 | 12 | 15 | 16 | 13 | 12 | 13 | 16 |
| 9 | 0.6 | 4 | 14 | 16 | 16 | 15 | 14 | 15 | 16 |
| 10 | 0.46 | 6 | 17 | 20 | 22 | 18 | 17 | 18 | 22 |
| 11 | 0.55 | 6 | 19 | 23 | 24 | 20 | 18 | 20 | 25 |
| 12 | 0.65 | 6 | 21 | 25 | 27 | 22 | 20 | 22 | 27 |
| 13 | 0.75 | 6 | 23 | 27 | 28 | 24 | 22 | 24 | 28 |
| 14 | 0.85 | 6 | 25 | 28 | 28 | 26 | 24 | 26 | 28 |
| 15 | 0.93 | 6 | 26 | 28 | 28 | 27 | 25 | 27 | 28 |

# 3	Simulation results

The test metrics of the existing wideband CQI reporting requirements are specified as follows:

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to δ

Table 4 and Table 5 show our simulation results of wideband CQI reporting with slot-PDSCH and subslot-PDSCH, respectively, with regard to the metrics above. We list the test points corresponding to the median CQI index 4 (1/3 QPSK), 8 (1/2 16QAM), and 13 (3/4 64QAM). Figure 1 and Figure 2 show the median CQI and throughput ratio values for reference.

We propose to set two sets of SNR test points {3, 4} and {13, 14} because the SNR estimation in the lower SNR (e.g. < 0dB) is not accurate. For the test metrics, our simulation results show we can reuse the existing metrics, i.e., α=20%, γ=1.05, and δ=0.02.

Proposal 1: Set two sets of test points {3, 4} and {13, 14} for both slot-based and subslot-based wideband CQI reporting test.

Proposal 2: Reuse the existing criteria, i.e., α=20%, γ=1.05, δ=0.02.

Table 4	Simulation results of wideband CQI with slot-PDSCH.

| SNR | Median CQI | 1 - Prob(medianCQI-1, medianCQI, medianCQI+1) > α | BLER with follow CQI > δ | Tput ratio > γ |
| --- | --- | --- | --- | --- |
| -6 | 4 | 34.0 | 0.20 | 1.42 |
| -5 | 4 | 34.1 | 0.16 | 1.37 |
| -4 | 4 | 38.4 | 0.14 | 1.41 |
| 3 | 8 | 35.8 | 0.08 | 1.41 |
| 4 | 8 | 39.9 | 0.07 | 1.32 |
| 13 | 13 | 35.8 | 0.08 | 1.57 |
| 14 | 13 | 34.1 | 0.08 | 1.41 |

Table 5	Simulation results of wideband CQI with subslot-PDSCH.

| SNR | Median CQI | 1 - Prob(medianCQI-1, medianCQI, medianCQI+1) > α | BLER with follow CQI > δ | Tput ratio > γ |
| --- | --- | --- | --- | --- |
| -5 | 4 | 33.5 | 0.30 | 1.28 |
| -4 | 4 | 37.1 | 0.27 | 1.30 |
| 3 | 8 | 36.9 | 0.24 | 1.27 |
| 4 | 8 | 40.5 | 0.24 | 1.19 |
| 13 | 13 | 36.1 | 0.11 | 1.50 |
| 14 | 13 | 33.2 | 0.10 | 1.36 |
| 15 | 13 | 37.0 | 0.09 | 1.22 |

Figure 1	Median CQI.

Figure 2	Throughput ratio between the follow CQI and Fixed median CQI.

# 4	Conclusion

Proposal 1: Set two sets of test points {3, 4} and {13, 14} for both slot-based and subslot-based wideband CQI reporting test.

Proposal 2: Reuse the existing criteria, i.e., α=20%, γ=1.05, δ=0.02.

# 5	References

- R4-1809375, “Way forward on reference channels for sTTI demodulation and CSI requirements”, Qualcomm, Ericsson, Huawei.

# Appendix

Table 6	TBS table for slot/subslot-PDSCH of 50PRB.

|  |  |  |  | Slot-PDSCH |  | Subslot-PDSCH |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IMCS | Qm | ITBS | TBS with 50PRB | TBSscaled | TBSslot | TBSscaled | TBSsubslot |
| 0 | 2 | 0 | 1384 | 692 | 696 | 230.7 | 224 |
| 1 | 2 | 1 | 1800 | 900 | 904 | 300 | 296 |
| 2 | 2 | 2 | 2216 | 1108 | 1096 | 369.3 | 376 |
| 3 | 2 | 3 | 2856 | 1428 | 1416 | 476 | 472 |
| 4 | 2 | 4 | 3624 | 1812 | 1800 | 604 | 600 |
| 5 | 2 | 5 | 4392 | 2196 | 2216 | 732 | 744 |
| 6 | 2 | 6 | 5160 | 2580 | 2600 | 860 | 872 |
| 7 | 2 | 7 | 6200 | 3100 | 3112 | 1033.3 | 1032 |
| 8 | 2 | 8 | 6968 | 3484 | 3496 | 1161.3 | 1160 |
| 9 | 2 | 9 | 7992 | 3996 | 4008 | 1332 | 1320 |
| 10 | 4 | 9 | 7992 | 3996 | 4008 | 1332 | 1320 |
| 11 | 4 | 10 | 8760 | 4380 | 4392 | 1460 | 1480 |
| 12 | 4 | 11 | 9912 | 4956 | 4968 | 1652 | 1672 |
| 13 | 4 | 12 | 11448 | 5724 | 5736 | 1908 | 1928 |
| 14 | 4 | 13 | 12960 | 6480 | 6456 | 2160 | 2152 |
| 15 | 4 | 14 | 14112 | 7056 | 6968 | 2352 | 2344 |
| 16 | 4 | 15 | 15264 | 7632 | 7736 | 2544 | 2536 |
| 17 | 6 | 15 | 15264 | 7632 | 7736 | 2544 | 2536 |
| 18 | 6 | 16 | 16416 | 8208 | 8248 | 2736 | 2728 |
| 19 | 6 | 17 | 18336 | 9168 | 9144 | 3056 | 3112 |
| 20 | 6 | 18 | 19848 | 9924 | 9912 | 3308 | 3368 |
| 21 | 6 | 19 | 21384 | 10692 | 10680 | 3564 | 3624 |
| 22 | 6 | 20 | 22920 | 11460 | 11448 | 3820 | 3880 |
| 23 | 6 | 21 | 25456 | 12728 | 12576 | 4242.7 | 4264 |
| 24 | 6 | 22 | 27376 | 13688 | 13536 | 4562.7 | 4584 |
| 25 | 6 | 23 | 28336 | 14168 | 14112 | 4722.7 | 4776 |
| 26 | 6 | 24 | 30576 | 15288 | 15264 | 5096 | 5160 |
| 27 | 6 | 25 | 31704 | 15852 | 15840 | 5284 | 5352 |
| 28 | 6 | 26 | 36696 | 18348 | 18336 | 6116 | 6200 |