3GPP TSG-RAN WG4 Meeting #88	R4-1810316

# Gothenburg, Sweden, 20th – 24th Aug, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS38.113 | CR |  | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.113:Test configurations (subclause 4.5) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-08-1 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Test configurations are not stated clearly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test configurations according to NRTCx in TS 38.141. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EMC test are not performed in correct test configurations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--------------Start of change-------------

4.5	BS test configurations

The present subclause defines the BS test configurations that shall be used for demonstrating conformance. A single NR carrier shall be used for testing of single-carrier capable BS. For other NR base stations, the test configurations in table 4.5-1 and table 4.5-2 shall be used. The NR test configurations (NRTCx) are defined in TS 38.141-1 [3], subclause 4.7 for BS type 1-C and BS type 1-H and TS 38.141-2 [4], subclause 4.8 for BS type 1-O and BS type 2-O.

Table 4.5-1: Test configurations for BS type 1-C and BS type 1-H

| BS test case | BS with single-band connector capable of contiguous spectrum in single band only | BS with single-band connector capable of both contiguous and non-contiguous spectrum in single band | BS with single-band connector capable of multi-band operation | BS with Multi-band connector capable of multi-band operation |
| --- | --- | --- | --- | --- |
| Emission tests | NRTC1 | NRTC3 or NRTC1/3 (Note1) | NRCT1/3(Note 2), NRTC 5 | NRCT1/3(Note 2), NRTC 5 |
| Immunity tests | NRTC1 | NRTC3 or NRTC1/3 (Note1) | NRCT1/3(Note 2), NRTC 5 | NRCT1/3(Note 2), NRTC 5 |
| NOTE 1: When parameters in the manufacture's declaration according to subclause 4.6 in TS38.141-1 are identical for contiguous (C) and non-contiguous (NC), NRTC 3 is used, otherwise, NRTC1/3 shall be used. NOTE 2: NRTC1 or NRTC3 shall be applied in each supported operating band. |  |  |  |  |

Table 4.5-2: Test configurations for BS type 1-O and BS type 2-O

| BS test case | BS with single-band RIB capable of contiguous spectrum in single band only | BS with single-band RIB capable of both contiguous and non-contiguous spectrum in single band | BS with single-band RIB capable of multi-band operation | BS withMulti-band RIB capable of multi-band operation |
| --- | --- | --- | --- | --- |
| Emission tests | NRTC1 | NRTC3 or NRTC1/3 (Note1) | NRCT1/3(Note 2), NRTC 5 | NRCT1/3(Note 2), NRTC 5 |
| Immunity tests | NRTC1 | NRTC3 or NRTC1/3 (Note1) | NRCT1/3(Note 2), NRTC 5 | NRCT1/3(Note 2), NRTC 5 |
| NOTE 1: When parameters in the manufacture's declaration according to subclause 4.6 in TS38.141-2 are identical for contiguous (C) and non-contiguous (NC), NRTC 3 is used, otherwise, NRTC1/3 shall be used. NOTE 2: NRTC1 or NRTC3 shall be applied in each supported operating band. NOTE 3: The test configurations for multi-band operation are only applicable for BS type 1-O |  |  |  |  |

-------------End of change-------------