3GPP TSG-RAN WG4 Meeting #88						 R4- 1810349

Gothenburg, SE, 20th – 24th Aug, 2018

Title: 	Discussion on BWP switching

Source: 	OPPO

Agenda item:	7.11.8.4

Document for:	Discussion

Introduction

In last RAN4 AH1807 meeting, an WF [2] on BWP switching delay was agreed, which suggest RAN4 provide further analysis on the potential Type 3 delay (more than 2ms) and BWP switching involving only baseband parameter changes in this meeting.

In this document, we further discuss the remaining RRM requirements for BWP switching, according to agreements from previous meetings [1] [2].

Discussion

As background, companies has discussed and come to align with the same understanding of scenarios for BWP switching delay. Here is the description of 4 different scenarios and the corresponding BWP switching delays, which were agreed in RAN4#86 [3].

- Scenario 1: The reconfiguration involves changing the center frequency of the BWP without changing its BW. The reconfiguration may or may not involve changing the SCS. - Scenario 2: The reconfiguration involves changing the BW of the BWP without changing its center frequency. The reconfiguration may or may not involve changing the SCS. - Scenario 3: The reconfiguration involves changing both the BW and the center frequency of the BWP. The reconfiguration may or may not involve changing the SCS. - Scenario 4: The reconfiguration involves changing only the SCS, where the center frequency and BW of the BWP remain unchanged. Figure 1: Illustration of BWP reconfiguration scenarios - Table 1: BWP switching delays. Note: the numbers in the table are calculated from the end of the last symbol including the DCI indicating the BWP switch and until BB processing delay and RF transition time has been completed.

In the last meeting, an WF [2] on BWP switching delay was agreed as below. Remaining issues provide further analysis on the potential Type 3 delay (more than 2ms). RAN4 suggest companies investigate potential requirements for BWP switching delay involving only Baseband parameter changes which may be added as “Scenario 5”, and provide the list of baseband parameters. Furthermore, define this kind of BWP switching delay, if needed.

- Depending on the proposals from interested companies, options to introduce new BWP switching delay are to be further discussed

  - Option 1: Keep type 1 and type 2 unchanged.

  - Option 2: Revise Type 1 delay and keep Type 2 unchanged.

  - Option 3: Keep Type 1 and 2 unchanged and introduce Type 3 delay more than 2ms.

  - Option 4: Revise type 1 delay less than 2ms, introduce type 3 delay more than 2ms and keep Type 2 unchanged.

  - If option 3 or 4 is agreed, an LS will be send to RAN2 to introduce Type 3 BWP switching delay.

- Interested companies are encouraged to provide the analysis in RAN4#88 on the impact of long BWP switching delay (i.e. >2ms) from both network and UE perspectives.

- It is FFS if the delay for BWP switching involving only baseband parameter changes is the same as scenario 1/2/3 or scenario 4.

Interested companies are encouraged to provide the list of baseband parameters, which results in the corresponding BWP switching delay the same as scenario 1/2/3 or scenario 4.

Some general agreement has been approved. We find that for either DCI-based BWP switch or timer-based BWP switch, UE shall finish BWP switch within the time duration Y defined in Table 8.5.2-1 (scenario 1/2/3) and Table 8.5.2-2 (scenario 4) for different scenarios in TS 38.133.

Currently, some companies proposed that the delay for BWP switching involving only baseband parameter changes is the same as scenario 4. We think it is better to discuss case by case for BWP switching delay caused by different baseband parameter changing. Here, such list of parameters can be taken into consideration like SCS, CP length, DCI format change, COSET, beam failure parameters, and subset of them.

Observation 1: Delays for BWP switching involving only baseband parameter changes need to be discussed case by case. And SCS, CP length, DCI format change, COSET, uplink SCS, beam failure parameters, and subset of them, should be taken into consideration, if list of parameters will be defined.

Generally, the RF/baseband parameter preparation procedure will only cause delay but no interruption. Whether BWP switching involving only baseband parameter or SCS change will cause interruptions or not has been discussed for several meetings. And we still need to further discuss the issue case by case once the list of parameters is clear.

In addition, about interruption requirement, we agree with most companies we can reuse interruption requirements for SCell activation. And different interruption requirements are needed to further discussed in the meeting, e.g., inter-band and intra-band case.

Observation 2: Whether BWP switching involving only baseband parameter or SCS change will cause interruptions need to reach a conclusion in RAN4 #88 meeting.

Besides, in last RAN2 meeting, agreements on first active BWP were approved in [5].

Agreement 1: Support configuring dedicated BWP (via either option 1 or option 2) in Msg4 for all cases (initial access, resume and re-establishment). This is applicable to PCells only.

Agreement 2: In all cases, UE does BWP switching immediately upon acting on the RRC (re)configuration. This is applicable to SpCells only.

Agreement 3: Support switching SpCell BWP or reconfiguring SpCell BWP without RRC reconfiguration with sync. PSCell addition or change still requires reconfiguration with sync.

Since RAN2 has concluded with the agreements for BWP switch based on such RRC (re)-configuration in Msg4, RAN4 needs to provide corresponding requirements for RRC-based BWP switch. Some BWP switching delay may occur from initial or RRC configured BWP to first active BWP, and need to be defined in RAN4 RRM spec TS 38.133. And it is FFS whether UE can be scheduled without any scheduling restrictions immediately after BWP switch has been completed.

So in our view, BWP switch requirements in RAN4 RRM spec are needed for

-    DCI-based BWP switch

-    Timer-based BWP switch

-    RRC-based BWP switch

Proposal 1: RAN4 needs to define some new requirements for RRC-based BWP switch (where RRC (re)-configuration is included in the Msg4) from initial or configured BWP to first active BWP.

Conclusions

Observation 1: Delays for BWP switching involving only baseband parameter changes need to be discussed case by case. And SCS, CP length, DCI format change, COSET, uplink SCS, beam failure parameters, and subset of them, should be taken into consideration, if list of parameters will be defined.

Observation 2: Whether BWP switching involving only baseband parameter or SCS change will cause interruptions need to reach a conclusion in RAN4 #88 meeting.

Proposal 1: RAN4 needs to define some new requirements for RRC-based BWP switch (where RRC (re)-configuration is included in the Msg4) from initial or configured BWP to first active BWP.

References

- R4-1808001, “Way forward on BWP switching”, Mediatek

- R4-1809536, “Way forward on BWP switching”, Intel, MediaTek, Nokia, Nokia Shanghai Bell, Samsung

- R4-1803283, “LS on BWP switching delay”, Intel Corporation

- R4-1809068, “Further discussion on BWP switching”, Huawei, HiSilicon

- R4-1809179, “RRM part of BWP switching”, Nokia, Nokia Shanghai Bell

- R2-1810927, “Summary of offline discussion 007: First active BWP in Msg4 Nokia discussion”, Nokia