3GPP TSG-RAN WG4 Meeting #88															R4-1810419

Gothenburg, SWE, 20 - 24 August 2018

Source: 	Nokia, Nokia Shanghai Bell

Title: 					A-MPR results for n1

Agenda Item: 			7.2.2.1.1

Document for:	Discussion

# 1	Introduction

This contribution presents simulation results for n1 A-MPR based on allocation regions proposed by Qualcom in [1].

# 2	Discussion

We present backoff results for n1 according to the channel centre frequency and allocation regions proposed by Qualcom in [1], which are shown also in here as Table 1 assuming max(backoff, MPR). The respective simulation results for different waveforms, modulations and regions are shown in Table 2.

In Table 3, the difference between the results is shown as backoff in [1] minus our simulation results. Compared to [1], the results are generally well aligned, greater part of the backoff number being within 0.5 dB variation. However, for set of A-MPR results depicted as A2, our results seem to differ depending on the applied waveform (highlighted with yellow in Table 3). Our simulated backoff for SC-FDMA waveforms is at most 4.5. dB less compared to [1] and up to 4 dB more for CP-OFDM.

In addition, our results show smaller backoff demand for 256QAM modulation in A1 and A3-outer scenarios especially with OFDM, while, at the same time, less backoff in A3-inner (highlighted with green in Table 3) compared to [1]. In contrast, for A3-inner our simulation indicates more backoff for OFDM 256QAM compared to [1]. This, however, remains anyway below specified MPR of 6.5 dB and thus does not need implemented as A-MPR when A-MPR = max(backoff, MPR).

According to our understanding inner regions tend to be limited by EVM or inband emissions more often which can give rise to differing backoff demands for OFDM modulations, as for example shown in our results for A3-inner. Meanwhile, outer regions tend to be emission-limited, resulting in close A-MPR demands for OFDM modulations due to the small effect of applied modulation scheme in OFDM PAPR which is dominated by the multicarrier nature of the waveform. This can be seen, for example, in our results for A1, A2 and A3-outer regions.

In Table 3, we have highlighted with orange and green regions that remain empty of valid allocations according to our simulations.

| Table 1: A-MPR regions for NS_08 in [1]Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Rbstart | LCRB | A-MPR | Rbstart | LCRB | A-MPR | Rbstart | LCRB | A-MPR |
| 5MHz | 1922.5 ≤ FC < 1927.5 | <1.62MHz/12/SCS | >2.52MHz/12/SCS | A3 |  |  |  |  |  |  |
| 5MHz | 1927.5 ≤ FC < 1932.5 |  |  | A4 |  |  |  |  |  |  |
| 5MHz | 1932.5 ≤ FC ≤ 1977.5 |  |  | 0 |  |  |  |  |  |  |
| 10MHz | 1920 ≤ FC < 1935 | <1.62 MHz/12/SCS | >0 | A1 | >1.62MHz/12/SCS ≤3.60MHz/12/SCS | >5.4MHz/12/SCS | A8 | >7.74MHz/12/SCS | ≤1.08MHz/12/SCS | A2 |
| 10MHz | 1935 ≤ FC < 1945 |  | >4.5MHz/12/SCS | A5 |  |  |  |  |  |  |
| 10MHz | 1945 ≤ FC ≤ 1975 |  |  | 0 |  |  |  |  |  |  |
| 15MHz | 1927.5 ≤ FC < 1932.5 | <3.24MHz/12/SCS | >0 | A1 | >3.24MHz/12/SCS ≤5.40MHz/12/SCS | >8.1MHz/12/SCS | A8 | >10.9MHz/12/SCS | ≤1.08MHz/12/SCS | A2 |
| 15MHz | 1932.5 ≤ FC < 1940 | <1.62MHz/12/SCS | >0 | A1 |  |  |  | >12.6 MHz/12/SCS | ≤1.08MHz/12/SCS | A2 |
| 15MHz | 1940 ≤ FC < 1947.5 |  | >7.2MHz/12/SCS | A6 |  |  |  |  |  |  |
| 15MHz | 1947.5 ≤ FC ≤ 1972.5 |  |  | 0 |  |  |  |  |  |  |
| 20MHz | 1930 ≤ FC < 1950 | <4.86MHz/12/SCS | >0 | A1 | >4.86MHz/12/SCS ≤7.20MHz/12/SCS | >9.0MHz/12/SCS | A8 | >14.2MHz/12/SCS | ≤1.08MHz/12/SCS | A2 |
| 20MHz | 1950 ≤ FC < 1960 |  | >9.0MHz/12/SCS | A7 |  |  |  |  |  |  |
| 20MHz | 1960 ≤ FC ≤ 1970 |  |  | 0 |  |  |  |  |  |  |
| NOTE 1: The A-MPR values are listed in Table 2A and 2B. NOTE 3: For any undefined region, MPR applies |  |  |  |  |  |  |  |  |  |  |

Table 2: Simulated backoff for modulation and waveform type

| Wavefrom & Mod | A1 | A2 | A3 Out | A3 In | A4 Out | A4 In | A5 Out | A5 In | A6 Out | A6 In | A7 Out | A7 In | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SC-FDMA pi/2 | 9,50 | 0,50 | 3,50 | 0,00 | 0,00 | 0,00 | 0,00 | N/A | 0,50 | N/A | 0,50 | N/A | 5,50 |
| SC-FDMA QPSK | 9,50 | 3,00 | 5,50 | 0,00 | 0,50 | 0,00 | 2,00 | N/A | 2,00 | N/A | 1,50 | N/A | 6,50 |
| SC-FDMA 16QAM | 9,50 | 5,00 | 6,00 | 0,00 | 1,00 | 0,00 | 2,00 | N/A | 3,00 | N/A | 1,50 | N/A | 6,50 |
| SC-FDMA 64QAM | 10,00 | 5,00 | 6,00 | 0,00 | 1,50 | 1,50 | 2,00 | N/A | 2,50 | N/A | 2,00 | N/A | 6,50 |
| SC-FDMA 256QAM | 10,00 | 7,00 | 6,00 | 3,00 | 3,00 | 3,00 | 3,50 | N/A | 3,50 | N/A | 3,50 | N/A | 6,50 |
| OFDMA QPSK | 10,50 | 8,50 | 7,00 | 1,00 | 2,50 | 0,00 | 3,00 | 0,00 | 4,00 | N/A | 3,00 | 0,50 | 7,50 |
| OFDMA 16QAM | 10,50 | 9,00 | 7,00 | 1,50 | 2,50 | 1,50 | 3,00 | 1,50 | 3,50 | N/A | 3,00 | 1,50 | 7,00 |
| OFDMA 64QAM | 10,50 | 9,00 | 7,00 | 3,00 | 3,00 | 3,00 | 3,00 | 2,50 | 3,50 | N/A | 3,00 | 3,00 | 7,50 |
| OFDMA 256QAM | 10,50 | 9,00 | 7,00 | 5,00 | 5,50 | 6,00 | 5,50 | 5,00 | 5,50 | N/A | 5,50 | 5,00 | 7,50 |

Table 3: A-MPR in [1] minus simulated backoff

| Waveform & Mod | A1 | A2 | A3 Out | A3 In | A4 Out | A4 In | A5 Out | A5 In | A6 Out | A6 In | A7 Out | A7 In | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SC-FDMA pi/2 | 0,50 | 4,50 | 0,50 | 0,00 | 0,50 | 0,00 | 0,50 | N/A | 0,00 | N/A | 0,00 | N/A | 0,50 |
| SC-FDMA QPSK | 0,50 | 2,00 | -1,00 | 0,50 | 0,50 | 0,00 | -1,00 | N/A | -0,50 | N/A | 0,00 | N/A | -0,50 |
| SC-FDMA 16QAM | 0,50 | 0,00 | 0,00 | 1,00 | 1,00 | 0,50 | 0,00 | N/A | -1,00 | N/A | 0,50 | N/A | -0,50 |
| SC-FDMA 64QAM | 1,00 | 0,00 | 0,00 | 0,50 | 0,50 | 0,50 | 0,00 | N/A | -0,50 | N/A | 0,00 | N/A | -0,50 |
| SC-FDMA 256QAM | 3,00 | -2,00 | 1,00 | 1,50 | 0,00 | 0,00 | 1,00 | N/A | 1,00 | N/A | 1,00 | N/A | -0,50 |
| OFDMA QPSK | -0,50 | -3,50 | 0,50 | 1,00 | 1,00 | 0,50 | 0,50 | 0,50 | -0,50 | N/A | 0,50 | 0,00 | -1,50 |
| OFDMA 16QAM | -0,50 | -4,00 | 0,50 | 0,50 | 1,00 | 0,50 | 0,50 | 0,50 | 0,00 | N/A | 0,50 | 0,50 | -1,00 |
| OFDMA 64QAM | 0,50 | -4,00 | 1,00 | -1,00 | 0,50 | 0,50 | 0,50 | 1,00 | 0,00 | N/A | 0,50 | 0,50 | -1,50 |
| OFDMA 256QAM | 2,50 | -4,00 | 3,00 | -3,00 | 0,00 | -0,50 | 1,00 | 1,50 | 1,00 | N/A | 1,00 | 1,50 | -1,50 |

# 3	Conclusion

In this contribution we presented simulation results for NS_08 A-MPR on n1.

# 4	References

[1] R4-1810043 A-MPR Band n1 NS_08, RAN4#88

# Appendix