3GPP TSG-RAN WG4 Meeting #88	R4-1810451

Gothenburg, SE, 20 – 24 Aug., 2018

Source:	SoftBank Corp.,

Title:	2UL UE co-ex: a problem in co-ex table

Agenda item:	          9.2/8.5

Document for:	Discussion

1.  Introduction

In case of 2UL inter-band CA/DC, it has been requested to calculate IM frequency ranges based on the tables in relevant TRs. For IMs caused by difference of two Tx frequencies or their integer multiples, current tables have a risk of overlooking IM in lower frequencies. This paper is to point out the issue.

2.  Problem

As an example, the following table is a copy from TR37.863-01-01(EN-DC 2UL/2DL in REL-15) [1].

# Table 6.1.3-1: Band 3 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 3420 – 3570 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 5130 – 5355 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1515 – 2090 |  |  | 5010 – 5585 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 270 – 380 |  |  | 4815 – 5890 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 6720 – 7370 |  |  | 8310 – 9385 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 1610 – 1885 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330 – 2055 |  |  | 8115 – 9690 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ |  |  |  |
| IMD frequency limits (MHz) | 3030 – 4180 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 – 9155 |  |  | 11610 – 13185 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |  |  |  |
| IMD frequency limits (MHz) | 10020 – 11170 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11415 – 13490 |  |  | 3040 – 3840 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6330 – 7980 |  |  | 1245 – 2470 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 – 16985 |  |  | 10140 – 10940 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 – 14970 |  |  | 11730 – 12955 |  |  |

The table seems to suggest that IMn bound values be calculated with absolute. In case of pfx-qfy type of IM, there are two cases: two “raw” values (i.e. before taking absolute) have the same sign (positive or negative) or different signs (positive and negative). When both raw values have the same sign, there is no problem. But in case of different signs, this implies that the valid range of IM should be 0 to |upper bound|, instead of |lower bound| to |upper bound| as the table might suggest. Then IM effect from DC to |lower bound| would be overlooked in this case.

[Observation-1] Current IM tables have a risk to overlook IMs in lower frequency.

Justification is simple: we are calculating range of IM so any frequency between two raw values can be taken (but taking absolute afterward). We take absolute just because there is no negative frequency in the real world. If raw values of certain IM is [- Fa MHz, Fb MHz], the IM frequency in reality moves Fa MHz to DC to Fb MHz or vice versa with sweeping 2UL frequencies in different directions. The situation is depicted in Figure 1, assuming both Fa and Fb are positive numbers and Fa < Fb.

# Figure 1 : Overlooked IM

3.  Problematic cases would be rare

There might be a number of overlooked IMs (then possible desense) when 2UL is implemented. Although the author cannot spend time to inspect many pairs, the good news is that problems would become visible in quite limited cases, probably almost none in LTE-CA, due to some fortunate situations as below (and this could be a reason to be overlooked thus far):

- We do not use frequency lower than 460MHz (B73) for Rx. As long as CA is concerned, it seems that the lowest Rx bound would be 617MHz (B71) at present.

- As shown in Figure 1, for overlooked IM portion (0 – Fa MHz) to reach 460/617MHz, sufficient length of raw range of IMn is needed. The length of raw IM is p*(Total bandwidth of Band X) + q*(Total bandwidth of Band Y) where p+q is the order of the IM and the portion of having both positive and negative frequency becomes “overlooked IM”. It is rare for the portion to reach 460 or 617MHz because most of LTE bands do not have BW more than 100MHz.

- Bands with larger BW tend to be in higher freq. such as 2.5GHz or 3.5GHz. So 3 or 4 times of the band frequency tend to go higher and rare to reach zero or DC point, except 2UL CA among higher bands.

- EN-DC or 2UL inter-band NR-CA might be suffered (depending on BW and frequency of Bands) as BW of some NR bands are larger. This could happen in 2UL CA/DC including n77-n79.

As an example, the author found a problem in (3*B41-2*n77) IM5 of 41A-n77A as [-912, 1470]: Rx affected in 461-912MHz could be overlooked with the current table. This could cause a problem to miss desense when we think about something like EN-DC 28A-41A-n77A. (For 41A-n78A, however, affected range of the same IM5 scenario would be [-112, 1470] so all the supported Rx becomes visible.)

[Observation-2] With current spectrum situation, problematic cases would be limited.

4.  Actions to be taken

It seems that, under the current situation, the problem exists but would rarely affect actual 3GPP bands. So firstly, it is better to correct the co-ex table to check if different signs happen in a column to avoid the problem in the future. In addition, it is recommended for proponents to check the interested combos and if such an overlook is found, we should agree to fix it from the earliest possible release of the combination. As a minimum,

[Proposal-1] The co-ex tables in TRs should be revised ASAP.

Anyway, we need to discuss on how to tackle with that.

5.  Reference

[1]       TR37.863-01-01 (REL-15)

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"