# 3GPP TSG-RAN WG4 Meeting #88                                R4-1810485

# Gothenburg, Sweden, 20th - 24th Aug, 2018

Source: 	ZTE Corporation

Title: 	TP to TS 38.141-2: FR2 physical layer parameters setting (Section 6.1.1)

Agenda Item:	7.9.2.3

Document for:	Approval

1	Introduction

This contribution provide a TP to TS38.141-2[3] for FR2 physical layer parameters setting for test models in subclause 6.1.x based on [1-2].

2	Reference

[1] R4-1810465, Discussion on PDCCH/PDSCH/DMRS/PTRS configuration for NR test model,ZTE

[2] R4-1810467, Discussion on power boosting/de-boosting for NR test model,ZTE

[3] 38.141-2 v0.4.0

--------------Start of TP  -------------

6.1.2.1	FR2 NR Test Model 1.1 (FR2-NR-TM1.1)

This FR2-NR-TM 1.1 model shall be used for tests on:

-	BS output power

-	Unwanted emissions

-	Occupied bandwidth

-	ACLR

-	Operating band unwanted emissions

-	Transmitter spurious emissions

Table 6.1.2.1-1. general physical layer parameters for FR2-NR-TM1.1

| Parameter | Configuration |
| --- | --- |
| PDCCH DM-RS |  |
| precoderGranluarity | 3*floor(NRB/3) |
| Density of PDCCH DMRS | 1/4 |
| PDCCH |  |
| # of symbols allocated to CORESET | 2 |
| # of available REGs | 2*NRB |
| # of PDCCH | floor(NRB/3) |
| # of CCEs per PDCCH | 1 |
| # of REGs per CCE | 6 |
| # of REGs allocated to PDCCH | 6*floor(NRB/3) |
| # of <NIL> REGs added for padding | 2*NRB-6*floor(NRB/3) |
| The ratio of PDCCH REG EPRE to EA [dB] | 10*log10(NRB/(3*floor(NRB/3))) |
| The ratio of PDCCH DMRS EPRE to EA [dB] | 10*log10(NRB/(3*floor(NRB/3))) |
| The ratio of <NIL> REG EPRE to EA [dB] | -inf |
| PDSCH DMRS |  |
| # of DM-RS CDM groups without data | 1 |
| Configuration type | 1 |
| dmrs-TypeA-Position | 2 |
| dmrs-AdditionalPosition, | 0 |
| PDSCH |  |
| Mapping type | A |
| # of symbols allocated to PDSCH | 12 |
| # of QPSK PDSCH PRBs | NRB |
| The ratio of PDSCH EPRE to EA [dB] | 0 |
| The ratio of PDSCH DMRS to EA [dB] | 0 |
| PTRS |  |
| LPT-RS | 4 |
| KPT-RS | 2 |
| The ratio of PT-RS EPRE to PDSCH EPRE [dB] | 0 |
| Note 1: The transmission bandwidth configuration NRB is defined in section 5.3.2 of TS38.104. Note 2: EA denotes average EPRE when NR BS is transmitting with maximum power. |  |

6.1.2.2	FR2 NR Test Model 2 (FR2-NR-TM2)

This FR2-NR-TM2 model shall be used for tests on:

-	Total power dynamic range (lower OFDM symbol power limit at min power),

-	EVM of single 64QAM PRB allocation (at min power)

-	Frequency error (at min power)

Table 6.1.2.2-1. general physical layer parameters for FR2 NR-TM 2

| Parameter | Configuration |
| --- | --- |
| PDCCH DM-RS |  |
| precoderGranluarity | 3 |
| Density of PDCCH DMRS | 1/4 |
| PDCCH |  |
| # of symbols allocated to CORESET | 2 |
| # of available REGs | 2*NRB |
| # of PDCCH | 1 |
| # of CCEs per PDCCH | 1 |
| # of REGs per CCE | 6 |
| # of REGs allocated to PDCCH | 6 |
| # of <NIL> REGs added for padding | 2*NRB-6 |
| The ratio of PDCCH REG EPRE to EA [dB] | 0 |
| The ratio of PDCCH DMRS to EA [dB] | 0 |
| The ratio of <NIL> REG EPRE to EA [dB] | -inf |
| PDSCH DMRS |  |
| # of DM-RS CDM groups without data | 1 |
| Configuration type | 1 |
| dmrs-TypeA-Position | 2 |
| dmrs-AdditionalPosition, | 0 |
| PDSCH |  |
| Mapping type | A |
| # of symbols allocated to PDSCH | 12 |
| # of 64QAM PDSCH PRBs | 1 |
| The ratio of PDSCH EPRE to EA [dB] | 0 |
| The ratio of PDSCH DMRS to EA [dB] | 0 |
| # of other PDSCH PRBs | NRB-1 |
| The ratio of PDSCH EPRE to EA [dB] | -inf |
| The ratio of PDSCH DMRS to EA [dB] | -inf |
| PTRS |  |
| LPT-RS | 4 |
| KPT-RS | 2 |
| The ratio of PT-RS EPRE to PDSCH EPRE [dB] | 0 |
| Note 1: The transmission bandwidth configuration NRB is defined in section 5.3.2 of TS38.104. Note 2: EA denotes average EPRE when NR BS is transmitting with maximum power. |  |

6.1.2.3	FR2 NR Test Model 3.1 (FR2-NR-TM3.1)

This FR2-NR-TM3.1 model shall be used for tests on:

-	Output power dynamics

-	Total power dynamic range (upper OFDM symbol power limit at max power with all 64QAM PRBs allocated)

-	Transmitted signal quality

-	Frequency error

-	EVM for 64QAM modulation (at max power)

Table 6.1.2.3-1. general physical layer parameters for FR2 NR-TM 3.1

| Parameter | Configuration |
| --- | --- |
| PDCCH DM-RS |  |
| precoderGranluarity | 3*floor(NRB/3) |
| Density of PDCCH DMRS | 1/4 |
| PDCCH |  |
| # of symbols allocated to CORESET | 2 |
| # of available REGs | 2*NRB |
| # of PDCCH | floor(NRB/3) |
| # of CCEs per PDCCH | 1 |
| # of REGs per CCE | 6 |
| # of REGs allocated to PDCCH | 6*floor(NRB/3) |
| # of <NIL> REGs added for padding | 2*NRB-6*floor(NRB/3) |
| The ratio of PDCCH REG EPRE to EA [dB] | 10*log10(NRB/(3*floor(NRB/3))) |
| The ratio of PDCCH DMRS to EA [dB] | 10*log10(NRB/(3*floor(NRB/3))) |
| The ratio of <NIL> REG EPRE to EA [dB] | -inf |
| PDSCH DMRS |  |
| # of DM-RS CDM groups without data | 1 |
| Configuration type | 1 |
| dmrs-TypeA-Position | 2 |
| dmrs-AdditionalPosition, | 0 |
| PDSCH |  |
| Mapping type | A |
| # of symbols allocated to PDSCH | 12 |
| # of 64QAM PDSCH PRBs | NRB |
| The ratio of PDSCH EPRE to EA [dB] | 0 |
| The ratio of PDSCH DMRS to EA [dB] | 0 |
| PTRS |  |
| LPT-RS | 4 |
| KPT-RS | 2 |
| The ratio of PT-RS EPRE to PDSCH EPRE [dB] | 0 |
| Note 1: The transmission bandwidth configuration NRB is defined in section 5.3.2 of TS38.104. Note 2: EA denotes average EPRE when NR BS is transmitting with maximum power. |  |

6.1.2.3a	 FR2 NR Test Model 3.1a (FR2-NR-TM3.1a) optional

This FR2-NR-TM3.1a model shall be used for tests on:

-	Transmitted signal quality

-	Frequency error

-	EVM for 16QAM modulation

Physical channel parameters are defined in Table 6.1.2.3-1, with all 64QAM PDSCH PRBs replaced by 16QAM PDSCH PRBs.

--------------End of TP ------------