3GPP TSG-RAN WG4 Meeting #88	R4-1810536

Gothenburg, Sweden, 20th – 24th Aug, 2018

Agenda item:	6.12.2.1

Source: 	Qualcomm Incorporated

Title: 	Simulation result for 8Rx demodulation requirements

Document for:	Discussion

# Introduction

In this paper, we present the simulation result for 8Rx demodulation requirement based on the way-forward [1].

# Discussion

## Rank ≤ 4

Earlier RAN4 agreed to introduce new TM2 and TM3 demodulation requirement for 8Rx UE. While the applicability of this requirement still needs to be discussed, in Figure 1 and 2, we present the throughput result for TM2 and TM3 based on the agreed simulation assumption. It can be seen that 70% of the configured throughput can be achieved at the SNR of -0.5dB and 18dB, respectively.

Observation 1. Without considering the implementation margin, 70% of the maximum configured throughput can be achieved at the SNR of -0.5dB and 18dB for TM2 and TM3 8Rx test cases, respectively.

Figure 1. TM2 8Rx demodulation simulation result: 16QAM ½ rate EVA5 MedCorr B

Figure 2. TM3 Rank2 8Rx demodulation simulation result: 16QAM ½ rate EVA70 low correlation

## Rank > 4

Figure 3 and 4 shows the PDSCH throughput performance of rank-6 and rank-8 transmission for 16QAM ½ rate in the 8x8 EPA5 low correlation channel. It is shown that 70% of the maximum throughput can be achieved at 13dB and 17.8dB, respectively, and hence both options are feasible from the test point perspective. Considering that 8-Layer demap is more challenging that 6-Layer, our proposal is to define the PDSCH demodulation test based on rank 8.

Proposal 1. Introduce a new PDSCH demodulation test for rank > 4 based on 16QAM ½ rate rank-8 in 8x8 EPA5 channel with low antenna correlation.

Figure 3. 16QAM ½ rate Rank-6 in EPA5

Figure 4. 16QAM ½ rate Rank-8 in EPA5

## SDR

Figure 5 and 6 shows the SDR simulation result for rank-8 transmission with 64QAM and 256QAM MCS. Considering the TxEVM of 6% and 3% in each modulation order and considering the SNR achieving 85% of the configured throughput, our view is that SDR test for 64QAM test can be defined based on MCS 23. The target code rate for the proposed MCS is given by Table 1.

For 256QAM test, it is observed that none of the MCSs can achieve the 85% of the maximum throughput at the SNR with enough margin from the TxEVM limited regime. Therefore, we propose to consider a lower TxEVM or SDR test based on rank 6.

Proposal 2. SDR test for 8Rx UE is defined based on rank-8 transmission with the fixed MCS 23 for 64QAM scenario.

Proposal 3. For 256QAM SDR test for 8Rx UE, consider the additional options rank-6 and rank-8 with lower TxEVM.

Table 1. FRC for 64QAM SDR test with MCS 23

|  | # RBs | # of coded bits | TBS | Code rate |
| --- | --- | --- | --- | --- |
| SF 1, 4, 6, 9 | 50 | 144000 | 101840 | 0.7072 |
| SF 0 | 41 | 118080 | 81176 | 0.6875 |
| SF 5 | 38 | 109440 | 76208 | 0.6963 |
| SF 2, 7 | 50 | 134400 | 101840 | 0.7577 |
| SF 3, 8 | 50 | 139200 | 101840 | 0.7316 |

Figure 5. SDR simulation result for 64QAM rank-8 (TxEVM 6%)

Figure 6. SDR simulation result for 256QAM rank-8 (TxEVM 3%)

# Conclusions

In this paper, we presented the simulation result for the 8Rx demodulation requirements. The list of observation and proposals made in this paper is summarized as follows:

Observation 1. Without considering implementation margin, 70% of the maximum configured throughput can be achieved at the SNR of -0.5dB and 18dB for TM2 and TM3 8Rx test cases, respectively.

Proposal 1. Introduce a new PDSCH demodulation test for rank > 4 based on 16QAM ½ rate rank-8 in 8x8 EPA5 channel with low antenna correlation.

Proposal 2. SDR test for 8Rx UE is defined based on rank-8 transmission with the fixed MCS 24 for 64QAM scenario.

Proposal 3. For 256QAM SDR test for 8Rx UE, consider the additional options rank-6 and rank-8 with lower TxEVM.

# References

[1] R4-1807964, “Way forward on 8Rx demodulation tests”, Huawei, HiSilicon, TSG-RAN4 #87