3GPP TSG-RAN WG4 Meeting #88              	R4-1810592

Gothenburg, Sweden, August 20 – 24 2018

Title:	[draft] reply LS for UE capability of measurement gap for inter-RAT NR measurement not yet configured with EN-DC

Response to:	R2-1809014, LS for UE capability of measurement gap for inter-RAT NR measurement not yet configured with EN-DC

Release:	Release 15

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

#### Name: 	Lars Dalsgaard

E-mail Address:	lars.dalsgaard@nokia.com

Attachments:

1. Overall Description:

RAN4 would like to thank RAN2 for the LS R2-1809014.

RAN4 discussed the need for measurement gap for inter-RAT NR measurement before EN-DC configuration, and reached the following agreements:

- If UE is only capable of per UE gap,

  - when there is no other measurement that requires measurement gaps, inter-RAT NR measurement does not require gaps if all the serving carriers and target carriers form a valid MR-DC band combination; otherwise inter-RAT NR measurement requires measurement gaps.

  - when there is measurement that requires measurement gaps, a per UE gap pattern should be provided.

- If UE is capable of per FR gap,

  - when there is no other measurement that requires measurement gaps,

    - inter-RAT NR measurement in FR1 carriers does not require gaps if all the serving carriers and target carriers in FR1 form a valid MR-DC band combination; otherwise inter-RAT NR measurement on FR1 carriers requires measurement gaps.

    - Inter-RAT NR measurement in FR2 carriers does not require gaps regardless if the target carriers in FR2 form a valid MR-DC band combination with serving cells or not.

  - when there is measurement on non-NR or NR FR1 carriers that requires measurement gaps, a gap pattern should be provided, and it is up to network configuration if it is per UE gap or per FR gap for FR1.

2. Actions:

To TSG RAN WG2

RAN4 kindly asks RAN2 to take above information into account in their work.

3. Date of Next TSG-RAN WG4 Meeting:

3GPPRAN4#88bis                       8 – 12 October 2018     Chengdu, China

3GPPRAN4#89	                         12 – 16 November 2018, Spokane, US

References