3GPP TSG-RAN WG4 #88	R4- 1810715

Gothenburg, Sweden 20th – 24th August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR |  | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 37.843 –Section 10.9 - General |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MVG Industries, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-08-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding text for clause 10.9. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The addition of text for TRP measurement grids |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The text is needed in order to introduce Near Field Power density measurement for TRP estimation. If the text is not approved, in-band TRP will be only estimated based on EIRP measurements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 10.9	TRP measurement grids

### 10.9.1	General

The TRP or the radiated power is simply the total power radiated by an antenna/EUT. Due to energy conservation, TRP is independent of choice of test distance, apart from measurement errors.

TRPReference is the total power radiated by the antenna of a transmitting device, when non-idealities such as mismatch and losses in the antenna are taken into account. In theory, TRPReference is defined as the integral of the electromagnetic antenna’s radiation pattern over a spherical surface, that is

TRPReference=ϕ=02πθ=0πRer*Sr2sinθdθdϕ

where S is the Poynting Vector.

- Far Field region:

The definite integral for TRPReference becomes

TRPReference= 14π θ=0πϕ=02πEIRPθ,ϕ sinθdθdϕ

- Near Field regions

The near field region consists of reactive field and radiating near field. The fields produced by a z directed electric current element at the origin valid for any point in space, are:

H=ϕIdlsinθ4πrjk0+1re-jk0r

E=jηIdl2πk0rcosθjk0r2+1r3+θsinθ2-k02r+jk0r2+1r3e-jk0r

Where η=μ0ε0    (intrinsic impedance of vacuum)

Figure 10.9.1-1 does show the error between TRP computed by using the fields of a current element at any point and TRP computed in FF (r>λ) for the same infinitesimal current element by using the below formula:

TRPReference= θ=0πϕ=02πr2PDr,θ,ϕ sinθdθdϕ

where PDr,θ,ϕ=Etr,θ,ϕ2η

Figure 10.9.1-1. TRP error vs range length for an infinitesimal current element

Based on experimental findings on an AAS BS radiation pattern, it is possible to measure power density in near field considering only the magnitude of the tangential part of E field with an acceptable level of accuracy. Thus equation TRPReference can be expressed as

TRPReference= θ=0πϕ=02πr2PDr,θ,ϕ sinθdθdϕ

where PDr,θ,ϕ is proportional to the E2(r,θ,ϕ).

The range of radiating near field is defined as

0.62D3λ<R<2D2λ

where D is the maximum dimension of the DUT antenna aperture.

In practice, discrete samples of either EIRP orpower density are measured at different directions (θ,ϕ) over the entire sphere, which are used to numerically approximate the surface TRPReference integral.  The obtained value, TRPEstimate , is an approximation of TRPReference and the difference between them is defined as the TRP systematic error.

TRPsyserr = | TRPEstimate – TRPreference |   (in dB)

When measuring radiated power, at each measurement point for both far field and near field, two partial results for two orthogonal polarizations needs to be measured. These can be the θ (theta) and φ (phi) polarizations or any pair of orthogonal polarizations.

Power measurements can be performed at distance less than 2D^2/λ (the traditional far-field criteria). In following subclauses we describe several spherical sampling grids which can be applied to Power density, and EIRP measurements.