3GPP TSG-RAN WG4 Meeting #88	R4-1810850

## Gothenbuth, Sweden, 20 - 24 July, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | DraftCR | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.843: Tx IMD MU correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-08-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CLTA MU is erroneously included |  |  |  |  |  |  |  |  |
| Summary of change: |  | CLTA MU is removed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CLTA MU stays erroneously included |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

### 10.6.4  	OTA Transmitter intermodulation

#### 10.6.4.1 	General

#### 10.6.4.2 	In-door anechoic chamber

##### 10.6.4.2.1 	General

This method measures the transmitter intermodulation in an anechoic chamber. The separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna depends on the unwanted emission requirements, ACLR, OBUE, SEM, and spurious emission, to be measured under the condition of transmitter intermodulaiton. See sub-clause 10.4.2, 10.4.3, 10.4.4, and 10.5.x. The measurement system setup is as depicted in figure 10.6.x.2.1-1.

{editor’s note: the separation is [] because the out-of-band spurious emission test may not satisfy the condition that the sparation is no less than 2D2/λ.}

- General set-up (top view)
(Positioner is not described here)

{editor’s note: detail of the co-location [test] antenna placement will be described here}

(b) Detailed placement of co-location [test] antenna

Figure 10.6.4.2.1-1: In-door Anechoic Chamber measurement system setup for transmitter intermodulation

{editor’s note: Diagrams needs to be updated to show the co-location reference antenna and AAS BS are on the same position  and also the PA associated with the interferer signal. }

##### 10.6.4.2.2 	Calibration

Calibration of the emission measurements shall be done with the procedure as shown in 10.2.2.2.2.

For calibration of [co-location reference antenna] part is FFS.

{editors note: we need to include calibration of the input to the collocation test antenna}

##### 10.6.4.2.3 	Procedure

1)	Uninstall the reference antenna and install the AAS BS with the manufacturer declared coordinate system reference point in the same place as the phase centre of the reference antenna. The manufacturer declared coordinate system orientation of the AAS BS is set to be aligned with the testing system.

2)	Set the co-location test antenna] as specified in 3GPP TS37.145-2 sub-clause 4.15 [24], [at the declared worst case side.]

3) 	Set the signal generator to generate the interfering signal to achieve the required level at the input port of co-location test antenna].

4)	Perform required ACLR, OBUE (for E-UTRA), SEM (for UTRA), occupied bandwidth, spurious emission tests according to the corresponding testing procedures.

##### 10.6.4.2.4 	MU assessment

###### 10.6.4.2.4.1 	MU Budget

[The MU of the unwanted emission (e.g. ACLR, OBUE, SEM, spurious emission) is the same as the MU of corresponding unwanted emission requirement. The MU of TX-IM is the uncertainty of the interference signal level at AAS BS.]

Table 10.6.4.2.4.1-1 MU contributors for the CLTA interferer signal

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Uncertainty of interference signal input level to co-location [test] antenna |  |  |
| 1 | Uncertainty of the RF signal generator |  |
| 2 | Uncertainty of the RF power measurement equipment |  |
| 3 | Impedance mismatch in the transmitting chain |  |
| 4 | Random uncertainty |  |
| 5 | Gain variation of power amplifier |  |
| 6 | Influence of the transmitting antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability |  |
| [7] | [Uncertainty of the network analyser at co-location [test] antenna calibration stage] |  |

###### 10.6.4.2.4.2 	MU Value

Table 10.6.4.2.4.2-1 MU for the CLTA interferer signal

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uncertainty of interference signal input level to co-location [test] antenna |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the RF signal generator | 0.46 | 0.46 | Gaussian | 1.00 | 1 | 0.46 | 0.46 |
| 2 | Uncertainty of the RF power measurement equipment | 0.14 | 0.26 | Gaussian | 1.00 | 1 | 0.14 | 0.26 |
| 3 | Impedance mismatch in the transmitting chain | 0.14 | 0.23 | U-shaped | 1.41 | 1 | 0.10 | 0.16 |
| 4 | Random uncertainty | 0.1 | 0.1 | Rectangular | 1.73 | 1 | 0.06 | 0.06 |
| 5 | Gain variation of power amplifier | [0.02] | [0.02] | Rectangular | 1.73 | 1 | 0.01 | 0.01 |
| 6 | Influence of the transmitting antenna feed cable a) Flexing cables, adapters, attenuators, and connector repeatability | 0.05 | 0.05 | Rectangular | 1.73 | 1 | 0.03 | 0.03 |
| [7] | [Uncertainty of the network analyser at co-location [test] antenna calibration stage] | [0.13] | [0.20] | [Gaussian] | [1.00] | [1] | [0.13] | [0.20] |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  |  |
| Expanded uncertainty (1.96σ – confidence interval of 95%) [dB] ue = 1.96uc |  |  |  |  |  |  |  |  |

#### 10.6.4.3	CATR

------------------------------ End of modified sections ------------------------------