3GPP TSG-RAN WG4 Meeting #88                 	R4-1810883

Gothenburg, SE, 20th – 24th Aug, 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results for NR PUSCH

Agenda item:	7.13.2.2

Document for:	Discussion

Introduction

In this contribution we introduce simulation results using the parameters as agreed in [1].

Simulation assumptions

Simulation assumptions as agreed for NR PUSCH in [1] are shown below.

Table 1: Simulation assumptions for NR PUSCH

| Parameter | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1 | 1 |
| Number of Rx | 2 | 2 |
| Number of layers | 1 | 1 |
| DMRS type | Type 1 | Type 1 |
| Number of DMRS symbols | 1, 1+1 | 1 |
| symbols length | 14 | 11 |
| Time domain resource allocation type | Type A | Type B |
| MCS index | 2, 16, 20 | 2, 16, 20 |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation channel | TDL-A 30ns 10Hz | AWGN |
| SCS and BW | 15kHz: 10 30kHz: 40 | 60kHz: 100 120kHz: 100 |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| PTRS | Not configured | Configured with KPTRS =2, LPTRS =1 |

Simulation results

Simulation results for NR PUSCH using the parameters from Table 1 are shown below in Table 2 for MCS index 2, Table 3 for MCS index 16 and Table 4 MCS index 20.

Table 2: Initial simulation results with MCS index 2.

| MCS index | SCS [KHz] | CBW [MHz] | Number of DMRS symbols | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- |
| 2 | 15 | 10 | 1 | 70% | -2.1 |
|  |  |  | 1 + 1 | 70% | -1.9 |
|  | 30 | 40 | 1 | 70% | -3.0 |
|  |  |  | 1 + 1 | 70% | -2.8 |
|  | 60 | 100 | 1 | 70% | -4.1 |
|  | 120 | 100 | 1 | 70% | -3.9 |

Table 3: Initial simulation results with MCS index 16.

| MCS index | SCS [KHz] | CBW [MHz] | Number of DMRS symbols | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- |
| 16 | 15 | 10 | 1 | 70% | 9.2 |
|  |  |  | 1 + 1 | 70% | 9.5 |
|  | 30 | 40 | 1 | 70% | 8.6 |
|  |  |  | 1 + 1 | 70% | 9.1 |
|  | 60 | 100 | 1 | 70% | 7.2 |
|  | 120 | 100 | 1 | 70% | 7.4 |

Table 4: Initial simulation results with MCS index 20.

| MCS index | SCS [KHz] | CBW [MHz] | Number of DMRS symbols | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- |
| 20 | 15 | 10 | 1 | 70% | 12.5 |
|  |  |  | 1 + 1 | 70% | 12.8 |
|  | 30 | 40 | 1 | 70% | 12.1 |
|  |  |  | 1 + 1 | 70% | 12.4 |
|  | 60 | 100 | 1 | 70% | 10.5 |
|  | 120 | 100 | 1 | 70% | 10.7 |

Conclusion

In this contribution we have presented simulation results for PUSCH with MCS index 2, 16 and 20.

- References

- R4-1809336, WF on NR PUSCH demodulation requirements in Rel-15 Nokia, Nokia Shanghai Bell