3GPP TSG-RAN4 Meeting 88 	R4-1810906

# Göteborg, Sweden, 2nd – 6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR 37.843 Description of pattern multiplication method in sub-clause 10.9.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-08-10 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | With this draft CR, the text in sub-clause 10.9.5 is finalized. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - General editorial corrections - Description of pattern multiplication is added - Title is edited to account for different number of cuts |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, TR 37.843, sub-clause 10.9.5 will not be complete. This sub-clause holds the technical background to in-band TRP assessment using two cuts method. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Sub-clause 10.9.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--- start of changes---------

### 10.9.5	Orthogonal cuts grids

Compared to the TX spurious emissions the OBUE emissions are likely to experience the similar beamforming pattern as the main beam. Due to this reason, it is easily predictable where the maximum of the emissions is going to be, hence the possibility to enhance the measurement method to achieve better accuracy. Here we choose to apply the pattern multiplication method [ref].

In this method, at least two cuts (default) shall be used, an optional third cut can be added if needed. The alignment of the cuts must be along the symmetry planes of the antenna array.

The first mandatory cut is a horizontal cut passing through the peak direction of the main beam.

The second mandatory is a vertical cut passing through the peak direction of the main beam.

Using the data from these two mandatory cuts, a conditional pattern multiplication can be used.

The third optional cut is a vertical cut orthogonal to the first and the second cut.

Once the number and the orientation of the cuts are decided, the total EIRP is measured on the orthogonal cuts and the TRP is then calculated as follows: First the contributions from each cut is calculated as

TRPcut-n=1Pj=1PEIRP(j)

where P is the number of smpling point. Here, overlining denotes angular average value in the cut. The final contribution for all cuts is calculated as

TRPEstimate=1Nn=1NTRPcut-n

where N is the number of cuts. Note that when orthogonal cuts are measured, the intersection points are measured multiple times and the repeated values can be removed from the samples before averaging.

| Figure 10.9.5-1: Example of orthogonal cuts geometry when the main lobe points along the x-axis. Two mandatory cuts grid (left) and the optional added third cut (right). The first two cuts are generated by rotating the EUT around its z-axis and y-axis, respectively, and the optional third cut is generated by rotating the EUT around its x-axis. |  |
| --- | --- |

Two cuts cut data gives a conservative TRP estimate (an overestimation of the real TRP). Through patter multiplication a less conservative estimate is obtained, based on the calculation of the antenna array factor as a product of two terms, corresponding to the two cuts.

The following conditions for being able to apply pattern multiplication method are mandatory:

    - The vertical cut is taken in the xz -plane

    - The frequency of the emission is within the BS RF Bandwidth or close to its edges (e.g. emissions will experience similar pattern as the wanted signal)

    - The bandwidth of the emission is the same as the bandwidth of the in-band modulated signal

    - The emission changes significantly when the Tx power is turned on or off.

    - The antenna arrays of the EUT

      - Have rectangular grids of antenna element positions

      - Have symmetry planes that are vertical and horizontal.

      - Have parallel antenna planes

The antenna array is here assumed to be placed in the yz-plane. The pattern multiplication is performed in uv-coordinates and the data in the two cuts are denoted EIRP(1)ϕ at θ=θH and a vertical cut with data EIRP(2)θ at ϕ=0. The data is split in two parts corresponding to the forward and backward hemisphere. The uv-coordinates are the projections of the angular directions onto the antenna plane, here the yz-plane. Using the spherical coordinates as depicted in Fig. 10.9.2.2.-1 the u and v coordinates are defined as

u=sinθsinϕv=cosθ

Note that only the data on the coordinate axes are measured, and hence only the data EIRP1u for v=cosθH (the horizontal cut) and EIRP(2)v for u=0 (the vertical cut) are known. Moreover, only the points in the circular disc u2+v2≤1 , a.k.a. the visible region, contribute to the TRP.

The pattern multiplication is used to calculate power density values outside the two cardinal cuts as

EIRPu,v=EIRP1uEIRP2vEIRP0,vH

As illustrated below for the case vH=0

The pattern multiplication is applied separately for the forward (fwd) and backward (bwd) hemisphere. The TRP is then calculated as

TRP=14πfwdEIRPfwdu,vdudv1-u2-v2+bwdEIRPbwdu,vdudv1-u2-v2

Note, the numerical singularity at u2+v2=1 must be treated with care, see e.g. .

------end of changes ------