3GPP TSG-RAN WG4 meeting #88	R4-1810938
Gothenburg, Sweden, 20 Aug – 24 Aug, 2018

Source:	ZTE Corporation

Title:	Further clarification on PRB alignment for multiple numerologies

Agenda item:	7.5.2.3

Document for:	Approval

## 1. Background

In this contribution, one issue is identified on the channel raster to resource element mapping resulting from an enumerated parameter in the current specs, and further clarifications are proposed.

## 2. Discussion

Channel raster is mapped to the resource element located at the centre of transmission bandwidth configuration and it was identified that the mapping may not be applicable for all the supported numerologies [1] [2], therefore the mapping is specified that it applies for at least one numerology supported by BS or UE [3] [4] [5].

But when checking some signalling, i.e., SCS-SpecificCarrier, we can find that one of the parameters corresponding to the PRB alignment k0 is enumerated to only 3 values: 0/-6/6, and this may impose some restriction on the applicability of channel raster to resource element mapping.

Furthermore, some confusion on how to place UE RF filter was raised in Busan’s meeting [8]. Although it turns out no extra signalling is needed for this purpose according to the majority view [9], it actually indicates that the ambiguity of which numerology the mapping applies to may need to be clarified in the specs.

SCS-SpecificCarrier

The IE SCS-SpecificCarrier provides parameters determining the location and width of the actual carrier [6]. It is defined specifically for a numerology (subcarrier spacing (SCS)) and in relation (frequency offset) to Point A.

-- ASN1START

-- TAG-SCS-SPECIFIC-CARRIER-START

SCS-SpecificCarrier ::=			SEQUENCE {

-- Offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier

-- in number of PRBs (using the subcarrierSpacing defined for this carrier). The maximum value corresponds to 275*8-1.

-- Corresponds to L1 parameter 'offset-pointA-low-scs' (see 38.211, section FFS_Section)

offsetToCarrier				INTEGER (0..2199),

-- Subcarrier spacing of this carrier. It is used to convert the offsetToCarrier into an actual frequency.

-- Only the values 15 or 30 kHz  (<6GHz), 60 or 120 kHz (>6GHz) are applicable.

-- Corresponds to L1 parameter 'ref-scs' (see 38.211, section FFS_Section)

subcarrierSpacing					SubcarrierSpacing,

-- Corresponds to L1 parameter k0 (see 38.211, section 5.3.1). n-6 corresponds to value -6, n0 corresponds to value 0 and n6 corresponds

-- to value +6.

k0									ENUMERATED {n-6, n0, n6},

-- Width of this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier)

-- Corresponds to L1 parameter 'BW' (see 38.211, section FFS_Section)

carrierBandwidth					INTEGER (1..maxNrofPhysicalResourceBlocks),

...

}

-- TAG-SCS-SPECIFIC-CARRIER-STOP

-- ASN1STOP

Purpose of k0 in PHY

The time-continuous signal  on antenna port  and subcarrier spacing configuration  for OFDM symbol  in a subframe for any physical channel or signal except PRACH is defined by

where ,  is given by clause 4.2, and  is the subcarrier spacing configuration.

The value of  is obtained from the higher-layer parameter k0 and is such that the lowest numbered subcarrier in a common resource block for subcarrier spacing configuration  coincides with the lowest numbered subcarrier in a common resource block for any subcarrier spacing configuration less than .

This equation shows k0 indicates the offset between the centre of transmission configuration bandwidth and the transmission frequency, and only three values {0, -6, 6} are allowed for each numerology according to the current specs.

Observation 1: k0 indicates the offset between the centre of transmission configuration bandwidth and the transmission frequency, and only three values {0, -6, 6} are allowed for each numerology according to the current specs.

With this restriction in mind, we can check the cases where the channel raster to RE mapping applies to the supported numerologies with the smallest SCS and the largest SCS respectively.

Our contributions [1] [2] actually show that there are k0 values other than {0, -6, 6} if the channel raster to RE mapping applies to the numerology with the smallest SCS.

Observation 2: Channel raster to RE mapping is NOT always applicable to the numerology with the smallest SCS supported by BS or UE.

(a)

(b)

(c)

(d)

Fig. 1, Channel raster to RE mapping applies to the numerology with the largest supported SCS

Fig. 1 illustrates k0 values in different cases resulting from the channel raster to RE mapping applied to the numerology with the largest supported SCS, and it turns out all of these k0 values are within the allowed set in the current specs, which means that the channel raster to RE mapping is always applicable to the numerology with the largest SCS supported by BS or UE.

Observation 3: Channel raster to RE mapping is always applicable to the numerology with the largest SCS supported by BS or UE.

With the above observations, we propose to clarify in the specs that the channel raster to RE mapping always applies to the numerology with the largest SCS supported by BS or UE.

Proposal 1:  RAN4 specs specify channel raster to RE mapping always applies to the numerology with the largest SCS supported by BS or UE.

## 3. Conclusions

Based on the above discussions, we have the following observations and proposal on channel raster to RE mapping for multiple numerologies:

Observation 1: k0 indicates the offset between the centre of transmission configuration bandwidth and the transmission frequency, and only three values {0, -6, 6} are allowed for each numerology according to the current specs.

Observation 2: Channel raster to RE mapping is NOT always applicable to the numerology with the smallest SCS supported by BS or UE.

Observation 3: Channel raster to RE mapping is always applicable to the numerology with the largest SCS supported by BS or UE.

Proposal 1:  RAN4 specs specify channel raster to RE mapping always applies to the numerology with the largest SCS supported by BS or UE.

# References

[1] R4-1713262 Further consideration on PRB placement, ZTE

[2] R4-1713263 Channel raster to subcarrier mapping, ZTE

[3] TS 38.104 v15.2.0

[4] TS 38.101-1 v15.2.0

[5] TS 38.101-2 v15.2.0

[6] TS 38.331 v15.2.0

[7] TS 38.211 v15.2.0

[8] R4-1807608	On the indication of the location of channel, Huawei

[9] Chairman notes RAN4#87