3GPP TSG-RAN WG4 Meeting #88	R4-1811132

Gothenburg, Sweden, 20th – 24th August 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	On measurement uncertainty for OTA FR1 BS maximum output power

Agenda item:	7.9.4.2.1

Document for:	Discussion

# 1	Introduction

This document discusses measurement uncertainty contributions for OTA FR1 BS maximum output power test requirement in CATR chambers. For frequencies below 4.2 GHz, the uncertainty values are based on the OTA AAS [1]. In the document, the uncertainty value for frequency ranging from 4.2 GHz up to 6 GHz is proposed.

# 2	Discussion

## 2.1 	Uncertainty budget format

From [1], the CATR uncertainty budget format is as follows:

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT & pointing error | B2-1 |
| 2 | RF power measurement equipment | E |
| 3 | Standing wave between DUT and test range antenna | B2-3 |
| 4 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | B2-4 |
| 5 | QZ ripple DUT | B2-5 |
|  | Test system frequency flatness | TBD |
| 19 | Miscellaneous uncertainty | B2-14 |
| Stage 1: Calibration measurement |  |  |
| 6 | Uncertainty of network analyser | E |
| 7 | Mismatch of receiver chain | B2-7 |
| 8 | Insertion loss variation of receiver chain | B2-8 |
| 9 | RF leakage, (SGH connector terminated & test range antenna connector cable terminated) | B2-4 |
| 10 | Influence of the calibration antenna feed cable: a) Flexing cables, adapters, attenuators, connector repeatability | B2-9 |
| 11 | Uncertainty of the absolute gain of the calibration antenna | E |
| 12 | Misalignment positioning system | B2-11 |
| 13 | Misalignment of calibration antenna and test range antenna | B2-1 |
| 14 | Rotary Joints | B2-12 |
| 15 | Standing wave between reference calibration antenna and test range antenna | B2-3 |
| 16 | Quality of quiet zone | B2-5 |
| 17 | Switching uncertainty | B2-15 |

Table 1: CATR uncertainty budget format for OTA FR1 BS maximum output power

## 2.2	Uncertainty calculation

The uncertainty calculation for OTA FR1 BS maximum output power is given in Table 2.

| OTA FR1 BS maximum output power uncertainty budget |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz < f ≤ 4.2GHz | Uncertainty value 4.2GHz < f ≤ 6GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz f ≤ 4.2GHz | Standard uncertainty ui [dB] 4.2GHz < f ≦ 6GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | 0.3 | 0.3 | [0.3] | Rectangular | √3 | 1 | 0.174 | 0.174 | [0.174] |
| 2 | RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | [0.27] | Gaussian | 1 | 1 | 0.14 | 0.26 | [0.27] |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | [0.21] | U-shaped | √2 | 1 | 0.15 | 0.15 | [0.15] |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | [0.0012] | Normal | 1 | 1 | 0.0012 | 0.0012 | [0.0012] |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | [0.0928] | Normal | 1 | 1 | 0.0928 | 0.0928 | [0.0928] |
|  | Test system frequency flatness | 0.25 | 0.25 | [0.25] | Gaussian | 1 | 1 | 0.25 | 0.25 | [0.25] |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.20 | [0.2] | Normal | 1 | 1 | 0.13 | 0.20 | [0.2] |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | [0.325] | U-shaped | √2 | 1 | 0.09 | 0.23 | [0.23] |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | [0.18] | Rectangular | √3 | 1 | 0.10 | 0.10 | [0.10] |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | [0.0012] | Normal | 1 | 1 | 0.0012 | 0.0012 | [0.0012] |
| 10 | Influence of the calibration antenna feed cable | 0.022 | 0.022 | [0.022] | U-shaped | √2 | 1 | 0.015 | 0.015 | [0.015] |
| 11 | SGH Calibration uncertainty | 0.50 | 0.433 | [0.433] | Rectangular | √3 | 1 | 0.29 | 0.25 | [0.25] |
| 12 | Misalignment positioning system | 0 | 0 | [0] | Exp. normal | 2 | 1 | 0 | 0 | [0] |
| 13 | Misalignment SGH and pointing error | 0.5 | 0.5 | [0.5] | Exp. normal | 2 | 1 | 0.25 | 0.25 | [0.25] |
| 14 | Rotary joints | 0.048 | 0.048 | [0.048] | U-shaped | √2 | 1 | 0.034 | 0.034 | [0.034] |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | [0.09] | U-shaped | √2 | 1 | 0.06 | 0.06 | [0.06] |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | [0.009] | Normal | 1 | 1 | 0.009 | 0.009 | [0.009] |
| 17 | Switching uncertainty | 0.26 | 0.26 | [0.26] | Rectangular | √3 | 1 | 0.15 | 0.15 | [0.15] |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  | 0.59 | 0.67 | [0.67 |

Table 2: CATR uncertainty calculation for OTA FR1 BS maximum output power

The total uncertainty MUtotal is calculated as

MUtotal=MUper-point2+TRPsyserr2, where

MUper-point= uc which is given in Table 3 and TRPsyserr = 0.75 dB.

The total uncertainty values are given in Table 3.

Table 3: CATR total uncertainty calculation for OTA FR1 BS maximum output power

|  | Uncertainty value [dB] f ≤ 3 GHz | Uncertainty value [dB] 3 GHz < f ≤ 4.2 GHz | Uncertainty value [dB] 4.2 GHz < f ≤ 6 GHz |
| --- | --- | --- | --- |
| MUper-point, uc | 0.59 | 0.67 | [0.67] |
| TRPsyserr | 0.75 | 0.75 | 0.75 |
| Total uncertainty, MUtotal=uc2+TRPsyserr2 [dB] | 0.96 | 1.00 | [1.00] |
| 95% confidence interval, total uncertainty MUtotal=1.96MUper-point2+TRPsyserr2 [dB] | 1.88 | 1.96 | [1.96] |

# 3	Conclusions

The document has discussed measurement uncertainty contributions for OTA FR1 BS maximum output power test requirement in CATR chambers. The total measurement uncertainty values are as follows:

|  | Uncertainty value [dB] f ≤ 3 GHz | Uncertainty value [dB] 3 GHz < f ≤ 4.2 GHz | Uncertainty value [dB] 3 GHz < f ≤ 4.2 GHz |
| --- | --- | --- | --- |
| MUper-point, uc | 0.59 | 0.67 | [0.67] |
| TRPsyserr | 0.75 | 0.75 | 0.75 |
| Total uncertainty, MUtotal=uc2+TRPsyserr2 [dB] | 0.96 | 1.00 | [1.00] |
| 95% confidence interval, total uncertainty MUtotal=1.96MUper-point2+TRPsyserr2 [dB] | 1.88 | 1.96 | [1.96] |

# References

- R4-1811136, On measurement uncertainty for OTA AAS BS maximum output power, Nokia, Nokia Shanghai Bell

- TR 37.843 v15.0.0, RF requirement background for AAS BS radiated requirements (Release 15)