3GPPRAN4# 88	R4-1811228

Gothenburg, Sweden August 20th - 24th, 2018

Title: 	Further discussion on Handover in FR2

Source: 	Qualcomm Incorporated

Agenda item:	7.11.6.1 - Handover and random access (Intra-NR handover) [NR_newRAT-Core]

Document for:	Discussion

# Introduction

In contribution we provide our view on the remaining TBD’s in handover involving FR2 in NR.

# Discussion

## Interruption Time for FR2

The two open items for FR2 HO are Tsearch for inter-band case and T∆, time for fine time tracking and acquiring full timing information.

In RAN4#87, it was decided that Tsearch for inter-band case will be Tsearch = [N1*(2 or 4)* SMTC periodicity + 5] ms.  On a per-beam basis, the UE will need at most 1 sample for AGC and then an additional sample to acquire timing using SSB. Thus, the UE should not need more than 2 sample for search.

Proposal 1: Tsearch for inter-band case shall be [N1*2* SMTC periodicity + 5] ms

With regards T∆, the finer time tracking is done after the UE has already acquired timing on a beam. This should not take more than 1 sample and there is no need to do a beam sweep at this point. There have been some proposals made that the first step, search, would just do a threshold in terms of beams, and the best beam needs to be selected after doing refinement on all beams that are above the threshold. We believe that would be sub-optimal implementation and degrade the system performance. The UE should choose the best beam out of search and use that for handover. After handover, if the UE decides that the beam is sub-optimal, it can then select the optimal beam based on it beam-management algorithm.

Proposal 2: T∆ shall be 1 SMTC period

# Conclusions

In this contribution, we provide our inputs on interruption time during NR handover.

Proposal 1: Tsearch for inter-band case shall be [N1*2* SMTC periodicity + 5] ms

Proposal 2: T∆ shall be 1 SMTC period