3GPP TSG-RAN WG4 Meeting RAN4#88                                               R4-180xxxxx

Gotherburg ,Sweden, 20 -24 Aug, 2018

Agenda Item:   7.13.1.5

Source:        Samsung

Title:          Test case design for static CQI test cases

Document for:  Discussion

Introduction

In last RAN4 meeting, list of test cases has been captured in WF [1]:

- Static CQI test cases - Test 1: FR1 FDD 10MHz &15kHz , 2*2, 2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 2*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - Wideband fading CQI test cases - Test 1: FR1 FDD 10MHz &15kHz , 2*2, 2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 2*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - Wideband PMI test cases - Test 1: FR1 FDD 10MHz &15kHz , 8*2, 8*4 - Test 2: FR1 TDD 40MHz&30kHz, 8*2, 8*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - 4Tx PMI test cases (FFS for wideband or sub-band) - Test 1: FR1 FDD 10MHz &15kHz , 4*2, 4*4 - Test 2: FR1 TDD 40MHz&30kHz, 4*2, 4*4 - Sub-band CQI test cases (2nd priority) - Test 1: FR1 FDD 10MHz&15kHz, 2*2,2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2,2*4 - RI test - Test 1: FR1 FDD 10MHz&15kHz, 2*2,4*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 4*4 - Test 3: FR2 100MHz&120kHz, 2*2

In this contribution, initial simulation results for static CQI test cases provided.

Discussion

Simulation assumption

| Parameter | Unit | Static CQI test (FR1 FDD) | Static CQI test (FR1 TDD) | Static CQI test (FR2) |
| --- | --- | --- | --- | --- |
| System parameter |  |  |  |  |
| Bandwidth/SCS | MHz/kHz | 10MHz /15kHz | 40MHz/30kHz | 100MHz/120kHz |
| Transmission Scheme |  | 1 | 1 | 1 |
| Duplex Mode |  | FDD | TDD | TDD |
| DLUL configuration |  | N.A | FFS | FFS |
| Propagation channel |  | AWGN | AWGN | AWGN |
| Correlation and antenna configuration |  | 2Rx 2*2 with 4Rx: 2*4 with | 2Rx 2*2 with 4Rx: 2*4 with | 2Rx 2*2 with FFS |
| CSI-RS resource setting (periodic) |  |  |  |  |
| ResourceConfigType |  | Periodic | Periodic | Periodic |
| CSI-RS/CSI-IM periodicity and slot offset |  | [5/0] | [5/0] | [5/0] |
| NZP CSI-RS ports |  | 2 port {3000,3001} | 2 port {3000,3001} | 2 port {3000,3001} |
| NZP CSI-RS configuration CDM type/CSIRS resource mapping/CSI-RS-Density |  | [FD-CDM2/density 1] | [FD-CDM2/density 1] | [FD-CDM2/density 1] |
| CSI-IM configuration |  | FFS | FFS | FFS |
| CSI report setting (periodic) |  |  |  |  |
| reportConfigType |  | periodic | periodic | periodic |
| Reporting periodicity and slot offset |  | FFS | FFS | FFS |
| Codebook Restriction |  | 010000 (fixed rank2) | 010000 (fixed rank2) | 010000 (fixed rank2) |
| Reporting granularity |  | Wide-band | Wide-band | Wide-band |
| Physical channel for reporting |  | PUSCH | PUSCH | PUSCH |
| PUCCH reporting format |  | FFS | FFS | FFS |
| CQI table |  | Table 2 with 256QAM | Table 2 with 256QAM | Table 1 without 256QAM |

Simulation results

Simulation results for FR1 FDD 2Rx and 4Rx were summarized in table 1 and table 2 below.

Table 1: Simulation results for static CQI FR1 FDD 2Rx

| SNR | Median CQI | Percentage of Reported CQI Index in { median CQI1 -1, median CQI1 +1} | BLER with median CQI-1 | BLER with median CQI+1 |
| --- | --- | --- | --- | --- |
| -1dB | 2 | 1 | 0 | 1 |
| 0dB | 2 | 1 | 0 | 0.9998 |
| 1dB | 3 | 1 | 0 | 1 |
| 2dB | 3 | 1 | 0 | 1 |
| 3dB | 3 | 1 | 0 | 1 |
| 4dB | 3 | 1 | 0 | 0.9438 |
| 5dB | 4 | 1 | 0 | 1 |
| 6dB | 4 | 1 | 0 | 1 |
| 7dB | 5 | 1 | 0 | 1 |
| 8dB | 5 | 1 | 0 | 1 |
| 9dB | 6 | 1 | 0 | 1 |
| 10dB | 7 | 1 | 0 | 1 |
| 11dB | 7 | 1 | 0 | 1 |
| 12dB | 7 | 1 | 0 | 1 |
| 13dB | 8 | 1 | 0 | 1 |
| 14dB | 8 | 1 | 0 | 1 |
| 15dB | 9 | 1 | 0 | 1 |
| 16dB | 9 | 1 | 0 | 1 |
| 17dB | 10 | 1 | 0 | 1 |
| 18dB | 10 | 1 | 0 | 1 |
| 19dB | 11 | 1 | 0 | 1 |
| 20dB | 12 | 1 | 0 | 1 |
| 21dB | 12 | 1 | 0 | 1 |
| 22dB | 13 | 1 | 0 | 1 |
| 23dB | 13 | 1 | 0 | 1 |
| 24dB | 14 | 1 | 0 | 1 |
| 26dB | 15 | 1 | 0 | 1 |

Table 2: Simulation results for static CQI FR1 FDD 4Rx

| SNR | Median CQI | Percentage of Reported CQI Index in { median CQI1 -1, median CQI1 +1} | BLER with median CQI-1 | BLER with median CQI+1 |
| --- | --- | --- | --- | --- |
| -5dB | 2 | 1 | 0 | 1 |
| -3dB | 3 | 1 | 0 | 1 |
| -1dB | 3 | 1 | 0 | 1 |
| 0dB | 3 | 1 | 0 | 1 |
| 1dB | 4 | 1 | 0 | 1 |
| 2dB | 4 | 1 | 0 | 1 |
| 3dB | 5 | 1 | 0 | 1 |
| 4dB | 5 | 1 | 0 | 1 |
| 5dB | 5 | 1 | 0 | 1 |
| 6dB | 6 | 1 | 0 | 1 |
| 7dB | 7 | 1 | 0 | 1 |
| 8dB | 7 | 1 | 0 | 1 |
| 9dB | 8 | 1 | 0 | 1 |
| 10dB | 8 | 1 | 0 | 1 |
| 11dB | 9 | 1 | 0 | 1 |
| 12dB | 9 | 1 | 0 | 1 |
| 13dB | 9 | 1 | 0 | 0.078 |
| 14dB | 10 | 1 | 0 | 1 |
| 15dB | 11 | 1 | 0 | 1 |
| 16dB | 11 | 1 | 0 | 1 |
| 17dB | 12 | 1 | 0 | 1 |
| 18dB | 12 | 1 | 0 | 1 |
| 19dB | 13 | 1 | 0 | 1 |
| 20dB | 13 | 1 | 0 | 1 |
| 21dB | 14 | 1 | 0 | 1 |
| 22dB | 14 | 1 | 0 | 0 |

Simulation results for FR2 TDD were provided in table 3 below.

| SNR | Percentage of Reported CQI Index in { median CQI1 -1, median CQI1 +1} | Median CQI | BLER_ medianCQI-1 | BLER_ medianCQI+1 |
| --- | --- | --- | --- | --- |
| -5dB | 1 | 3 | 0.2638 | 1 |
| -4dB | 1 | 3 | 0 | 1 |
| -3dB | 1 | 3 | 0 | 1 |
| -2dB | 1 | 4 | 0 | 1 |
| -1dB | 1 | 4 | 0 | 1 |
| 0dB | 1 | 4 | 0 | 1 |
| 1dB | 1 | 5 | 0 | 1 |
| 2dB | 1 | 5 | 0 | 0.9642 |
| 3dB | 1 | 6 | 0 | 1 |
| 4dB | 1 | 6 | 0 | 0.9576 |
| 5dB | 1 | 7 | 0 | 1 |
| 6dB | 1 | 7 | 0 | 0.7596 |
| 7dB | 1 | 8 | 0 | 1 |
| 8dB | 1 | 8 | 0 | 0.7584 |
| 9dB | 1 | 9 | 0 | 1 |
| 10dB | 1 | 10 | 0 | 1 |
| 11dB | 1 | 10 | 0 | 1 |
| 12dB | 1 | 10 | 0 | 0.75 |
| 13dB | 1 | 11 | 0 | 1 |
| 14dB | 1 | 11 | 0 | 0.6688 |
| 15dB | 1 | 12 | 0 | 0.9044 |
| 16dB | 1 | 13 | 0 | 1 |
| 17dB | 1 | 13 | 0 | 0.7504 |
| 18dB | 1 | 13 | 0 | 0.6432 |
| 19dB | 1 | 14 | 0 | 0.75 |
| 20dB | 1 | 14 | 0 | 0.5818 |
| 21dB | 1 | 15 | 0 | 0 |
| 22dB | 1 | 15 | 0 | 0 |

Based on initial simulation results, we can observe it’s feasible to reuse existing LTE test metric for both FR1 and FR2 CQI table test.

Observation1: It’s feasible to reuse LTE test metric for static CQI test cases of both FR1 and FR2

Proposal 1: Introduce below test metric for static CQI test cases

- The reported CQI value shall be in the range of ±1 of the reported median more than 90% of the time.

- If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Proposal 2: introduce below test SNR points

- FR1 2Rx:[8/9]dB, [14/15]dB

- FR1 4Rx:[5/6]dB, [11/12]dB

- FR2 2Rx:[8/9]dB, [14/15]dB

Conclusion

In this contribution, we provided initial simulation results for static CQI test with below observations:

Observation1: It’s feasible to reuse LTE test metric for static CQI test cases of both FR1 and FR2

Proposal 1: Introduce below test metric for static CQI test cases

- The reported CQI value shall be in the range of ±1 of the reported median more than 90% of the time.

- If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Proposal 2: introduce below test SNR points

- FR1 2Rx:[8/9]dB, [14/15]dB

- FR1 4Rx:[5/6]dB, [11/12]dB

- FR2 2Rx:[8/9]dB, [14/15]dB

Reference

[1] R4-1809358, “Way forward on NR UE CSI requirements”, Samsung