3GPP TSG-RAN4 Meeting #88 	R4-1811405

# Gothenburg, Sweden, 20 – 24 Aug, 2018

Title:	Draft LS for UE capability of measurement gap for inter-RAT NR measurement not yet configured with EN-DC

Response to:	R2-1809203

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG 4

To:	RAN WG 2

CC:

Contact Person:

#### Name:	Jie Cui

E-mail Address:	jie.cui@intel.com

Attachments:	N/A

1. Overall Description:

RAN4 would like to thank RAN2 for the LS on UE capability of measurement gap for inter-RAT NR measurement not yet configured with EN-DC in which RAN2 would like to ask RAN4 whether the MR-DC band combinations reported in UE capability can allow the network to identify the cases where the LTE UE not yet configured with EN-DC can perform gapless measurements.

RAN4 would like to inform RAN2 the following conclusion in RAN4.

- In general, network cannot identify the cases of gapless measurement purely based on MR-DC band combinations reported from UE. However, if this LTE UE (not yet configured with EN-DC) has per-FR measurement capability and it is configured to measure FR2 inter-RAT MO only, gapless measurement is possible.

2. Actions:

To RAN WG2:

RAN4 respectfully asks RAN2 to take the above information into account in their future work.

3. Date of Next RAN4 Meetings:

3GPP RAN4 #88bis 		8 - 12 Oct 2018, 		Chengdu, China

3GPP RAN4 #89 		12 - 16 Nov 2018, 		Spokane, US