3GPP TSG-RAN WG4 Meeting #88	R4-1811415

# Gothenburg, Sweden, 20th- 24th August, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | CRNum | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Carrier-specific scaling factor for multiple measurement objects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core |  |  |  |  |  |  | Date: |  | 2018-23-08 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Scaling of requierments is not specified when multiple measurement objects are configured to be measured in the same gap pattern |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definition of carrier-specific scaling factor CSFi is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requierments for multiple layers will not be specified. Specificaton could be misunderstood as meaning that the single layer requirements apply no matter how many layers are configured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 9.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

9.1.5	Carrier-specific scaling factor

9.1.5.1	NSA: Monitoring of multiple layers using gaps

For UE configured with the E-UTRA-NR dual connectivity operation, the effective total number of frequencies being monitored is N’freq, NSA, which is defined as:

N’freq, NSA = Nfreq, NSA + Nfreq, NSA, NR, intra-freq,

where

Nfreq, NSA is defined in Section 9.1.3.1.

Nfreq, NSA, NR, intra-freq is the number of NR intra-frequency carriers being monitored within the measurement gap

Editor’s Note: the number of intra-freqeuncy carriers Nfreq, NSA, NR, intra-freq only includes the carriers that need to conduct the measurement of intra-frequency measurement with gap and intra-frequency measurement without gap but all the SMTC occasions are overlapped with the measurement gap. (only Typc C and Type A/B under 1a/1b)

The scaling value CSFi below have been derived without considering the BSIC verification of GSM inter-RAT carriers or the phase for BSIC initial search.

For UE which support and are configured with per FR gaps, CSFi is defined on a per frequency range basis, considering the gap pattern and carriers configured in the relevant frequency range. For UE which are configured with per UE gaps, CSFi is defined considering the per UE gap pattern and all confifured carriers.

Considering that there are N'freq,NSA carriers to be measured, denote each carrier with index i where i={0…N’freq,NSA-1}

Determine the gap utilization reputation period G

G=max(MGRP, SMTC period0, … SMTC periodi , PRS period1,…))

Denote each gap in the period G with index j where j={0…(G/MGRP)-1}

For each carrier i, and each gap j, determine if the carrier i is a candidate to be measured in the measurement gap j, considering its SMTC configuration.

An NR carrier is a candidate to be measured if its SMTC occasion at least partially coincides in time with the considered measurement gap excluding RF switching time.

The result of this determination is

λi,j=1 if carrier i is a candidate to be measured in gap j

λi,j=0  otherwise

For each measurement gap, count the number of competing measurement objects according to

θj=i=0Nfreq,NSA-1λi,ji=0Nfreq,NSA-1λi,jθj=i=0Nfreq,NSA'-1λi,j

If gap sharing is configured with non equal sharing, for each measurement gap, count the number of competing intra-frequency measurement objects according to

θintra,j=i=0Nfreq,NSA'-1λi,jXi, where Xi=1,  if carrier i is intra-frequencyXi=0,  otherwise

θinter,j=i=0Nfreq,NSA'-1λi,jYi ,where Yi=1,  if carrier i is inter-frequencyYi=0,  otherwise

For each measurement gap j and each carrier i, compute the assumed measurement scaling factor according to the following

- If λi,j=1 and carrier i involves an LTE RSTD measurement [or GSM BSIC verification measurement] with periodicity Tprs larger than largest configured NR SMTC periodicity then

εi,j=1 and εk,j=0 for all k≠i, and

do not evaluate rules (2), (3), (4) and (5), for gap j.

- If λi,j=1, measGapSharingScheme is not “Equal splitting” and θintra,j>0 and θinter,j>0, and carrier i is an intra-frequency measurement object then  εi,j=K1Nintra,j

εi,j=1Kintra×θintra,j εi,j=1Kintra×θintra, where θintra=max 0≤j'≤GMGRP-1 θintra,j'.=max j θintra,jεi,j=1Kintra×Nintra,j

- If λi,j=1, measGapSharingScheme is not “Equal splitting” and θintra,j>0 and θinter,j>0, and carrier i is an inter-frequency/inter-RAT measurement object then

εi,j=1Kinter×Ninter,jεi,j=1Kinter×θinter, where θinter=max0≤j'≤GMGRP-1 θinter,j'.εi,j=1Kintra×Nintra,j

- If λi,j=1 and measGapSharingScheme is “Equal splitting” then

εi,j=1θ, where  θ=max0≤j'≤GMGRP-1 θj'.

- If  λi,j=0 then

εi,j=0.

For each carrier i, compute the total number of measurement opportunities according to

Δi=j=0GMGRP-1λi,j

The scaling factor for carrier i is then determined according to

CSFi=ceilΔij=0GMGRP-1εi,j

#### 9.1.5.1a	SA: Monitoring of multiple layers using gaps

For UE configured with the NR SA operation, the effective total number of frequencies is N’freq, SA, which is defined as:

N’freq, SA = Nfreq, SA + Nfreq, SA, NR, intra-freq,

where

Nfreq, SA, NR is defined in Section 9.1.3.1a.

Nfreq, SA, NR, intra-freq is the number of NR intra-frequency carriers being monitored within the measurement gap

Editor’s Note: the number of intra-freqeuncy carriers Nfreq, NSA, NR, intra-freq only includes the carriers that need to conduct the measurement of intra-frequency measurement with gap and intra-frequency measurement without gap but all the SMTC occasions are overlapped with the measurement gap. (only Typc C and Type A/B under 1a/1b)

For UE which support and are configured with per FR gaps, CSFi is defined on a per frequency range basis, considering the gap pattern and carriers configured in the relevant frequency range. For UE which are configured with per UE gaps, CSFi is defined considering the per UE gap pattern and all confifured carriers.

Considering that there are N’freq,SA carriers to be measured, denote each carrier with index i where i={0…N’freq,SA-1}

Determine the gap utilization reputation period G

G=max(MGRP, SMTC period0, … SMTC periodi , PRS period1,…))

Denote each gap in the period G with index j where j={0…(G/MGRP)-1}

For each carrier i, and each gap j, determine if the carrier i is a candidate to be measured in the measurement gap j, considering its SMTC configuration.

An NR carrier is a candidate to be measured if its SMTC occasion at least partially coincides in time with the considered measurement gap excluding RF switching time.

The result of this determination is

λi,j=1 if carrier i is a candidate to be measured in gap j

λi,j=0 otherwise

For each measurement gap, count the number of competing measurement objects according to

θj=i=0Nfreq,NSA-1λi,jθj=i=0Nfreq,SA'-1λi,j

If gap sharing is configured with non equal sharing, for each measurement gap, count the number of competing intra-frequency measurement objects according to

θj=i=0Nfreq,NSA-1λi,jθintra,j=i=0Nfreq,SA'-1λi,jXi, where Xi=1,  if carrier i is intra-frequencyXi=0,  otherwise

θj=i=0Nfreq,NSA-1λi,jθinter,j=i=0Nfreq,SA'-1λi,jYi ,where Yi=1,  if carrier i is inter-frequencyYi=0,  otherwise

For each measurement gap j and each carrier i, compute the assumed measurement scaling factor according to the following

- If λi,j=1 and carrier i involves an LTE RSTD measurement with periodicity Tprs larger than largest configured NR SMTC periodicity then

εi,j=1 and εk,j=0 for all k≠i, and

do not evaluate rules (2), (3), (4) and (5), for gap j.

- If λi,j=1, measGapSharingScheme is not “Equal splitting” and θintra,j>0 and θinter,j>0, and carrier i is an intra-frequency measurement object then  εi,j=K1Nintra,j

εi,j=1Kintra×θintra, where θintra=max 0≤j'≤GMGRP-1 θintra,j'.εi,j=1Kintra×Nintra,j

- If λi,j=1, measGapSharingScheme is not “Equal splitting” and θintra,j>0 and θinter,j>0, and carrier i is an inter-frequency/inter-RAT measurement object then

εi,j=1Kinter×Ninter,jεi,j=1Kinter×θinter, where θinter=max0≤j'≤GMGRP-1 θinter,j'.εi,j=1Kintra×Nintra,j

- If λi,j=1 and measGapSharingScheme is “Equal splitting” then

εi,j=1θ, where  θ=max0≤j'≤GMGRP-1 θj'.

- If  λi,j=0 then

εi,j=0.

For each carrier i, compute the total number of measurement opportunities according to

Δi=j=0GMGRP-1λi,j

The scaling factor for carrier i is then determined according to

CSFi=ceilΔij=0GMGRP-1εi,j