3GPP TSG RAN WG4 Meeting #88	R4-1811487

Gothenburg, Sweden, Aug 20-24, 2018

Source: 	Qualcomm Incorporated

Title: 	Update FR2 CA MPR

Agenda item:	7.6.16.5.2

Document for:	Approval

# Introduction

We would like to clarify and generalize PC3 CA MPR and PC1 CA MPR.

# Discussion

In Busan[1], we had late contribution that generalized PC3 CA MPR. We could like to extend that work to the current agreement and CA MPR table shown in [2] and generalize CA FR2 MPR for both Power class 1 and Power class 3. Currently, the MPR table is only for aggregated BW <=400M for only Power Class3.

To generalize, we would like to review some observations when the CA cumulative aggregated BW increases

- MPR for DFT-s-OFDM increases per number of CC’s due to increasing PAPR

- MPR increases due to large BW for all waveforms due to RF droop effects and PA memory effects.

Based on these 2 items, we showed the following:

MPRcontiguous=MPRsingleCCMaxCCBW+∆MPRnCC+∆MPRBW

Where:

∆MPRBW= .1875*AggBWMaxccBW…….See Figure 1

∆MPRnCC=See Table 1

Figure 1: MPR delta due to large BW

Figure 1 shows that how the MPR can increase when having a 100MCC or a 400M CC. The MPR delta is more significant when a 100M CC is used as opposed to a 400M CC.

Table 1: ∆MPRnCC  as a function of Waveform and Modulation Order

| Wave-type | Mod | MPRsingleCC dB [3] |  |  |  | ∆MPRnCC, dB |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50MHz | 100MHz | 200MHz | 400MHz | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| DFT-s-OFDM | BPSK | 1.5 | 1.5 | 1.5 | 3 | 0 | 1 | 1.5 | 1.5 | 2 | 2.5 | 2.5 | 3 |
|  | QPSK | 1.5 | 1.5 | 1.5 | 3 | 0 | 1 | 1.5 | 1.5 | 2 | 2.5 | 2.5 | 3 |
|  | 16QAM | 2.5 | 2.5 | 2.5 | 4 | 0 | 1 | 1.5 | 1.5 | 2 | 2.5 | 2.5 | 3 |
|  | 64QAM | 4.5 | 4.5 | 4.5 | 6 | 0 | 1 | 1.5 | 1.5 | 2 | 2.5 | 2.5 | 3 |
| CP-OFDM | QPSK | 3.5 | 3.5 | 3.5 | 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 16QAM | 4.5 | 4.5 | 4.5 | 6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 64QAM | 7 | 7 | 7 | 8.5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 1 shows simulations and measurements of how the MPR increases when the number of CC’s increase for DFT-s-OFDM.

As a result of figure 1 and Table 1, we would like to propose the CA MPR in Table 2. The first column in the table has already been agreed. We are adding 2nd and 3rd columns as highlighted.

The following notes apply to CA FR2 MPR:

- The UE is defined to be configured for CA operation when it has at least one of UL or DL configured for CA.

- In CA operation, the UE may reduce its maximum output power due to higher order modulations and transmit bandwidth configurations.

- The cumulative aggregated channel bandwidth is defined as the frequency difference between the highest edge of the highest CC and the lower edge of the lowest CC of all UL and DL configured CCs as in section 2.3.

- The requirements of this section also apply to UEs configured for single CC operation with different channel bandwidths in UL and DL.

- When different waveform types exist across CCs, the requirement is set by the waveform type used in the configuration with the highest contiguous MPR.

## Power Class 3

Proposal 1: For Power Class 3, use the MPR values in Table 2

Table 2: Maximum power reduction (MPRC_CA) for UE power class 3

|  |  | Cumulative aggregated channel bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | < 400MHz | >=400M and <800M | >=800M and <=1400M |
| DFT-s-OFDM | Pi/2 BPSK | 5.0 | 7.7 | 8.2 |
|  | QPSK | 5.0 | 7.7 | 8.2 |
|  | 16 QAM | 6.0 | 8.2 | 8.7 |
|  | 64 QAM | 8.5 | 10.7 | 11.2 |
| CP-OFDM | QPSK | 5.0 | 7.2 | 7.7 |
|  | 16 QAM | 6.0 | 8.2 | 8.7 |
|  | 64 QAM | 8.5 | 10.7 | 11.2 |

From [3], For non-contiguous RB allocations, the following rule for MPR applies:

MPRNC_CA = max(MPRC_CA, -10*A + 11.0)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of simultaneously transmitted UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth

## Power Class 1

We note that for Power Class 1, the PA will be biased higher to meet EVM requirements. However, the SEM mask will remain at the same level as Power class 3, so for < 400MHz, we expect the MPR values to be dominated by SEM more so than EVM [4]. But, as the bandwidth increases, the EVM will dominate as in Power Class 3. So, the same delta over aggregated BW should apply for Power Class 1 as it did in Power Class 3.

Proposal 2: For Power Class 1, use the MPR values in Table 3

Table 3: Maximum power reduction (MPRWT_C_CA) for UE power class 1

| Waveform Type |  | Cumulative aggregated channel bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | < 400MHz | >=400M and <800M | >=800M and <=1400M |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | 7.7 | 8.2 |
|  | QPSK | ≤ 6.5 | 8.7 | 9.7 |
|  | 16 QAM | ≤ 6.5 | 8.7 | 9.2 |
|  | 64 QAM | ≤ 6.5 | 10.7 | 11.2 |
| CP-OFDM | QPSK | ≤ 6.5 | 8.7 | 8.7 |
|  | 16 QAM | ≤ 6.5 | 8.7 | 8.7 |
|  | 64 QAM | ≤ 7.5 | 10.7 | 11.2 |

From [3], For non-contiguous RB allocations, the following rule for MPR applies:

MPRNC_CA = max(MPRC_CA, -10*A + 11.0)

Where:

A = NRB_alloc / NRB_agg_C.

NRB_alloc is the total number of simultaneously transmitted UL RBs

NRB_agg_C is the number of the aggregated RBs within the fully allocated cumulative aggregated channel bandwidth

## Cumulative aggregated channel bandwidth definition

The cumulative aggregated channel bandwidth is defined as the frequency difference between the highest edge of the highest CC and the lower edge of the lowest CC of all UL and DL configured CCs

DL Configuration

UL Configuration

CC

Cumulative Aggregated Bandwidth

Figure 1. Contiguous intraband CA Cumulative Aggregated Bandwidth

DL Configuration

UL Configuration

CC

Cumulative Aggregated Bandwidth

Figure 2. Non-contiguous intraband CA Cumulative Aggregated Bandwidth

# Conclusion

Proposal 1:  For Power Class 3 use MPR values in Table 2

Proposal 2:  For Power Class 1 use MPR values in Table 3

# References

- R4-180xxxx, CA FR2 MPR rev4, on reflector, 3GPP TSG RAN WG4 Meeting #88, Busan, KR.

- 3GPP TS 38.101-2 V15.2.0 (06-18)

- R4-1810458, Analyzing CA MPR for non-contiguous RB allocations with multiple CC's, Gothenburg, SWE

- R4-1810431, PC1 FR2 MPR, Gothenburg, SWE