3GPP TSG-RAN WG4 Meeting #88	R4-1811879

Gothenburg, Sweden, 20th – 24th August 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TS 38.141-2 – Overview of radiated Tx and Rx requirements (4.13)

Agenda item:	7.9.4.2.2

Document for:	Approval

# 1	Introduction

This is a revision of R4-1811131, which adds two tables that provides an overview of radiated transmitter and receiver requirements to TS 38.141-2 [1].

Constructive comments are encouraged.

# 4	References

- After Montreal Version, Draft TS 38.141-2 (v0.3.0), Huawei

# 5	Text proposal

Draft TS 38.141-2 v0.3.0_afterMontreal

--------------Start of text proposal-------------

## 4.1	Measurement uncertainties and test requirements

### 4.1.1	General

The requirements of this clause apply to all applicable tests in 3GPP TS 38.141-2 (the present document), i.e. to all radiated tests defined for BS type 1-H, BS type 1-O and BS type 2-O.

The minimum requirements are given in TS 38.104 [2]. Test Tolerances for the radiated test requirements (TTOTA) explicitly stated in the present document are given in annex C.

Test Tolerances are individually calculated for each test. Test Tolerances are used to relax the minimum requirements to create test requirements.

When a test requirement differs from the corresponding minimum requirement, then the Test Tolerance applied for the test is non-zero. The Test Tolerance for the test and the explanation of how the minimum requirement has been relaxed by the Test Tolerance are given in annex C.

Table 4.1.1-1: Overview of radiated Tx requirements

| Tx requirement |  | Classification |  | Notes |
| --- | --- | --- | --- | --- |
| Radiated transmit power |  | Directional – OTA peak directions set |  | This requirement is based on Rel-13 AAS BS requirement for EIRP accuracy |
| OTA BS output power |  | TRP |  |  |
| OTA output power dynamics |  | Directional – OTA peak directions set |  | Conformance testing is carried in the reference direction |
| OTA transmit ON/OFF power |  | FR1 | FR2 |  |
|  |  | Co-location | TRP |  |
| OTA transient period |  | Co-location |  |  |
| OTA modulation quality |  | Directional – OTA coverage range |  | Conformance testing is carried in the reference direction and the maximum directions of the OTA coverage range on each axis |
| OTA frequency error |  | Directional – OTA coverage range |  | Conformance testing is carried out in the reference direction |
| OTA time alignment error |  | Directional – OTA coverage range |  | Conformance testing is carried out in the reference direction |
| OTA occupied bandwidth |  | Directional – OTA coverage range |  | Conformance testing is carried out in the reference direction |
| OTA ACLR |  | TRP |  |  |
| OTA operating band unwanted emission |  | TRP |  |  |
| OTA transmitter spurious emission | Mandatory | TRP |  | For FR1 and FR2 |
|  | Protection of the BS receiver of own or different BS | Co-location |  | For FR1 only |
|  | Additional spurious emissions | TRP |  | For FR1 only |
|  | Co-location with other base stations | Co-location |  | For FR1 only |
| OTA transmitter intermodulation |  | Co-location |  | For FR1 only |
| NOTE: Directional does not imply one compliance direction only. The requirement applies to a single direction at a time. |  |  |  |  |

Table 4.1.1-2: Overview of radiated Rx requirements

| Rx requirement |  | Classification | Applicability levels |  | Coverage range |  | Number of conformance directions |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FR1 | FR2 | FR1 | FR2 |  |
| OTA sensitivity |  | Directional | Minimum EIS | N/A | OSDD | N/A | 5 |
| OTA reference sensitivity |  | Directional | OTA REFSENS | OTA REFSENS | OTA REFSENS RoAoA |  | 5 |
| OTA Dynamic range |  | Directional | OTA REFSENS | N/A | OTA REFSENS RoAoA |  | 1 |
| OTA adjacent channel selectivity |  | Directional | minSENS | OTA REFSENS | minSENS RoAoA | OTA REFSENS RoAoA | 1 |
| OTA in-band blocking |  | Directional | OTA REFSENS and minSENS | OTA REFSENS | OTA REFSENS RoAoA and minSENS RoAoA | OTA REFSENS RoAoA | 5 |
| OTA out-of-band blocking | mandatory | Directional | minSENS | OTA REFSENS | minSENS RoAoA | OTA REFSENS RoAoA | 1 |
|  | Co-location with other base stations | Co-location | For FR1 only |  | N/A |  | N/A |
| OTA receiver spurious emissions |  | TRP | N/A |  | N/A |  | N/A |
| OTA receiver intermodulation |  | Directional | OTA REFSENS and minSENS | OTA REFSENS | OTA REFSENS RoAoA and minSENS RoAoA | OTA REFSENS RoAoA | 1 |
| OTA in-channel selectivity |  | Directional | minSENS | OTA REFSENS | minSENS RoAoA | OTA REFSENS RoAoA | 1 |
| NOTE: Directional does not imply one compliance direction only. The requirement applies to a single direction at a time. |  |  |  |  |  |  |  |

--------------end of text proposal-------------