3GPP TSG-RAN WG4 Meeting #88bis	R4-1812131

# Chengdu, China, 8th – 12th, October 2018

Agenda item:	7.6.9

Source: 	Intel Corporation

Title: 	Simultaneous RxTx - DC_20_SUL_n78-n83 and DC_20_n78 MSD

Document for:	Approval

# Introduction

In the RAN4#88meeting we provided our contribution [1] to discuss the simultaneous RxTx capability for several DC/CA combinations. In this contribution we concentrate the discussion on the SUL combination DC_20_SUL_n78-n83 and DC_20_n78.

# Discussion

In this contribution we concentrate the discussion on the SUL combination DC_20_SUL_n78-n83 and DC_20_n78. The first subsection provides a summary of the current definition of REFSENS and MSD for the bands 20, 42 and n78 and the second subsection shows an analysis of the harmonics and intermodulation for band 20 and n78.

## Reference Sensitivity and MSD

FDD band B20 and TDD band B42 (frequency range between 3400 to 3600 MHz) can be compared to combination of B20 and TDD band n78. In TS36.101 for CA_20A-42A an exception due to harmonic issue for the reference sensitivity has been defined. The reference sensitivity of B42 has been modified in between 14.2 dBm and 8.5 dBm depending on the bandwidth as shown in Table 2.

Table 1: Reference sensitivity QPSK PREFSENS

| E-UTRA Band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- | --- |
| 20 | -97 | -94 | -91.2 | -90 | FDD |
| 42 | -99 | -96 | -94.2 | -93 | TDD |

Table 2: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA 
(exceptions due to harmonic issue)

| EUTRA CA Configuration | EUTRA band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_20A-42A12,13, CA_20A-42A-42A12,13 | 20 | -97 | -94 | -91.2 | -90 | FDD |
|  | 4233 | -84.8 | -84.7 | -84.6 | -84.5 | TDD |

In TS38.101-3 MSD values have been considered for dual uplink operation between B20 and n78 as in Table 3. An 4th order intermodulation issues fall into RX B20, however the case for 4th harmonic when B20 is transmitting and n78 is receiving has not been taken into account.

- MSD Definition only for 2UL: Reference sensitivity exceptions for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving two bands (38.101-3)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_20A_n78A, DC_20A-SUL_n78A-n82A | 20 | 850 | 5 | 25 | 810 | 21.7 | FDD | IMD44 |
|  | n78 | 3360 | 10 | 50 | 3360 | N/A | TDD | N/A |

## Harmonics and intermodulation analysis

In TS38.0101-3 was only defined the MSD for dual uplink for DC_20A_n78A and DC_20A_SUL_n78A-n82A. However, it has not been analysis the 4th harmonic for the case where B20 is transmitting and n78 is receiving, which can be for the DC_20_n78, DC_20_SUL_n78-n82 and DC_20_SUL_n78-n83. The table below shows the case where B20 is UL and n78 is DL and provides with a “TRUE” for the case harmonic or intermodulation occurs.

Table 3: B20 and n78 harmonics and IMD

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 6600 | 8400 |
|  | FALSE | FALSE | FALSE | FALSE |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 9900 | 12600 |
|  | FALSE | FALSE | FALSE | FALSE |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_low |
| 4th harmonics frequency limits (MHz) | 3328 | 3448 | 13200 | 16800 |
|  | FALSE | TRUE | FALSE | FALSE |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2438 | 3368 | 4132 | 5062 |
|  | 2438 | 3368 | 4132 | 5062 |
|  | FALSE |  | FALSE |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2536 | 1576 | 5738 | 7568 |
|  | 1576 | 2536 | 5738 | 7568 |
|  | FALSE | FALSE | FALSE | FALSE |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4964 | 5924 | 7432 | 9262 |
|  | 4964 | 5924 | 7432 | 9262 |
|  | FALSE | FALSE | FALSE | FALSE |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ | ¦2*fy_low – 2*fx_high¦ | ¦2*fy_high – 2*fx_low¦ |
| IMD frequency limits (MHz) | 8264 | 10124 | 4876 | 6736 |
|  | 8264 | 10124 | 4876 | 6736 |
|  | FALSE | FALSE | FALSE |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1704 | 714 | 9038 | 11768 |
|  | 714 | 1704 | 9038 | 11768 |
|  | TRUE | FALSE | FALSE | FALSE |
| Two-tone 4th order IMD products | ¦3*fx_low +* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 | 6786 | 10732 | 13462 |
|  | 5796 | 6786 | 10732 | 13462 |
|  | FALSE | FALSE | FALSE | FALSE |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15968 | 12338 | 148 | 872 |
|  | 12338 | 15968 | 148 | 872 |
|  | FALSE | FALSE | TRUE | FALSE |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 | 17662 | 6628 | 7648 |
|  | 14032 | 17662 | 6628 | 7648 |
|  | FALSE | FALSE | FALSE | FALSE |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 10936 | 8176 | 4014 | 5904 |
|  | 8176 | 10936 | 4014 | 5904 |
|  | FALSE | FALSE | FALSE | FALSE |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 | 14324 | 9096 | 10986 |
|  | 11564 | 14324 | 9096 | 10986 |
|  | FALSE | FALSE | FALSE | FALSE |

The analysis results showed that there is not only an issue for 4th order intermodulation and 5th order intermodulation but also for the 4th harmonic. Therefore, we propose the study of the MSD for the 4th harmonic for the EN-DC combination when B20 is UL and n78 is DL.

Observation 12: The analysis showed a 4th harmonic issue when B20 is transmitting and n78 is receiving.

Proposal 1:	The study of the MSD for the 4th harmonic for the EN-DC combination B20 and n78.

# Conclusion

In this contribution, we provide an analysis of the harmonics and intermodulation for B20 and n78 and propose the following:

Observation 1: The analysis showed a 4th harmonic issue when B20 is transmitting and n78 is receiving, besides the 4th order intermodulation issue for dual uplink.

Proposal 1:	The study of the MSD for the 4th harmonic for the EN-DC combination B20 and n78.

# References

- R4-1810958 “Simultaneous RxTx”, Intel, RAN4 #88.

- 3GPP TS 38.101-3 V15.2.0 (2018-09)