3GPP TSG-RAN WG4 Meeting #88bis 	R4-1812154

Chengdu, China, October 8 – 12, 2018

Agenda item:	7.12.5.11

Source:	Intel Corporation

Title:	Test case list for timing advance

Document for:	Approval

# Introduction

Test case list for timing advance was discussed in [1] in last RAN4 #88. Since no formal agreement was made, we would like to propose some updated test case list based on the discussion in the last meeting and the email discussion after the meeting.

# Discussion

## Test case list

It was agreed in [2] that the TA accuracy requirement needs to be tested separately under EN-DC and SA operation.

| Test case number | Test purpose | Company for CR editor |
| --- | --- | --- |
| 1 | EN-DC/SA cell search and L1 meas period | Huawei |
| 3 | EN-DC Timing accuracy and adjustment | Qualcomm |
| 4 | SA Timing accuracy and adjustment (draft if possible for RAN4#88, otherwise RAN4#88bis) | Qualcomm |
| 5 | EN-DC TA accuracy | Intel |
| 6 | SA TA accuracy | Intel |
| 7 | EN-DC SSB RLM for PSCell IS and OOS | Mediatek |
| 9 | SA SSB RLM for PCell IS and OOS | Mediatek |
| 10 | Random access | Samsung |
| 11 | Intra-freq RSRP accuracy for FR1 and FR2 | Ericsson |
| 12 | EN-DC SCell activation/deact delay | Nokia |
| 13 | EN-DC/SA CSI RLM for PSCell or PCell | Nokia |
| 14 | EN-DC interruptions due to DRX transition or deactivated SCell operationsa | CATT |

As can be observed in TS38.133 section 7.3, TA accuracy requirement is not differentiated between EN-DC and SA. Thus the same cell specific test parameters for the NR cell could be used in EN-DC and SA test cases. Furthermore, it was agreed in the NR RRM test cases ad-hoc meeting in RAN4 #88 on the numerology selection:

- TA accuracy tests:

  - Typical numerology selection for FR1

    - Define both and UE needs to pass one of them: 15k for FDD and 30k for TDD

We’d like to use these two numerologies for FR1 testing. Regarding FR2 testing, since only TDD is supported, we believe one test would be enough and we suggest to use 120k numerology.

As for other parameters, e.g. BW, ONCG, RMC and etc. there is also agreement reached under corresponding session. Here we would like to summarize the test case list:

Proposal 1: TA accuracy test case list is as below:

| Operation mode | Test case | SCS and BW |
| --- | --- | --- |
| EN-DC | - LTE FDD PCell + FR1 FDD PScell | 15kHz & 10MHz |
|  | - LTE FDD PCell + FR1 TDD PScell | 30kHz & 40MHz |
|  | - LTE FDD PCell + FR2 TDD PScell | 120kHz & 100MHz |
| SA | - FR1 FDD PCell | 15kHz & 10MHz |
|  | - FR1 TDD PCell | 30kHz & 40MHz |
|  | - FR2 TDD PCell | 120kHz & 100MHz |

## Test parameters

Table 1. Test case summary

| - No | Feature/core requirements | Type of Test | No of tests | Basic test configuration | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Timing Advance (section 7.3.1) | UE Timing Advance Adjustment Accuracy Test with FDD E-UTRAN PCell and FDD PSCell in FR1 | 1 | Number of cells = 2. Inter-Band EN-DC is configured. Cell1 = PCell (E-UTRAN FDD). Cell2 = PScell(NR FDD in FR1). Cell BW = 10+10 MHz Cell2 SCS=15kHz Non-DRX. Channel model: AWGN Two TAG configuration are configured, PCell in pTAG and PSCell in sTAG. General Test Parameters for NR PSCell in this test case is in Table 2. General Test Parameters for E-UTRAN PCell in this test case is in Table A.7.2.4.1-1 in TS36.133. Cell specific Test Parameters for NR PSCell in this test case is in Table 3. Cell specific Test Parameters for E-UTRAN PCell in this test case is in Table A.3.7.3.1-1. Sounding Reference Symbol Configuration for this test case is in table 4. | Two cells are used. The test is performed on PSCell. The E-UTRAN PCell is needed to be able to configure the PSCell. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table 4, are sent from the UE and received by the test equipment. New proposed section: A.7.3.1.1 UE Timing Advance Adjustment Accuracy Test with FDD E-UTRAN PCell and FDD PSCell in FR1 |
| 2 | Timing Advance (section 7.3.1) | UE Timing Advance Adjustment Accuracy Test with FDD E-UTRAN PCell and TDD PSCell in FR1 | 1 | Number of cells = 2. Inter-Band EN-DC is configured. Cell1 = PCell (E-UTRAN FDD). Cell2 = PScell(NR FDD in FR1). Cell BW = 10+40 MHz Cell2 SCS=30kHz Non-DRX. Channel model: AWGN Two TAG configuration are configured, PCell in pTAG and PSCell in sTAG. General Test Parameters for NR PSCell in this test case is in Table 2. General Test Parameters for E-UTRAN PCell in this test case is in Table A.7.2.4.1-1 in TS36.133. Cell specific Test Parameters for NR PSCell in this test case is in Table 3. Cell specific Test Parameters for E-UTRAN PCell in this test case is in Table A.3.7.3.1-1. Sounding Reference Symbol Configuration for this test case is in table 4. | Two cells are used. The test is performed on PSCell. The E-UTRAN PCell is needed to be able to configure the PSCell. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table 4, are sent from the UE and received by the test equipment. New proposed section: A.7.3.1.2 UE Timing Advance Adjustment Accuracy Test with FDD E-UTRAN PCell and TDD PSCell in FR1 |
| 3 | Timing Advance (section 7.3.1) | UE Timing Advance Adjustment Accuracy Test with FDD E-UTRAN PCell and TDD PSCell in FR2 | 1 | Number of cells = 2. Inter-Band EN-DC is configured. Cell1 = PCell (E-UTRAN FDD). Cell2 = PScell(NR FDD in FR1). Cell BW = 10+100 MHz Cell2 SCS=120kHz Non-DRX. Channel model: AWGN Two TAG configuration are configured, PCell in pTAG and PSCell in sTAG. General Test Parameters for NR PSCell in this test case is in Table 2. General Test Parameters for E-UTRAN PCell in this test case is in Table A.7.2.4.1-1 in TS36.133. Cell specific Test Parameters for NR PSCell in this test case is in Table 3. Cell specific Test Parameters for E-UTRAN PCell in this test case is in Table A.3.7.3.1-1. Sounding Reference Symbol Configuration for this test case is in table 4. | Two cells are used. The test is performed on PSCell. The E-UTRAN PCell is needed to be able to configure the PSCell. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table 4, are sent from the UE and received by the test equipment. New proposed section: A.7.3.1.3 UE Timing Advance Adjustment Accuracy Test with FDD E-UTRAN PCell and TDD PSCell in FR2 |
| 4 | Timing Advance (section 7.3.2) | UE Timing Advance Adjustment Accuracy Test with FDD PCell in FR1 | 1 | Number of cells = 1. Cell1 = PCell (NR FDD in FR1). Cell BW = 10 MHz Cell1 SCS=15kHz Non-DRX. Channel model: AWGN General Test Parameters for this test case is in Table 2. Cell specific Test Parameters for NR PScell in this test case is in Table 3. Sounding Reference Symbol Configuration for this test case is in table 4. | One cell is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table 4, are sent from the UE and received by the test equipment. New proposed section: A.7.3.2.1 UE Timing Advance Adjustment Accuracy Test with FDD PCell in FR1 |
| 5 | Timing Advance (section 7.3.2) | UE Timing Advance Adjustment Accuracy Test with TDD PCell in FR1 | 1 | Number of cells = 2. Inter-Band EN-DC is configured. Cell1 = PCell (E-UTRAN FDD). Cell2 = PScell(NR FDD in FR1). Cell BW = 10+40 MHz Cell2 SCS=30kHz Non-DRX. Channel model: AWGN Two TAG configuration are configured, PCell in pTAG and PSCell in sTAG. General Test Parameters for NR PSCell in this test case is in Table 2. General Test Parameters for E-UTRAN PCell in this test case is in Table A.7.2.4.1-1 in TS36.133. Cell specific Test Parameters for NR PSCell in this test case is in Table 3. Cell specific Test Parameters for E-UTRAN PCell in this test case is in Table A.3.7.3.1-1. Sounding Reference Symbol Configuration for this test case is in table 4. | One cell is used The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table 4, are sent from the UE and received by the test equipment. New proposed section: A.7.3.2.2 UE Timing Advance Adjustment Accuracy Test with TDD PSCell in FR1 |
| 6 | Timing Advance (section 7.3.2) | UE Timing Advance Adjustment Accuracy Test with TDD PCell in FR2 | 1 | Number of cells = 2. Inter-Band EN-DC is configured. Cell1 = PCell (E-UTRAN FDD). Cell2 = PScell(NR FDD in FR1). Cell BW = 10+100 MHz Cell2 SCS=120kHz Non-DRX. Channel model: AWGN Two TAG configuration are configured, PCell in pTAG and PSCell in sTAG. General Test Parameters for NR PSCell in this test case is in Table 2. General Test Parameters for E-UTRAN PCell in this test case is in Table A.7.2.4.1-1 in TS36.133. Cell specific Test Parameters for NR PSCell in this test case is in Table 3. Cell specific Test Parameters for E-UTRAN PCell in this test case is in Table A.3.7.3.1-1. Sounding Reference Symbol Configuration for this test case is in table 4. | One cell is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table 4, are sent from the UE and received by the test equipment. New proposed section: A.7.3.2.3 UE Timing Advance Adjustment Accuracy Test with TDD PSCell in FR2 |

Table 2. General test parameters of NR PCell/PScell for TA accuracy tests

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| SMTC configuration |  | SMTC.1 FR1 for FDD NR cell in FR1; SMTC.2 FR1 for TDD NR cell in FR1; SMTC.1 FR2 for TDD NR cell in FR2 | As specified in clause A.3.2.1 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA_new = NTA_old for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | NTA_new = NTA_old + 8192*Tc (based on equation in TS38.213 section 4.2) |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table 3. Cell specific test parameters for NR PCell/PScell for TA accuracy tests

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| NR RF Channel Number |  | “1” in SA test case and “2” in EN-DC test case |  |
| BWchannel | MHz | 10 for 15kHz SCS; 40 for 40kHz SCS; 100 for 120kHz SCS |  |
| PDSCH parameters defined in clause A.3.1.1 |  | SR.1.1 FDD for FDD NR cell in FR1; SR.2.1 TDD for TDD NR cell in FR1; SR.3.1 TDD for TDD NR cell in FR2 |  |
| RMSI CORESET defined in clause A.3.1.2 |  | CR.1.1 FDD for FDD NR cell in FR1; CR.2.1 TDD for TDD NR cell in FR1; CR.3.1 TDD for TDD NR cell in FR2 |  |
| OCNG Patterns defined in A.3.2.1.1 |  | OP.1 |  |
| ratio of SSS EPRE to PBCH DM-RS EPRE | dB | 0 |  |
| ratio of PBCH EPRE to PBCH DM-RS EPRE | dB |  |  |
| ratio of PDSCH EPRE to PBCH DM-RS EPRE | dB |  |  |
| ratio of PDCCH EPRE to PBCH DM-RS EPRE | dB |  |  |
| ratio of OCNG EPRE to PBCH DM-RS EPRE | dB |  |  |
| Timing Advance Command (TA) |  | 31 | 39 |
|  | dB | 3 |  |
|  | dBm/SCS | -98 |  |
|  | dB | 3 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that cell(s) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 4. Sounding Reference Symbol Configuration for NR PCell/PScell for TA accuracy tests

| Field | Value | Comment |
| --- | --- | --- |
| c-SRS | 12 for 10MHz; 24 for 40MHz; 16 for 100MHz | frequency hopping is disabled, SRS BW= 48PRBs for 10MHz test; 96PRBs for 40MHz test; 60PRBs for 100MHz test; |
| b-SRS | 0 |  |
| b-hop | 0 |  |
| freqDomainPosition | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 |  |
| groupOrSequenceHopping | neither | No group or sequence hopping |
| SRS-PeriodicityAndOffset | sl5=0 | Once every 5 slots |
| pathlossReferenceRS | ssb-Index=0 | SSB #0 is used for SRS path loss estimation |
| usage | nonCodebook | Non-codebook based UL transmission |
| startPosition | 0 | resourceMapping setting. SRS on last symbol of slot, and 1symbols for SRS without repetition. |
| nrofSymbols | n1 |  |
| repetitionFactor | n1 |  |
| combOffset-n2 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 |  |
| nrofSRS-Ports | port1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 38.331. |  |  |

# Conclusion

In this contribution we summarize the test case list and test parameters for timing advance accuracy. After discussion the following test case list is proposed:

Proposal 1: TA accuracy test case list is as below:

| Operation mode | Test case | SCS and BW |
| --- | --- | --- |
| EN-DC | - LTE FDD PCell + FR1 FDD PScell | 15kHz & 10MHz |
|  | - LTE FDD PCell + FR1 TDD PScell | 30kHz & 40MHz |
|  | - LTE FDD PCell + FR2 TDD PScell | 120kHz & 100MHz |
| SA | - FR1 FDD PCell | 15kHz & 10MHz |
|  | - FR1 TDD PCell | 30kHz & 40MHz |
|  | - FR2 TDD PCell | 120kHz & 100MHz |

# References

- R4-1809867, “On test case design for TA accuracy”, Intel Corporation.

- R4-1809388, “Test case list for NR”, Ericsson, RAN4 #1807

- R4-1811713, “Ad hoc minutes for RRM test cases”, Huawei, HiSilicon