3GPP TSG-RAN WG4 Meeting #88bis	R4-1812170

Chengdu, China, 8th – 12th Oct, 2018

Agenda item:	6.6.4.1

Source: 		Intel Corporation

Title: 			Requirements in fading channel conditions with 1024QAM

Document for:	Discussion

# Introduction

In RAN4#88 the test cases for PDSCH demodulation requirements for 1024QAM under fading conditions were agreed in [1]

| No. | TM | PMI feedback (pre-coding) | MIMO layer | MCS | Antenna config. |
| --- | --- | --- | --- | --- | --- |
| 1 | TM4 | Wideband PMI Feedback (PUSCH 3-1, Reporting interval =1) | 1-layer | MCS#23 (with 52752 bits) | 4x2 Low |
| 2 | TM4 | Wideband PMI feedback (PUSCH 3-1, Reporting interval =1) | 2-layer | MCS#23 (with 52752 bits) | 4x4 Low |
| 3 | TM9 | Per-PRG Random Precoder | 1-layer | MCS#23 (with 52752 bits) | 4x2 Low |
| 4 | TM9 | Wideband PMI feedback | 2-layer | MCS#23 (with 52752 bits) | 4x4 Low |

Companies were encouraged to provide simulation results with impairments for 1024QAM DL FDD and TDD demodulation tests in fading channel conditions. In this contribution we provide simulation results with impairments for defining requirements with 1024QAM.

# Simulation Results

## FDD

The simulation assumptions and FRC tables for FDD testcases are listed in Appendix A.1

The impairment results for CRS and DMRS based test cases are provided in Table 1 below:

Table 1: Impairment results for FDD test cases

| Test number | Bandwidth and MCS | Transmission Mode/ # MIMO Layers | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1 | 10 MHz 1024QAM | TM4; 1 Layer | EPA5 | 4x2 Low | 70 | 24.5 |
| 2 | 10MHz 1024QAM | TM4; 2 Layers | EPA5 | 4x4 Low | 70 | 27.1 |
| 3 | 10 MHz 1024QAM | TM9; 1 Layer | EPA5 | 4x2 Low | 70 | 30.3 |
| 4 | 10MHz 1024QAM | TM9; 2 Layers | EPA5 | 4x4 Low | 70 | 28.5 |

## TDD

For TDD testcases the simulation assumptions have not yet been agreed and alignment results have not been provided by companies. For TDD testcases the simulation assumptions and alignment results for TM4 and TM9 test cases are provided in tables below:

Table 2: Simulation assumptions for 1024QAM test case in TM4 for TDD

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 (Note 1) | -6 (Note 1) |
|  |  | dB | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| ULDL config |  |  | 1 | 1 |
| Special subframe |  |  | 4 | 4 |
| Cell-specific reference signals |  |  | Antenna Ports 0,1,2,3 | Antenna Ports 0,1,2,3 |
| PDSCH transmission mode |  |  | 4 | 4 |
| PDSCH rank |  |  | 1 | 2 |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 | 50 |
| Precoding granularity |  | PRB | 50 | 50 |
| PMI delay |  | ms | 8 | 8 |
| Reporting interval |  | ms | 1 | 1 |
| Reporting mode |  |  | PUSCH 3-1 | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  | 0x000000000000FFFF | 0x00000000FFFF0000 |
| Propagation Condition |  |  | EPA5 | EPA5 |
| Correlation Matrix and Antenna Configuration |  |  | 4x2 Low | 4x4 Low |
| Note 1: . Note 2 : 50 resource blocks are allocated in sub-frames 4, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0, 1, 6. TBS in SF#0: 43816; TBS in SF#1,6: 39232; TBS in SF#4,9: 52752 |  |  |  |  |

Alignment results for TM4 test cases in TDD are provided below:

| Test 1 | Test 2 |
| --- | --- |

Figure 1: Results in TDD with 1024QAM-TM4

Observation #1: With no impairments for TM4 tests in TDD with 1024QAM 70% of max TP is achieved at 23.7 dB and 26.6dB for Test1 and Test2 respectively.

Table 3: Simulation assumptions for 1024QAM test case in TM9 for TDD

| Parameter |  | Unit | Test 3 | Test4 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | -3 | -3 |
| Beamforming model |  |  | Random beamforming | 2-layer precoding based on WB PMI feedback |
| ULDL config |  |  | 1 | 1 |
| Special subframe |  |  | 4 | 4 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 | Antenna ports 15,…,18 |
| Beamforming model |  |  | Annex B.4.1 | Annex B.4.3 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | 5 / 2 |
| CSI reference signal configuration |  |  | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 3) | OCNG (Note 3) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 | 50 |
| Simultaneous transmission |  |  | No | No |
| PDSCH transmission mode |  |  | 9 | 9 |
| PDSCH rank |  |  | 1 | 2 |
| Precoding granularity |  |  | 3 | 50 |
| PMI delay |  |  | 8 | 8 |
| Reporting interval |  |  | 1 | 1 |
| Reporting mode |  |  | PUSCH 3-1 | PUSCH 3-1 |
| alternativeCodeBookEnabledFor4TX-r12 |  |  | False | False |
| CodeBookSubsetRestriction bitmap |  |  | 0x000000000000FFFF | 0x00000000FFFF0000 |
| Propagation Condition |  |  | EPA5 | EPA5 |
| Correlation Matrix and Antenna Configuration |  |  | 4x2 Low | 4x4 Low |
| Note 1: . Note 2: 50 resource blocks are allocated in sub-frames 4, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0, 1, 6. TBS in SF#0: 43816; TBS in SF#1,6: 31704; TBS in SF#4,9: 52752 Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |

Alignment results for TM9 test cases in TDD are provided below

| Test 1 | Test 2 |
| --- | --- |

Figure 1: Results in TDD with 1024QAM-TM4

Observation #2: With no impairments for TM9 tests in TDD with 1024QAM 70% of max TP is achieved at 26.1 dB and 24.5dB for Test1 and Test2 respectively.

Proposal #1: For 1024QAM fading channel testcases in TDD align simulation assumptions and results with no impairments

# Conclusion

In this paper we provide impairment results for 1024QAM test cases in fading channel conditions for FDD. We also present alignment results for TDD test cases. Our results for FDD tests are summarized below:

Table 5: Impairment results for FDD test cases

| Test number | Bandwidth and MCS | Transmission Mode/ # MIMO Layers | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1 | 10 MHz 1024QAM | TM4; 1 Layer | EPA5 | 4x2 Low | 70 | 24.5 |
| 2 | 10MHz 1024QAM | TM4; 2 Layers | EPA5 | 4x4 Low | 70 | 27.1 |
| 3 | 10 MHz 1024QAM | TM9; 1 Layer | EPA5 | 4x2 Low | 70 | 30.3 |
| 4 | 10MHz 1024QAM | TM9; 2 Layers | EPA5 | 4x4 Low | 70 | 28.5 |

Our observations and proposals for TDD testcases:

Observation #1: With no impairments for TM4 tests in TDD with 1024QAM 70% of max TP is achieved at 23.7 dB and 26.6dB for Test1 and Test2 respectively.

Observation #2: With no impairments for TM9 tests in TDD with 1024QAM 70% of max TP is achieved at 26.1 dB and 24.5dB for Test1 and Test2 respectively.

Proposal #1: For 1024QAM fading channel testcases in TDD align simulation assumptions and results with no impairments

# References

- R4-1811378, “Way forward on demodulation and CSI requirements for high capacity stationary wireless link and 1024QAM”, Huawei, HiSilicon

# Appendix

## A.1 Simulation Assumptions for FDD Testcases

### CRS-Based

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 (Note 1) | -6 (Note 1) |
|  |  | dB | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cell-specific reference signals |  |  | Antenna Ports 0,1,2,3 | Antenna Ports 0,1,2,3 |
| PDSCH transmission mode |  |  | 4 | 4 |
| PDSCH rank |  |  | 1 | 2 |
| Precoding granularity |  | PRB | 50 | 50 |
| PMI delay |  | ms | 8 | 8 |
| Reporting interval |  | ms | 1 | 1 |
| Reporting mode |  |  | PUSCH 3-1 | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  | 0x000000000000FFFF | 0x00000000FFFF0000 |
| Note 1: . |  |  |  |  |

### DMRS-Based

| Parameter |  | Unit | Test 3 | Test4 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | -3 | -3 |
| Beamforming model |  |  | Random beamforming | 2 layer precoding based on WB PMI feedback |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 | Antenna ports 15,…,18 |
| Beamforming model |  |  | Annex B.4.1 | Annex B.4.3 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | 5 / 2 |
| CSI reference signal configuration |  |  | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 3) | OCNG (Note 3) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 | 50 |
| Simultaneous transmission |  |  | No | No |
| PDSCH transmission mode |  |  | 9 | 9 |
| PDSCH rank |  |  | 1 | 2 |
| Precoding granularity |  |  | 3 | 50 |
| PMI delay |  |  | 8 | 8 |
| Reporting interval |  |  | 1 | 1 |
| Reporting mode |  |  | PUSCH 3-1 | PUSCH 3-1 |
| alternativeCodeBookEnabledFor4TX-r12 |  |  | False | False |
| CodeBookSubsetRestriction bitmap |  |  | 0x000000000000FFFF | 0x00000000FFFF0000 |
| Note 1: . Note 2: 50 resource blocks are allocated in sub-frames 1,2,3,4,6,7,8,9 Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |

### FRC Tables

| Pararameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.1024-1.FDD | R.1024-2.FDD | R.1024-3.FDD | R.1024-4.FDD |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 | 50 | 50 |
| Allocated subframes per Radio Frame |  | 9 | 9 | 9 | 9 |
| Modulation |  | 1024QAM | 1024QAM | 1024QAM | 1024QAM |
| Target Coding Rate |  | 3/4 | 3/4 | 3/4 | 3/4 |
| Information Bit Payload |  |  |  |  |  |
| For Sub-Frames 1,4,6,9 | Bits | 52752 | 52752 | 52752 | 52752 |
| For Sub-Frames 2,3,7,8 | Bits | 52752 | 52752 | 52752 | 52752 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | N/A | N/A | N/A | N/A |
| Number of Code Blocks (Note 3) |  |  |  |  |  |
| For Sub-Frames 1,4,6,9 | Code blocks | 9 | 9 | 9 | 9 |
| For Sub-Frames 2,3,7,8 | Code blocks | 9 | 9 | 9 | 9 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | N/A | N/A | N/A | N/A |
| Binary Channel Bits |  |  |  |  |  |
| For Sub-Frames 1,6 | Bits | 68000 | 68000 | 66000 | 66000 |
| For Sub-Frames 4,9 | Bits | 68000 | 68000 | 66000 | 66000 |
| For Sub-Frames 2,7 | Bits | 68000 | 68000 | 64000 | 64000 |
| For Sub-Frames 3,8 | Bits | 68000 | 68000 | 66000 | 66000 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | N/A | N/A | N/A | N/A |
| Max. Throughput averaged over 1 frame (Note 4) | Mbps | 42.2016 | 42.2016 | 42.2016 | 42.2016 |
| UE Category |  | TBD | TBD | TBD | TBD |
| UE DL Category |  | 20, ≥ 22 | 20, ≥ 22 | 20, ≥ 22 | 20, ≥ 22 |
| Note 1: 1 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 4: Per transport block |  |  |  |  |  |

## A.2 Simulation Assumptions for TDD Testcases

### FRC Tables

| Pararameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.1024-1.TDD | R.1024-2.TDD | R.1024-3.TDD | R.1024-4.TDD |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 | 50 (Note 3) | 50 (Note 3) |
| Allocated subframes per Radio Frame |  | 9 | 9 | 9 | 9 |
| Modulation |  | 1024QAM | 1024QAM | 1024QAM | 1024QAM |
| Target Coding Rate |  | 3/4 | 3/4 | 3/4 | 3/4 |
| Information Bit Payload |  |  |  |  |  |
| For Sub-Frames 1,6 | Bits | 39232 | 39232 | 31704 | 31704 |
| For Sub-Frames 4,9 | Bits | 52752 | 52752 | 52752 | 52752 |
| For Sub-Frames 2,3,7,8 | Bits | N/A | N/A | N/A | N/A |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 43816 | 43816 | 43816 | 43816 |
| Number of Code Blocks (Note 4) |  |  |  |  |  |
| For Sub-Frames 1,6 | Code blocks | 7 | 7 | 6 | 6 |
| For Sub-Frames 4,9 | Code blocks | 9 | 9 | 9 | 9 |
| For Sub-Frames 2,3,7,8 | Code blocks | N/A | N/A | N/A | N/A |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 9 | 9 | 7 | 7 |
| Binary Channel Bits |  |  |  |  |  |
| For Sub-Frames 1,6 | Bits | 55280 | 55280 | 44280 | 44280 |
| For Sub-Frames 4,9 | Bits | 68000 | 68000 | 64000 | 64000 |
| For Sub-Frames 2,7,3,8 | Bits | N/A | N/A | N/A | N/A |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 65600 | 65600 | 54120 | 54120 |
| Max. Throughput averaged over 1 frame (Note 5) | Mbps | 22.7784 | 22.7784 | 21.2728 | 21.2728 |
| UE Category |  | TBD | TBD | TBD | TBD |
| UE DL Category |  | 20, ≥ 22 | 20, ≥ 22 | 20, ≥ 22 | 20, ≥ 22 |
| Note 1: 1 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: 50 resource blocks are allocated in sub-frames 4, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0, 1, 6. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: Per transport block |  |  |  |  |  |