3GPP TSG-RAN4 Meeting #88bis	R4-1812259

# Chengdu, China, 8th Oct – 12th Oct, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test cases for SA interruptions at SCell activation/deactivation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-9-6 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no test cases for SA interruptions at SCell activation/deactivation |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following test cases: 1. SA FR1 TDD interruptions at SCell activation and deactivation of known SCell with 15KHz SCS 2. SA FR1 FDD interruptions at SCell activation and deactivation of unknown SCell with 15KHz SCS 3. SA FR1 TDD interruptions at SCell activation and deactivation of known SCell with 30KHz SCS 4. SA FR1 FDD interruptions at SCell activation and deactivation of unknown SCell with 30KHz SCS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases for SA interruptions at SCell activation/deactivation will be missed in TS38.133. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section A.6.5.2.1.1, A.6.5.2.1.2, A.6.5.2.1.3 and A.6.5.2.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change #1>>

A.6.5.2.1.1	SA FR1 TDD interruptions at SCell activation and deactivation of known SCell with 15KHz SCS

A.6.5.2.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 8.3, when the SCell is known by the UE at the time of activation.

The test parameters are given in Tables A.6.5.2.1.1.1-1 and cell-specific parameters in A.6.5.2.1.1.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) but is not aware of Cell 2 on radio channel 2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2 (SCC). The UE now starts monitoring also the SCC. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in a slot #n, defines the start of time period T2. The UE shall be able to report valid CSI for the activated SCell at latest in slot (n+ [THARQ + Tactivation_time + TCSI_Reporting]). The UE shall start reporting CSI in slot (n+ THARQ + 3ms) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot (n+1+ THARQ) to (n+1+ THARQ +3ms + TSSB,max + TSMTC_duration).

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # m, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot (m+1+THARQ +3ms), and any PCell interruption due to the deactivation shall occur in the slot (m+1+ THARQ) to (m+1+ THARQ +3ms).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.6.5.2.1.1.1-1: General test parameters for SA FR1 TDD interruptions at SCell activation and deactivation of known SCell with 15KHz SCS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two NR RF channels |
| Active PCell |  | Cell1 | PCell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| CP length |  | Normal | Applicable to cell1 and cell 2 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| T1 | S | 7 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | S | 1 | During this time the UE shall activate the SCell. |
| T3 | S | 1 | During this time the UE shall deactivate the SCell. |

Table A.6.5.2.1.1.1-2: Cell specific test parameters for SA FR1 TDD interruptions at SCell activation and deactivation of known SCell with 15KHz SCS

| Parameter | Unit | Cell1 |  |  | Cell2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| RF Channel Number |  | 1 |  |  | 2 |  |  |
| Duplex mode |  | TDD |  |  | TDD |  |  |
| BWchannel | MHz | 10MHz |  |  | 10MHz |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote1 |  | SR.1.1 TDD |  |  | - |  |  |
| CORESET parametersNote1: |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
| OCNG PatternsNote1 |  | OP.1 |  |  | OP.1 |  |  |
| SMTC configuration |  | SMTC.1 FR1 |  |  | SMTC.1 FR1 |  |  |
| Note 2 | dBm/15 KHz | -104 |  |  | -104 |  |  |
|  | dB | 17 |  |  | 17 |  |  |
| Note 3 | dB | 17 |  |  | 17 |  |  |
| SS-RSRP Note 3 | dBm/15 KHz | -87 |  |  | -87 |  |  |
| SSB_RP Note 3 | dBm/15 KHz | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Time offset to cell1 Note 4 | s | - |  |  | 33 |  |  |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |  |  |

A.4.5.2.1.1.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in a slot (n+ THARQ + 3ms).

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot (n+ THARQ + Tactivation_time + TCSI_Reporting).

During T3 the UE shall stop sending CSI reports for SCell at latest in a slot (m+ THARQ + 3ms).

During T2 interruption of PCell during SCell activation shall not happen outside the slot (n+1+ THARQ) to (n+1+ THARQ +3ms + TSSB,max + TSMTC_duration).

During T3 interruption of PCell during SCell deactivation shall not happen outside the slot (m+1+ THARQ) to (m+1+ THARQ +3ms).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 8.2.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

A.6.5.2.1.2	SA FR1 FDD interruptions at SCell activation and deactivation of unknown SCell with 15KHz SCS

A.6.5.2.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 8.3, when the SCell is unknown by the UE at the time of activation.

The test parameters are given in Tables A.6.5.2.1.2.1-1 and cell-specific parameters in A.6.5.2.1.2.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two carriers, each with one cell. Cell1 have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) but is not aware of Cell 2 on radio channel 2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2 (SCC). The UE now starts monitoring also the SCC. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in a slot #n, defines the start of time period T2. Immediately at beginning of T2 the transmission power of cell 2 is increased to same level as for cell 1. The UE shall be able to report valid CSI for the activated SCell at latest in slot (n+ THARQ + Tactivation_time + TCSI_Reporting). The UE shall start reporting CSI in slot (n+ THARQ + 2*TSMTC_MAX +5ms) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot (n+1+ THARQ) to (n+1+ THARQ +5ms + 2*TSMTC_MAX + TSMTC_duration).

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # m, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot (m+1+THARQ +3ms), and any PCell interruption due to the deactivation shall occur in the slot (m+1+ THARQ) to (m+1+ THARQ +3ms).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.6.5.2.1.2.1-1: General test parameters for SA FR1 FDD interruptions at SCell activation and deactivation of unknown SCell with 15KHz SCS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two NR RF channels |
| Active PCell |  | Cell1 | PCell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| CP length |  | Normal | Applicable to cell1 and cell 2 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| T1 | s | 7 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |

Table A.6.5.2.1.2.1-2: Cell specific test parameters for SA FR1 FDD interruptions at SCell activation and deactivation of unknown SCell with 15KHz SCS

| Parameter | Unit | Cell1 |  |  | Cell2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| RF Channel Number |  | 1 |  |  | 2 |  |  |
| Duplex mode |  | FDD |  |  | FDD |  |  |
| BWchannel | MHz | 10MHz |  |  | 10MHz |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote1 |  | SR.1.1 FDD |  |  | - |  |  |
| CORESET parametersNote1: |  | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| OCNG PatternsNote1 |  | OP.1 |  |  | OP.1 |  |  |
| SMTC configuration |  | SMTC.1 FR1 |  |  | SMTC.1 FR1 |  |  |
| Note 2 | dBm/15 KHz | -104 |  |  | -104 |  |  |
|  | dB | 17 |  |  | 17 |  |  |
| Note 3 | dB | 17 |  |  | 17 |  |  |
| SS-RSRP Note 3 | dBm/15 KHz | -87 |  |  | -87 |  |  |
| SSB_RP Note 3 | dBm/15 KHz | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Time offset to cell1 Note 4 | s | - |  |  | 33 |  |  |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |  |  |

A.4.5.2.1.1.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in a slot (n+ THARQ + 2*TSMTC_MAX +5ms).

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot (n+ THARQ + Tactivation_time + TCSI_Reporting).

During T3 the UE shall stop sending CSI reports for SCell at latest in a slot (m+ THARQ + 3ms).

During T2 interruption of PCell during SCell activation shall not happen outside the slot (n+1+ THARQ) to (n+1+ THARQ +5ms + 2*TSMTC_MAX + TSMTC_duration).

During T3 interruption of PCell during SCell deactivation shall not happen outside the slot (m+1+ THARQ) to (m+1+ THARQ +3ms).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 8.2.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

A.6.5.2.1.3	SA FR1 TDD interruptions at SCell activation and deactivation of known SCell with 30KHz SCS

A.6.5.2.1.3.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 8.3, when the SCell is known by the UE at the time of activation.

The test parameters are given in Tables A.6.5.2.1.3.1-1 and cell-specific parameters in A.6.5.2.1.3.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) but is not aware of Cell 2 on radio channel 2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2 (SCC). The UE now starts monitoring also the SCC. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in a slot #n, defines the start of time period T2. The UE shall be able to report valid CSI for the activated SCell at latest in slot (n+ [THARQ + Tactivation_time + TCSI_Reporting]). The UE shall start reporting CSI in slot (n+ THARQ + 3ms) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot (n+1+ THARQ) to (n+1+ THARQ +3ms + TSSB,max + TSMTC_duration).

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # m, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot (m+1+THARQ +3ms), and any PCell interruption due to the deactivation shall occur in the slot (m+1+ THARQ) to (m+1+ THARQ +3ms).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.6.5.2.1.1.1-1: General test parameters for SA FR1 TDD interruptions at SCell activation and deactivation of known SCell with 30KHz SCS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two NR RF channels |
| Active PCell |  | Cell1 | PCell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| CP length |  | Normal | Applicable to cell1 and cell 2 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| T1 | s | 7 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |

Table A.6.5.2.1.1.1-2: Cell specific test parameters for SA FR1 TDD interruptions at SCell activation and deactivation of known SCell with 30KHz SCS

| Parameter | Unit | Cell1 |  |  | Cell2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| RF Channel Number |  | 1 |  |  | 2 |  |  |
| Duplex mode |  | TDD |  |  | TDD |  |  |
| BWchannel | MHz | 40MHz |  |  | 40MHz |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote1 |  | SR.2.1 TDD |  |  | - |  |  |
| CORESET parametersNote1: |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| OCNG PatternsNote1 |  | OP.1 |  |  | OP.1 |  |  |
| SMTC configuration |  | SMTC.2 FR1 |  |  | SMTC.2 FR1 |  |  |
| Note 2 | dBm/15 KHz | -104 |  |  | -104 |  |  |
|  | dB | 17 |  |  | 17 |  |  |
| Note 3 | dB | 17 |  |  | 17 |  |  |
| SS-RSRP Note 3 | dBm/15 KHz | -87 |  |  | -87 |  |  |
| SSB_RP Note 3 | dBm/15 KHz | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Time offset to cell1 Note 4 | s | - |  |  | 33 |  |  |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |  |  |

A.4.5.2.1.1.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in a slot (n+ THARQ + 3ms).

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot (n+ THARQ + Tactivation_time + TCSI_Reporting).

During T3 the UE shall stop sending CSI reports for SCell at latest in a slot (m+ THARQ + 3ms).

During T2 interruption of PCell during SCell activation shall not happen outside the slot (n+1+ THARQ) to (n+1+ THARQ +3ms + TSSB,max + TSMTC_duration).

During T3 interruption of PCell during SCell deactivation shall not happen outside the slot (m+1+ THARQ) to (m+1+ THARQ +3ms).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 8.2.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

A.6.5.2.1.4	SA FR1 FDD interruptions at SCell activation and deactivation of unknown SCell with 30KHz SCS

A.6.5.2.1.4.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 8.3, when the SCell is unknown by the UE at the time of activation.

The test parameters are given in Tables A.6.5.2.1.4.1-1 and cell-specific parameters in A.6.5.2.1.4.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two carriers, each with one cell. Cell1 have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) but is not aware of Cell 2 on radio channel 2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2 (SCC). The UE now starts monitoring also the SCC. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in a slot #n, defines the start of time period T2. Immediately at beginning of T2 the transmission power of cell 2 is increased to same level as for cell 1. The UE shall be able to report valid CSI for the activated SCell at latest in slot (n+ THARQ + Tactivation_time + TCSI_Reporting). The UE shall start reporting CSI in slot (n+ THARQ + 2*TSMTC_MAX +5ms) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot (n+1+ THARQ) to (n+1+ THARQ +5ms + 2*TSMTC_MAX + TSMTC_duration).

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # m, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot (m+1+THARQ +3ms), and any PCell interruption due to the deactivation shall occur in the slot (m+1+ THARQ) to (m+1+ THARQ +3ms).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.6.5.2.1.4.1-1: General test parameters for SA FR1 FDD interruptions at SCell activation and deactivation of unknown SCell with 30KHz SCS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two NR RF channels |
| Active PCell |  | Cell1 | PCell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| CP length |  | Normal | Applicable to cell1 and cell 2 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| T1 | s | 7 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |

Table A.6.5.2.1.4.1-2: Cell specific test parameters for SA FR1 FDD interruptions at SCell activation and deactivation of unknown SCell with 30KHz SCS

| Parameter | Unit | Cell1 |  |  | Cell2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| RF Channel Number |  | 1 |  |  | 2 |  |  |
| Duplex mode |  | FDD |  |  | FDD |  |  |
| BWchannel | MHz | 40MHz |  |  | 40MHz |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote1 |  | SR.2.1 FDD |  |  | - |  |  |
| CORESET parametersNote1: |  | CR.2.1 FDD |  |  | CR.2.1 FDD |  |  |
| OCNG PatternsNote1 |  | OP.1 |  |  | OP.1 |  |  |
| SMTC configuration |  | SMTC.2 FR1 |  |  | SMTC.2 FR1 |  |  |
| Note 2 | dBm/15 KHz | -104 |  |  | -104 |  |  |
|  | dB | 17 |  |  | 17 |  |  |
| Note 3 | dB | 17 |  |  | 17 |  |  |
| SS-RSRP Note 3 | dBm/15 KHz | -87 |  |  | -87 |  |  |
| SSB_RP Note 3 | dBm/15 KHz | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Time offset to cell1 Note 4 | s | - |  |  | 33 |  |  |
| Note 1: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |  |  |

A.4.5.2.1.1.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in a slot (n+ THARQ + 2*TSMTC_MAX +5ms).

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot (n+ THARQ + Tactivation_time + TCSI_Reporting).

During T3 the UE shall stop sending CSI reports for SCell at latest in a slot (m+ THARQ + 3ms).

During T2 interruption of PCell during SCell activation shall not happen outside the slot (n+1+ THARQ) to (n+1+ THARQ +5ms + 2*TSMTC_MAX + TSMTC_duration).

During T3 interruption of PCell during SCell deactivation shall not happen outside the slot (m+1+ THARQ) to (m+1+ THARQ +3ms).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 8.2.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

<< End of Change #1>>