3GPP TSG-RAN WG4 Meeting #88-Bis                                  R4-1812345

Chengdu, China, 08 - 12 October 2018

Agenda item:	7.13.2.4

Source: 	ZTE

Title: 	On NR BS PRACH performance requirements

Document for:	Discussion

Introduction

During RAN4#88 meeting in August two WFs [1] [2] relevant for NR PRACH performance requirements and simulation assumptions for alignment purpose have been approved. This contribution presents the simulation results.

Simulation parameters and simulation results

Here are the simulation parameters for FR1 and FR2:

| Simulation parameters | Value |
| --- | --- |
| SCS | - FR1: Format 0: 1.25 kHz - FR1: Formats A1/A2/A3/B4/C2: 15 & 30 kHz - FR2: Formats A1/A2/A3/B4/C2: 60 & 120 kHz |
| Tx | 1 |
| Rx | 2 |
| Channel condition | AWGN |
| Frequency offset | 0 |
| Phase noise | Not modelled |

The FR1&FR2 1T2R AWGN channel simulation results of SNR for missed detection probability < 1% and false alarm probability < 0.1% are summarized in the table below.

Table 1: PRACH simulation results for FR1 and FR2 (1T2R AWGN channel): SNR (dB)

|  | Format | SCS | Logical sequence index | Ncs | SNR [dB] (1T2R, AWGN channel) |
| --- | --- | --- | --- | --- | --- |
| FR1 | 0 | 1.25KHz | 22 | 13 (v=0) | -13.6 |
|  | A1 | 15KHz | 0 | 23 (v=0) | -8.7 |
|  |  | 30KHz | 0 | 23 (v=0) | -8.7 |
|  | A2 | 15KHz | 0 | 23 (v=0) | -11.8 |
|  |  | 30KHz | 0 | 23 (v=0) | -11.8 |
|  | A3 | 15KHz | 0 | 12 (v=0) | -13.5 |
|  |  | 30KHz | 0 | 12 (v=0) | -13.5 |
|  | B4 | 15KHz | 0 | 12 (v=0) | -16.5 |
|  |  | 30KHz | 0 | 12 (v=0) | -16.5 |
|  | C2 | 15KHz | 0 | 12 (v=0) | -11.7 |
|  |  | 30kHz | 0 | 12 (v=0) | -11.7 |
| FR2 | A1 | 60kHz | 0 | 0 /69(v=0) | -9.3 |
|  |  | 120kHz | 0 | 0 /69(v=0) | -8.2 |
|  | A2 | 60kHz | 0 | 0 /69(v=0) | -12.3 |
|  |  | 120kHz | 0 | 0 /69(v=0) | -11.2 |
|  | A3 | 60kHz | 0 | 0 (v=0) | -14.1 |
|  |  | 120kHz | 0 | 0 (v=0) | -13.1 |
|  | B4 | 60kHz | 0 | 0 (v=0) | -17.1 |
|  |  | 120kHz | 0 | 0 (v=0) | -16.1 |
|  | C2 | 60kHz | 0 | 0 (v=0) | -12.3 |
|  |  | 120kHz | 0 | 0 (v=0) | -11.2 |

Conclusions

The simulation results are summarized in Table 1.

# References

- R4-1811726 Approved Way forward on genera part of NR BS demodulation performance, August 2018.

- R4-1811728 Approved Way forward on NR PRACH demodulation performance, August 2018.