3GPP TSG-RAN4 Meeting #88bis	R4-1812529

# Chengdu, China, 8th Oct – 12th Oct, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS38.133 for BWP switching delay test case(section A4.5.6, A5.5.6, A6.5.6 and A7.5.6) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-9-21 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no test cases for BWP switching delay in EN-DC and SA |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add test cases: - E-UTRAN FDD – NR FR1 FDD DL active BWP switching of PSCell with non-DRX in asynchronous EN-DC - E-UTRAN TDD – NR FR1 TDD DL active BWP switching of PSCell with non-DRX in synchronous EN-DC - E-UTRAN TDD – NR FR2 TDD DL active BWP switching of PSCell with non-DRX in synchronous EN-DC - NR FR1 FDD DL active BWP switching of PCell with non-DRX in SA - NR FR1 TDD DL active BWP switching of PCell with non-DRX in SA - NR FR2 TDD DL active BWP switching of PCell with non-DRX in SA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases for BWP switching delay in EN-DC and SA are missed in TS38.133. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.5.6, A.5.5.6, A.6.5.6, A.7.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change #1>>

A.4.5.6	Active BWP switch delay

A.4.5.6.1	E-UTRAN FDD – NR FR1 FDD DL active BWP switch of PSCell with non-DRX in asynchronous EN-DC

A.4.5.6.1.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirements stated in section 8.6.

The test scenario comprises of one E-UTRA FDD PCell (Cell 1), one NR FDD PSCell (Cell 2) as given in Tables A.4.5.6.1.1-1. Cell-specific parameters of E-UTRA PCell are specified in Table A.3.7.2.1-1 and Cell-specific parameters of NR PSCell are specified in Table A.4.5.6.1.1-2 below, each with one cell.

PDCCHs indicating new transmissions shall be sent continuously on PSCell to ensure that the UE would have ACK/NACK sending, except for the time duration on Cell 2 where BWP switch occurs.

Before the test starts,

- UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (PSCell) on radio channel 2 (PSCC).

- UE is configured with 2 different UE-specific bandwidth parts, BWP-1 and BWP-2, in Cell 2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

- UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1

- UE is configured with a bwp-InactivityTimer timer value for PSCell.

All cells have constant signal levels throughout the test.

The test consists of 4 successive time periods, with durations of T1, T2, T3 and T4, respectively.

During T1,

The point in time at which the RRC message setting firstActiveDownlinkBWP-Id to BWP-2 and defaultDownlinkBWP-Id to BWP-2 is received at the UE side, in a slot # denoted i, defines the start of time period T1 by which the UE receives BWP switch request to BWP-2 in PSCell (Cell 2).

The UE shall be continuously scheduled on PSCell’s BWP-2.

The UE shall be able to receive PDSCH at slot (i+Y) as stated in section 8.6 and starts to report valid ACK/NACK for the PSCell at latest after slot (i+Y1).

During T2,

Time period T2 starts when a DCI format 1_1 command for PSCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in a slot # denoted j. The UE should switch its bandwidth part from BWP-2 to BWP-1.

The UE shall be continuously scheduled on PSCell’s BWP-1.

The UE shall be able to receive PDSCH at slot (i+Y) as stated in section 8.6 and starts to report valid ACK/NACK for the PSCell no later than slot (j+Y2).

During T3, the network won’t transmit DCI format for PDSCH reception on PSCell(Cell 2).

During T4,

The time period T4 starts immediately after the occurrence of the bwp-InactivityTimer timer expired, in a slot # denoted k. The UE should switch its bandwidth part from BWP1 back to the default bandwidth part – BWP-2.

During T4 the UE shall be continuously scheduled on PSCell’s BWP-2.

The UE shall be able to receive PDSCH at slot (i+Y) as stated in section 8.6 and starts to report valid ACK/NACK for the PSCell at latest after slot (k+Y2).

The test equipment verifies the DL BWP switch time by counting the slots from the time when RRC signalling is re-configuration, the BWP switch command is received or bwp-InactivityTimer timer expires until an ACK/NACK is received.

Table 4.5.6.1.1-1: General test parameters for FDD DL BWP switch in asynchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 2 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active PSCell |  | Cell 2 | PSCell on RF channel number 2. |
| E-UTRA parameters |  |  | Table A.3.7.2.1-1 |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| bwp-InactivityTimer | ms | [200] |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell2 timing offset to cell1 | s | 500 | Asynchronous EN-DC |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in 3GPP TS 38.104 | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | [0.2] |  |
| T2 | s | [0.2] |  |
| T3 | s | [0.2] |  |
| T4 | s | [0.2] |  |

Table 4.5.6.1.1-2: NR Cell specific test parameters for FDD DL BWP switch in asynchronous EN-DC

| Parameter | Unit | Cell 2 |
| --- | --- | --- |
| Connection Type |  | Conducted |
| Frequency Range |  | FR1 |
| BWchannel | MHz | 20 |
| Subcarrier Spacing | KHz | 15 |
| PDSCH parameters |  | SR.1.1 FDD |
| CORESET parameters |  | CR.1.1 FDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| BWP ID |  | 1, 2 |
| OCNG Patterns |  | OP.1 |
| SMTC Configuration |  | SMTC.1 FR1 |
| NocNote 2 | dBm/15 kHz | [-104] |
| SS-RSRP Note 3 | dBm/15 kHz | [-87] |
| Ês/Iot | dB | 17 |
| Ês/Noc | dB | 17 |
| Propagation Condition |  | AWGN |
| NBWP,1size Note 4 |  | TBD |
| NBWP,1start Note 4 |  | TBD |
| NBWP,2size Note 4 |  | TBD |
| NBWP,2start Note 4 |  | TBD |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T1. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For NBWP,isize and NBWP,istart definitions see section 4.4.4 and 4.4.5 in TS38.211[6] |  |  |

A.4.5.6.1.2	Test Requirements

During T1 the UE shall start to send the ACK for PSCell in a slot (i+Y1).

During T2 the UE shall start to send the ACK for PSCell in a slot (j+Y2).

During T4 the UE shall start to send the ACK for PSCell in a slot (k+Y2).

All of the above test requirements shall be fulfilled in order for the observed PSCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least [TBD]%.

NOTE:	During T1, T2, T4 if there are no uplink resources for reporting the ACK in a slot (i+Y1), (j+Y2), (k+Y2) then the UE shall use the next available uplink resource for reporting the corresponding ACK.

Editor’s note: FFS values of Y1, Y2 for type 1 UE type 2 UE.

A.4.5.6.2 E-UTRAN TDD – NR FR1 TDD DL active BWP switch of PSCell with non-DRX in synchronous EN-DC

A.4.5.6.2.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirements stated in section 8.6.

The test scenario comprises of one E-UTRA TDD PCell (Cell 1), one NR TDD PSCell (Cell 2) as given in Tables A.4.5.6.2.1-1. Cell-specific parameters of E-UTRA PCell are specified in Table A.3.7.2.1-1 and Cell-specific parameters of NR PSCell are specified in Table A.4.5.6.2.1-2 below, each with one cell.

PDCCHs indicating new transmissions shall be sent continuously on PSCell to ensure that the UE would have ACK/NACK sending, except for the time duration on Cell 2 where BWP switch occurs.

Before the test starts,

- UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (PSCell) on radio channel 2 (PSCC).

- UE is configured with 2 different UE-specific bandwidth parts, BWP-1 and BWP-2, in Cell 2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

- UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1

- UE is configured with a bwp-InactivityTimer timer value for PSCell.

All cells have constant signal levels throughout the test.

The test consists of 4 successive time periods, with durations of T1, T2, T3 and T4, respectively.

During T1,

The point in time at which the RRC message setting firstActiveDownlinkBWP-Id to BWP-2 and defaultDownlinkBWP-Id to BWP-2 is received at the UE side, in a slot # denoted i, defines the start of time period T1 by which the UE receives BWP switch request to BWP-2 in PSCell (Cell 2).

The UE shall be continuously scheduled on PSCell’s BWP-2.

The UE shall be able to receive PDSCH at slot (i+Y) as stated in section 8.6 and starts to report valid ACK/NACK for the PSCell at latest after slot (i+Y1).

During T2,

Time period T2 starts when a DCI format 1_1 command for PSCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in a slot # denoted j. The UE should switch its bandwidth part from BWP-2 to BWP-1.

The UE shall be continuously scheduled on PSCell’s BWP-1.

The UE shall be able to receive PDSCH at slot (i+Y) as stated in section 8.6 and starts to report valid ACK/NACK for the PSCell no later than slot (j+Y2).

During T3, the network won’t transmit DCI format for PDSCH reception on PSCell(Cell 2).

During T4,

The time period T4 starts immediately after the occurrence of the bwp-InactivityTimer timer expired, in a slot # denoted k. The UE should switch its bandwidth part from BWP-1 back to the default bandwidth part – BWP-2.

During T4 the UE shall be continuously scheduled on PSCell’s BWP-2.

The UE shall be able to receive PDSCH at slot (i+Y) as stated in section 8.6 and starts to report valid ACK/NACK for the PSCell at latest after slot (k+Y2).

The test equipment verifies the DL BWP switch time by counting the slots from the time when RRC signalling is re-configuration, the BWP switch command is received or bwp-InactivityTimer timer expires until an ACK/NACK is received.

Table 4.5.6.2.1-1: General test parameters for TDD DL BWP switch in synchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 2 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active PSCell |  | Cell 2 | PSCell on RF channel number 2. |
| E-UTRA parameters |  |  | Table A.3.7.2.1-1 |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| bwp-InactivityTimer | ms | [200] |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell2 timing offset to cell1 | s | 3 | Synchronous EN-DC |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in 3GPP TS 38.104 | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | [0.2] |  |
| T2 | s | [0.2] |  |
| T3 | s | [0.2] |  |
| T4 | s | [0.2] |  |

Table 4.5.6.2.1-2: NR Cell specific test parameters for TDD DL BWP switch in synchronous EN-DC

| Parameter | Unit | Cell 2 |
| --- | --- | --- |
| Connection Type |  | Conducted |
| Frequency Range |  | FR1 |
| BWchannel | MHz | 40 |
| Subcarrier Spacing | KHz | 30 |
| Uplink-Downlink Configuration |  | 3D1S1U |
| Flexible Slot ‘S’ Configuration |  | 9DL:3GP:2UL |
| PDSCH parameters |  | SR.2.1 TDD |
| CORESET parameters |  | CR.2.1 TDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| BWP ID |  | 1, 2 |
| OCNG Patterns |  | OP.1 |
| SMTC Configuration |  | SMTC.1 FR1 |
| NocNote 2 | dBm/15 kHz | [-104] |
| SS-RSRP Note 3 | dBm/15 kHz | [-87] |
| Ês/Iot | dB | 17 |
| Ês/Noc | dB | 17 |
| Propagation Condition |  | AWGN |
| NBWP,1size Note 4 |  | TBD |
| NBWP,1start Note 4 |  | TBD |
| NBWP,2size Note 4 |  | TBD |
| NBWP,2start Note 4 |  | TBD |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T1. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For NBWP,isize and NBWP,istart definitions see section 4.4.4 and 4.4.5 in TS38.211[6] |  |  |

A.4.5.6.2.2	Test Requirements

During T1 the UE shall start to send the ACK for PSCell in a slot (i+Y1).

During T2 the UE shall start to send the ACK for PSCell in a slot (j+Y2).

During T4 the UE shall start to send the ACK for PSCell in a slot (k+Y2).

All of the above test requirements shall be fulfilled in order for the observed PSCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least [TBD]%.

NOTE:	During T1, T2, T4 if there are no uplink resources for reporting the ACK in a slot (i+Y1), (j+Y2), (k+Y2) then the UE shall use the next available uplink resource for reporting the corresponding ACK.

Editor’s note: FFS values of Y1, Y2 for type 1 UE type 2 UE.

<< End of Change #1>>

<< Start of Change #2>>

A.5.5.6	Active BWP switch delay

A.5.5.6.1 E-UTRAN TDD – NR FR2 TDD DL active BWP switch of PSCell with non-DRX in synchronous EN-DC

A.5.5.6.1.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirements stated in section 8.6.

The test scenario comprises of one E-UTRA TDD PCell (Cell 1), one NR TDD PSCell (Cell 2) as given in Tables A.5.5.6.1.1-1. Cell-specific parameters of E-UTRA PCell are specified in Table A.3.7.2.2-1 and Cell-specific parameters of NR PSCell are specified in Table A.5.5.6.1.1-2 below, each with one cell.

PDCCHs indicating new transmissions shall be sent continuously on PSCell to ensure that the UE would have ACK/NACK sending, except for the time duration on Cell 2 where BWP switch occurs.

Before the test starts,

- UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (PSCell) on radio channel 2 (PSCC).

- UE is configured with 2 different UE-specific bandwidth parts, BWP-1 and BWP-2, in Cell 2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

- UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1

- UE is configured with a bwp-InactivityTimer timer value for PSCell.

All cells have constant signal levels throughout the test.

The test consists of 4 successive time periods, with durations of T1, T2, T3 and T4, respectively.

During T1,

The point in time at which the RRC message setting firstActiveDownlinkBWP-Id to BWP-2 and defaultDownlinkBWP-Id to BWP-2 is received at the UE side, in a slot # denoted i, defines the start of time period T1 by which the UE receives BWP switch request to BWP-2 in PSCell (Cell 2).

The UE shall be continuously scheduled on PSCell’s BWP-2.

The UE shall be able to receive PDSCH at slot (i+Y) as stated in section 8.6 and starts to report valid ACK/NACK for the PSCell at latest after slot (i+Y1).

During T2,

Time period T2 starts when a DCI format 1_1 command for PSCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in a slot # denoted j. The UE should switch its bandwidth part from BWP-2 to BWP-1.

The UE shall be continuously scheduled on PSCell’s BWP-1.

The UE shall be able to receive PDSCH at slot (i+Y) as stated in section 8.6 and starts to report valid ACK/NACK for the PSCell no later than slot (j+Y2).

During T3, the network won’t transmit DCI format for PDSCH reception on PSCell(Cell 2).

During T4,

The time period T4 starts immediately after the occurrence of the bwp-InactivityTimer timer expired, in a slot # denoted k. The UE should switch its bandwidth part from BWP-1 back to the default bandwidth part – BWP-2.

During T4 the UE shall be continuously scheduled on PSCell’s BWP-2.

The UE shall be able to receive PDSCH at slot (i+Y) as stated in section 8.6 and starts to report valid ACK/NACK for the PSCell at latest after slot (k+Y2).

The test equipment verifies the DL BWP switch time by counting the slots from the time when RRC signalling is re-configuration, the BWP switch command is received or bwp-InactivityTimer timer expires until an ACK/NACK is received.

Table 5.5.6.1.1-1: General test parameters for TDD DL BWP switch in synchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 2 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active PSCell |  | Cell 2 | PSCell on RF channel number 2. |
| E-UTRA parameters |  |  | Table A.3.7.2.2-1 |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| bwp-InactivityTimer | ms | [200] |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell2 timing offset to cell1 | s | 3 | Synchronous EN-DC |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in 3GPP TS 38.104 | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | [0.2] |  |
| T2 | s | [0.2] |  |
| T3 | s | [0.2] |  |

Table 5.5.6.1.1-2: NR Cell specific test parameters for TDD DL BWP switch in synchronous EN-DC

| Parameter | Unit | Cell 2 |
| --- | --- | --- |
| Connection Type |  | OTA |
| Frequency Range |  | FR2 |
| BWchannel | MHz | 100 |
| Subcarrier Spacing | KHz | 120 |
| Uplink-Downlink Configuration |  | 3D1S1U |
| Flexible Slot ‘S’ Configuration |  | 9DL:3GP:2UL |
| PDSCH parameters |  | SR.3.1 TDD |
| CORESET parameters |  | CR.3.1 TDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| BWP ID |  | 1, 2 |
| OCNG Patterns |  | OP.1 |
| SMTC Configuration |  | SMTC.1 FR2 |
| NocNote 2 | dBm/15 kHz | [-104] |
| SS-RSRP Note 3 | dBm/15 kHz | [-87] |
| Ês/Iot | dB | 17 |
| Ês/Noc | dB | 17 |
| Propagation Condition |  | AWGN |
| NBWP,1size Note 4 |  | TBD |
| NBWP,1start Note 4 |  | TBD |
| NBWP,2size Note 4 |  | TBD |
| NBWP,2start Note 4 |  | TBD |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T1. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For NBWP,isize and NBWP,istart definitions see section 4.4.4 and 4.4.5 in TS38.211[6] |  |  |

A.5.5.6.1.2	Test Requirements

During T1 the UE shall start to send the ACK for PSCell in a slot (i+Y1).

During T2 the UE shall start to send the ACK for PSCell in a slot (j+Y2).

During T4 the UE shall start to send the ACK for PSCell in a slot (k+Y2).

All of the above test requirements shall be fulfilled in order for the observed PSCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least [TBD]%.

NOTE:	During T1, T2, T4 if there are no uplink resources for reporting the ACK in a slot (i+Y1), (j+Y2), (k+Y2) then the UE shall use the next available uplink resource for reporting the corresponding ACK.

Editor’s note: FFS values of Y1, Y2 for type 1 UE type 2 UE.

<< End of Change #2>>

<< Start of Change #3>>

A.6.5.6	Active BWP switch delay

A.6.5.6.1 NR FR1 FDD DL active BWP switch of PCell with non-DRX in SA

A.6.5.6.1.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirements stated in section 8.6.

The test scenario comprises of one NR FDD PCell (Cell 1) as given in Tables A.6.5.6.1.1-1. Cell-specific parameters of NR PCell are specified in Table A.6.5.6.1.1-2 below with one cell.

PDCCHs indicating new transmissions shall be sent continuously on PCell to ensure that the UE would have ACK/NACK sending, except for the time duration on Cell 1 where BWP switch occurs.

Before the test starts,

- UE is connected to Cell 1 (PCell) on radio channel 1(PCC).

- UE is configured with 2 different UE-specific bandwidth parts, BWP-1 and BWP-2, in Cell 1 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

- UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1.

- UE is configured with a bwp-InactivityTimer timer value for PCell.

All cells have constant signal levels throughout the test.

The test consists of 4 successive time periods, with durations of T1, T2, T3 and T4, respectively.

During T1,

The point in time at which the RRC message setting firstActiveDownlinkBWP-Id to BWP-2 and defaultDownlinkBWP-Id to BWP-2 is received at the UE side, in a slot # denoted i, defines the start of time period T1 by which the UE receives BWP switch request to BWP-2 in PCell (Cell 1).

The UE shall be continuously scheduled on PCell’s BWP-2.

The UE shall be able to receive PDSCH at slot (i+Y) as stated in section 8.6 and starts to report valid ACK/NACK for the PCell at latest after slot (i+Y1).

During T2,

Time period T2 starts when a DCI format 1_1 command for PCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in a slot # denoted j. The UE should switch its bandwidth part from BWP-2 to BWP-1.

The UE shall be continuously scheduled on PCell’s BWP-1.

The UE shall be able to receive PDSCH at slot (i+Y) as stated in section 8.6 and starts to report valid ACK/NACK for the PCell no later than slot (j+Y2).

During T3, the network won’t transmit DCI format for PDSCH reception on PCell(Cell 1).

During T4,

The time period T4 starts immediately after the occurrence of the bwp-InactivityTimer timer expired, in a slot # denoted k. The UE should switch its bandwidth part from BWP-1 back to the default bandwidth part – BWP-2.

During T4 the UE shall be continuously scheduled on PCell’s BWP-2.

The UE shall be able to receive PDSCH at slot (i+Y) as stated in section 8.6 and starts to report valid ACK/NACK for the PSCell at latest after slot (k+Y2).

The test equipment verifies the DL BWP switch time by counting the slots from the time when RRC signalling is re-configuration, the BWP switch command is received or bwp-InactivityTimer timer expires until an ACK/NACK is received.

Table 6.5.6.1.1-1: General test parameters for FDD DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| bwp-InactivityTimer | ms | [200] |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on primary component carrier. |
| T1 | s | [0.2] |  |
| T2 | s | [0.2] |  |
| T3 | s | [0.2] |  |
| T4 | s | [0.2] |  |

Table 6.5.6.1.1-2: NR Cell specific test parameters for FDD DL BWP switch in SA

| Parameter | Unit | Cell 2 |
| --- | --- | --- |
| Connection Type |  | Conducted |
| Frequency Range |  | FR1 |
| BWchannel | MHz | 20 |
| Subcarrier Spacing | KHz | 15 |
| PDSCH parameters |  | SR.1.1 FDD |
| CORESET parameters |  | CR.1.1 FDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| BWP ID |  | 1, 2 |
| OCNG Patterns |  | OP.1 |
| SMTC Configuration |  | SMTC.1 FR1 |
| NocNote 2 | dBm/15 kHz | [-104] |
| SS-RSRP Note 3 | dBm/15 kHz | [-87] |
| Ês/Iot | dB | 17 |
| Ês/Noc | dB | 17 |
| Propagation Condition |  | AWGN |
| NBWP,1size Note 4 |  | TBD |
| NBWP,1start Note 4 |  | TBD |
| NBWP,2size Note 4 |  | TBD |
| NBWP,2start Note 4 |  | TBD |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T1. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For NBWP,isize and NBWP,istart definitions see section 4.4.4 and 4.4.5 in TS38.211[6] |  |  |

A.6.5.6.1.2	Test Requirements

During T1 the UE shall start to send the ACK for PCell in a slot (i+Y1).

During T2 the UE shall start to send the ACK for PCell in a slot (j+Y2).

During T4 the UE shall start to send the ACK for PCell in a slot (k+Y2).

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least [TBD]%.

NOTE:	During T1, T2, T4 if there are no uplink resources for reporting the ACK in a slot (i+Y1), (j+Y2), (k+Y2) then the UE shall use the next available uplink resource for reporting the corresponding ACK.

Editor’s note: FFS values of Y1, Y2 for type 1 UE type 2 UE.

A.6.5.6.2 NR FR1 TDD DL active BWP switch of PCell with non-DRX in SA

A.6.5.6.2.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirements stated in section 8.6.

The test scenario comprises of one NR TDD PCell (Cell 1) as given in Tables A.6.5.6.2.1-1. Cell-specific parameters of NR PCell are specified in Table A.6.5.6.2.1-2 below with one cell.

PDCCHs indicating new transmissions shall be sent continuously on PCell to ensure that the UE would have ACK/NACK sending, except for the time duration on Cell 1 where BWP switch occurs.

Before the test starts,

- UE is connected to Cell 1 (PCell) on radio channel 1(PCC).

- UE is configured with 2 different UE-specific bandwidth parts, BWP-1 and BWP-2, in Cell 1 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

- UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1.

- UE is configured with a bwp-InactivityTimer timer value for PCell.

All cells have constant signal levels throughout the test.

The test consists of 4 successive time periods, with durations of T1, T2, T3 and T4, respectively.

During T1,

The point in time at which the RRC message setting firstActiveDownlinkBWP-Id to BWP-2 and defaultDownlinkBWP-Id to BWP-2 is received at the UE side, in a slot # denoted i, defines the start of time period T1 by which the UE receives BWP switch request to BWP-2 in PCell (Cell 1).

The UE shall be continuously scheduled on PCell’s BWP-2.

The UE shall be able to receive PDSCH at slot (i+Y) as stated in section 8.6 and starts to report valid ACK/NACK for the PCell at latest after slot (i+Y1).

During T2,

Time period T2 starts when a DCI format 1_1 command for PCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in a slot # denoted j. The UE should switch its bandwidth part from BWP-2 to BWP-1.

The UE shall be continuously scheduled on PCell’s BWP-1.

The UE shall be able to receive PDSCH at slot (i+Y) as stated in section 8.6 and starts to report valid ACK/NACK for the PCell no later than slot (j+Y2).

During T3, the network won’t transmit DCI format for PDSCH reception on PCell(Cell 1).

During T4,

The time period T4 starts immediately after the occurrence of the bwp-InactivityTimer timer expired, in a slot # denoted k. The UE should switch its bandwidth part from BWP-1 back to the default bandwidth part – BWP-2.

During T4 the UE shall be continuously scheduled on PCell’s BWP-2.

The UE shall be able to receive PDSCH at slot (i+Y) as stated in section 8.6 and starts to report valid ACK/NACK for the PSCell at latest after slot (k+Y2).

The test equipment verifies the DL BWP switch time by counting the slots from the time when RRC signalling is re-configuration, the BWP switch command is received or bwp-InactivityTimer timer expires until an ACK/NACK is received.

Table 6.5.6.2.1-1: General test parameters for TDD DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| bwp-InactivityTimer | ms | [200] |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on primary component carrier. |
| T1 | s | [0.2] |  |
| T2 | s | [0.2] |  |
| T3 | s | [0.2] |  |
| T4 | s | [0.2] |  |

Table 6.5.6.2.1-2: NR Cell specific test parameters for TDD DL BWP switch in SA

| Parameter | Unit | Cell 2 |
| --- | --- | --- |
| Connection Type |  | Conducted |
| Frequency Range |  | FR1 |
| BWchannel | MHz | 40 |
| Subcarrier Spacing | KHz | 30 |
| Uplink-Downlink Configuration |  | 3D1S1U |
| Flexible Slot ‘S’ Configuration |  | 9DL:3GP:2UL |
| PDSCH parameters |  | SR.2.1 TDD |
| CORESET parameters |  | CR.2.1 TDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| OCNG Patterns |  | OP.1 |
| SMTC Configuration |  | SMTC.1 FR1 |
| NocNote 2 | dBm/15 kHz | [-104] |
| SS-RSRP Note 3 | dBm/15 kHz | [-87] |
| Ês/Iot | dB | 17 |
| Ês/Noc | dB | 17 |
| Propagation Condition |  | AWGN |
| NBWP,1size Note 4 |  | TBD |
| NBWP,1start Note 4 |  | TBD |
| NBWP,2size Note 4 |  | TBD |
| NBWP,2start Note 4 |  | TBD |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T1. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For NBWP,isize and NBWP,istart definitions see section 4.4.4 and 4.4.5 in TS38.211[6] |  |  |

A.6.5.6.2.2	Test Requirements

During T1 the UE shall start to send the ACK for PCell in a slot (i+Y1).

During T2 the UE shall start to send the ACK for PCell in a slot (j+Y2).

During T4 the UE shall start to send the ACK for PCell in a slot (k+Y2).

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least [TBD]%.

NOTE:	During T1, T2, T4 if there are no uplink resources for reporting the ACK in a slot (i+Y1), (j+Y2), (k+Y2) then the UE shall use the next available uplink resource for reporting the corresponding ACK.

Editor’s note: FFS values of Y1, Y2 for type 1 UE type 2 UE.

<< End of Change #3>>

<< Start of Change #4>>

A.7.5.6	Active BWP switch delay

A.7.5.6.1	NR FR2 TDD DL active BWP switch of PCell with non-DRX in SA

A.7.5.6.1.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirements stated in section 8.6.

The test scenario comprises of one NR TDD PCell (Cell 1) as given in Tables A.7.5.6.1.1-1. Cell-specific parameters of NR PCell are specified in Table A.7.5.6.1.1-2 below with one cell.

PDCCHs indicating new transmissions shall be sent continuously on PCell to ensure that the UE would have ACK/NACK sending, except for the time duration on Cell 1 where BWP switch occurs.

Before the test starts,

- UE is connected to Cell 1 (PCell) on radio channel 1(PCC).

- UE is configured with 2 different UE-specific bandwidth parts, BWP-1 and BWP-2, in Cell 1 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

- UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1.

- UE is configured with a bwp-InactivityTimer timer value for PCell.

All cells have constant signal levels throughout the test.

The test consists of 4 successive time periods, with durations of T1, T2, T3 and T4, respectively.

During T1,

The point in time at which the RRC message setting firstActiveDownlinkBWP-Id to BWP-2 and defaultDownlinkBWP-Id to BWP-2 is received at the UE side, in a slot # denoted i, defines the start of time period T1 by which the UE receives BWP switch request to BWP-2 in PCell (Cell 1).

The UE shall be continuously scheduled on PCell’s BWP-2.

The UE shall be able to receive PDSCH at slot (i+Y) as stated in section 8.6 and starts to report valid ACK/NACK for the PCell at latest after slot (i+Y1).

During T2,

Time period T2 starts when a DCI format 1_1 command for PCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in a slot # denoted j. The UE should switch its bandwidth part from BWP-2 to BWP-1.

The UE shall be continuously scheduled on PCell’s BWP-1.

The UE shall be able to receive PDSCH at slot (i+Y) as stated in section 8.6 and starts to report valid ACK/NACK for the PCell no later than slot (j+Y2).

During T3, the network won’t transmit DCI format for PDSCH reception on PCell(Cell 1).

During T4,

The time period T4 starts immediately after the occurrence of the bwp-InactivityTimer timer expired, in a slot # denoted k. The UE should switch its bandwidth part from BWP-1 back to the default bandwidth part – BWP-2.

During T4 the UE shall be continuously scheduled on PCell’s BWP-2.

The UE shall be able to receive PDSCH at slot (i+Y) as stated in section 8.6 and starts to report valid ACK/NACK for the PSCell at latest after slot (k+Y2).

The test equipment verifies the DL BWP switch time by counting the slots from the time when RRC signalling is re-configuration, the BWP switch command is received or bwp-InactivityTimer timer expires until an ACK/NACK is received.

Table 7.5.6.1.1-1: General test parameters for TDD DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| bwp-InactivityTimer | ms | [200] |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on primary component carrier. |
| T1 | s | [0.2] |  |
| T2 | s | [0.2] |  |
| T3 | s | [0.2] |  |
| T4 | s | [0.2] |  |

Table 7.5.6.1.1-2: NR Cell specific test parameters for TDD DL BWP switch in SA

| Parameter | Unit | Cell 2 |
| --- | --- | --- |
| Connection Type |  | OTA |
| Frequency Range |  | FR2 |
| BWchannel | MHz | 100 |
| Subcarrier Spacing | KHz | 120 |
| Uplink-Downlink Configuration |  | 3D1S1U |
| Flexible Slot ‘S’ Configuration |  | 9DL:3GP:2UL |
| PDSCH parameters |  | SR.3.1 TDD |
| CORESET parameters |  | CR.3.1 TDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| OCNG Patterns |  | OP.1 |
| SMTC Configuration |  | SMTC.1 FR2 |
| NocNote 2 | dBm/15 kHz | [-104] |
| SS-RSRP Note 3 | dBm/15 kHz | [-87] |
| Ês/Iot | dB | 17 |
| Ês/Noc | dB | 17 |
| Propagation Condition |  | AWGN |
| NBWP,1size Note 4 |  | TBD |
| NBWP,1start Note 4 |  | TBD |
| NBWP,2size Note 4 |  | TBD |
| NBWP,2start Note 4 |  | TBD |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T1. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For NBWP,isize and NBWP,istart definitions see section 4.4.4 and 4.4.5 in TS38.211[6] |  |  |

A.7.5.6.2.2	Test Requirements

During T1 the UE shall start to send the ACK for PCell in a slot (i+Y1).

During T2 the UE shall start to send the ACK for PCell in a slot (j+Y2).

During T4 the UE shall start to send the ACK for PCell in a slot (k+Y2).

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least [TBD]%.

NOTE:	During T1, T2, T4 if there are no uplink resources for reporting the ACK in a slot (i+Y1), (j+Y2), (k+Y2) then the UE shall use the next available uplink resource for reporting the corresponding ACK.

Editor’s note: FFS values of Y1, Y2 for type 1 UE type 2 UE.

<< End of Change #4>>