3GPP TSG-RAN WG4 Meeting #88bis	R4-1812545

Chengdu, China, 8th - 12th Oct, 2018

Agenda item:	7.9.4.3.3.1

Source: 	ZTE Corporation .

Title: 	TP to TS 38.141-2:  transient period (6.5.2)

Document for:	Approval

Introduction

This contribution is a text proposal to draft TS 38.141-2 [1] to complete transient period conformance testing.

Text Proposal

--------------Start of text proposal-------------

6.5.2	OTA transmitter transient period

6.5.2.1	Definition and applicability

The OTA transmitter transient period requirements apply only to TDD operation of BS.

The OTA transmitter transient period is the time period during which the transmitter unit is changing from the OFF period to the ON period or vice versa. The OTA transmitter transient period is illustrated in figure 6.5.2.1-1.

Transmitter output power

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transmitter transient

period

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 6.5.2.1-1: Illustration of the relations of transmitter ON period,
transmitter OFF period and transmitter transient period

This requirement applies at each co-location test antenna conducted output supporting reception in the operating band for BS type 1-O, and at each RIB supporting transmission in the operating band for BS type 2-O.

6.5.2.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], subclause 9.5.3.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], subclause 9.5.3.3.

6.5.2.3	Test purpose

The purpose of this test is to verify the OTA transmitter transient periods are within the limits of the minimum requirements.

6.5.2.4	Method of test

6.5.2.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested: M; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested:

-	MRFBW in single band operation, see subclause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation; see subclause 4.9.1.

6.5.2.4.2	Procedure

6.5.2.4.2.1	General procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (Dx.y) of the BS with the test system.

3)	Set the BS in the direction of the declared beam peak direction of the beam direction pair, for the beam to be tested.

6.5.2.4.2.2	BS type 1-O

4)	Place the co-location test antenna as specified in subclause 4.12.

5)	Configure the beam peak direction of the BS according to the declared beam direction pair.

6)	Set the BS to transmit according to the applicable test configuration in subclause 4.8 using the corresponding test models or set of physical channels in subclause 4.9.

7)	Measure the mean power spectrum density measured over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the BS centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

8)	For an BS supporting contiguous CA, measure the mean power spectral density over 70/N μs filtered with a square filter of bandwidth equal to the Aggregated Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs.N = SCS/15, where SCS is the smallest Sub Carrier Spacing in kHz used in the Aggregated BS Channel Bandwidth.

In addition, for a multi-band RIB, the following steps shall apply:

9)	For a multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

6.5.2.4.2.3	BS type 2-O

4)	Place the measurement antenna at the transmitter reference direction

5)	Configure the beam peak direction of the NR BS according to the declared beam direction pair.

6)	Set the NR BS to transmit according to the applicable test configuration in subclause 4.8 using the corresponding test models or set of physical channels in subclause 4.9.

In addition, for an NR BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in subclause 4.7.3.

7)	Measure EIRP sample when transmitter is ON at maximum rated power, call this P_EIRP_ON by either a) or b) below. EIRP is measured over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the NR BS centred on the central frequency of the RF bandwidth. From maximum transmit power measurement also corresponding TRP, P_TRP_ON is known.

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the NR BS.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

8)	Measure EIRP sample when transmitter is OFF, call this P_EIRP_OFF by either a) or b) above. EIRP is measured over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the NR BS centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 3μs to 35/N μs before start of next transmitter ON period - 3μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz. From maximum transmit power measurement also corresponding TRP, P_TRP_ON is known.

9)	For an NR BS supporting contiguous CA, measure EIRP by either a) or b) above. EIRP is measured over 70/N μs filtered with a square filter of bandwidth equal to the Aggregated Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 3 μs to 35/N μs before start of next transmitter ON period - 3 μs. N = SCS/15, where SCS is the smallest Sub Carrier Spacing in kHz used in the Aggregated BS Channel Bandwidth, call this P_EIRP_OFF.

10)	 Calculate the difference betweenTRP when transmitter is on and off, call this  Delta_TRP=P_TRP_ON_declare+36dBm/MHz, where P_TRP_ON_declare is declared TRP by vendors and expressed in the unit of dBm/MHz

11)	 Calculate the required EIRP off power as  P_EIRP_ON_declare-Delta_TRP where P_EIRP_ON_declare is declared at the conformance testing direction by vendors and expressed in the unit of dBm/MHz.

6.5.2.5	Test requirements

6.5.2.5.1	BS type 1-O

The measured TRP according to subclause 6.5.2.4.2.2 shall be less than -102.6 dBm/MHz for carrier frequency f ≤ 3.0 GHz.

The measured TRP according to subclause 6.5.2.4.2.2 shall be less than -102.4 dBm/MHz for carrier frequency 3.0 GHz < f ≤ 6.0 GHz.

For multi-band connector, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.1.

6.5.2.5.2	BS type 2-O

[The measured EIRP OFF power P_EIRP_OFF  according to subclause 6.5.2.4.2.3 shall be less than [ P_EIRP_ON_declare-Delta_TRP+XX] dBm/MHz for carrier frequency f ≤ XX GHz.

The measured EIRP OFF power  P_EIRP_OFF according to subclause 6.5.2.4.2.3 shall be less than [ P_EIRP_ON_declare-Delta_TRP+XX] dBm/MHz for carrier frequency XX GHz < f ≤ XX GHz.

The measured EIRP OFF power  P_EIRP_OFF according to subclause 6.5.2.4.2.3 shall be less than [ P_EIRP_ON_declare-Delta_TRP+XX] dBm/MHz for carrier frequency XX GHz < f ≤ XX GHz.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in subclause 4.1.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.1.]

--------------End of text proposal-------------

- Reference

- RP-181664, “TS 38.141-2 v1.0.0”, Huawei,approved.

- R4-18xxx, Discussion on FR2 transient period, ZTE