3GPP TSG-RAN WG4 Meeting #88bis	R4-1812747

Chengdu, China, 8 – 12 October 2018

Source:	Ericsson

Title:	TP to TS 38.141-2: Section 4.9.3.2.2 NR test model 2

Agenda item:	7.9.4.2.2

Document for:	Approval

# 1	Introduction

The following has been taken from the chairman’s notes during RAN4 #88 meeting:

|  | TDD configuration | Power boosting | Mixed numerologies | PDCCH layout | Others |
| --- | --- | --- | --- | --- | --- |
| Agreement | Option 1: TDD configuration 3 Special subframe 8 for NR. TDD configuration 1 special subframe 7 for MSR Option 2: TDD configuration 2 special subframe 7 for NR; TDD configuration 1special subframe 7 for MSR. Option 3: TDD configuration 1 and special subframe configuration 7 for NR and MSR => Option 1: TDD configuration 1 special subframe 7 for MSR Option 2: TDD configuration 2 special subframe 7 for NR Agreement: TDD configuration 2 special subframe 7 for NR for FR1 and FR2 TDD configuration 1 special subframe 7 for MSR in FR1 2 frames for TDD for conformance testing | ACLR and unwanted emission, EVM for FR1: Power boosting with scalable test model design. For power boosting/deboosting, [xth] PRB will be boosted/deboosted Detailed test model is FFS For FR2: No boosting | No mixed numerolgoies | Option 1: 1 symbol Option 2: 2 symbols FFS on CCE configuration. | FFS on if SSB is needed for test model |

The current text outline for NR test models has still two options open for PDCCH layout.  As the tentative agreement for PDCCH 1 CCE configuration was implemented in NR test model 1, this contribution continues to implement the same principle for remaining NR test models.

# 2	References

- R4-1811902, “TS 38.141-1 v0.4.0”, Huawei

- R4-1810616, “Physical Channel Parameters for NR Test Model Design in FR2”, Ericsson

[Start of Text Proposal]

##### 4.9.3.2.1	NR test model 1 for FR2 (NR-FR2-TM1)

This model shall be used for tests on:

-	BS output power

-	Unwanted emissions

-	Occupied bandwidth

-	ACLR

-	Operating band unwanted emissions

-	Transmitter spurious emissions

Table 4.9.3.2.1-1: Physical channel parameters of NR-FR2-TM1

| Parameter |  |
| --- | --- |
| PDCCH |  |
| # of symbols used for control channel | 1 |
| # of CCEs allocated to PDCCH | 1 |
| # of available REGs | 6 |
| Aggregation level(s) | 1 |
| # of RBs not allocated by PDCCH in the first symbol (# of RBs for PDSCH) | NRB – 6 |
| DM-RS configuration and density | Comb structure with same frequency density of ¼ (i.e., every 4th subcarrier) on all REGs |
| PDSCH |  |
| DM-RS configuration and density | Type 1, Comb2 (every other subcarrier) in symbols 3 |
| PT-RS pattern and density | Distributed pattern, every symbol for every second PRB (time density 1 and frequency density ½) |
| # of QPSK PDSCH PRBs which are boosted or deboosted | 0 |

Where NRB is considering the maximum transmission bandwidth configuration seen in table 5.3.2-2.

##### 4.9.3.2.2	NR test model 2 for FR2 (NR-FR2-TM2)

This model shall be used for tests on:

-	Total power dynamic range (lower OFDM symbol power limit at min power),

-	EVM of single 64QAM PRB allocation (at min power)

-	Frequency error (at min power)

Note: EVM shall be tested on highest modulation without power back off

Table 4.9.3.2.2-1: Physical channel parameters of NR-FR2-TM2

| Parameter |  |
| --- | --- |
| PDCCH |  |
| # of symbols used for control channel | 1 |
| # of CCEs allocated to PDCCH | 1 |
| # of available REGs | 6 |
| Aggregation level(s) | 1 |
| # of RBs not allocated by PDCCH in the first symbol (# of RBs for PDSCH) | 1 |
| # of <NIL> RBs added for padding | NRB - 5 |
| DM-RS configuration and density | Comb structure with same frequency density of ¼ (i.e., every 4th subcarrier) on all REGs |
| PDSCH |  |
| # of 64 QAM PDSCH PRBs within a slot for EVM measured | 1 |
| # of PDSCH PRBs which are not allocated | NRB - 1 |
| DM-RS configuration and density | Type 1, Comb2 (every other subcarrier) in symbols 3 |
| PT-RS pattern and density | Distributed pattern, every symbol for every second PRB (time density 1 and frequency density ½) |
| # of QPSK PDSCH PRBs which are boosted or deboosted | 0 |

Where NRB is considering the maximum transmission bandwidth configuration seen in table 5.3.2-2.

#### 4.9.3.2.3	NR test model 3.1 for FR2 (NR-FR2-TM3.1)

This model shall be used for tests on:

-	Output power dynamics

-	Total power dynamic range (upper OFDM symbol power limit at max power with all 64QAM PRBs allocated)

-	Transmitted signal quality

-	Frequency error

-	EVM for 64QAM modulation (or highest modulation without power back off)

Note: EVM shall be tested on highest modulation without power back off

Physical channel parameters are defined in Table 4.9.3.2.2-1 with all allocated PRBs to be at max power.

[End of Text Proposal]