3GPP TSG-RAN WG4 Meeting #88bis	R4-1812842

Chengdu, China, 8 – 12 October 2018

Agenda Item:	7.11.8.1

Source:	Ericsson

Title:	L1-RSRP measurement period for SSB based new beam identification

Document for:	Discussion

# 1	Introduction

In RAN4#88, it was discussed the L1-RSRP measurement period for new beam identification. In this contribution we show SSB based L1-RSRP measurement simulation result and discuss the measurement period.

# 2	Discussion

## 2.1	SSB based measurement

Table 1 is the simulation parameters for SSB based L1-RSRP measurement accuracy, and Figure 1 summaries the simulation results. Like CSI-RS based measurement accuracy, the accuracy with 1 sample is more than 1dB especially in FR1. Since the beam detection threshold rsrp-ThresholdSSB is common for both CSI-RS based and SSB based, and resolution of the threshold is 1dB, we think 3 samples are good candidate for SSB based new beam identification.

Proposal 1: Use 3 samples for CSI-RS based new beam identification.

Table 1	Simulation parameters for SSB based L1-RSRP measurement accuracy

| Parameters | Value |
| --- | --- |
| SCS | 15kHz, 30kHz, 120kHz |
| Side condition (SNR) | -3dB |
| Number of samples | 1, 3, 5 |

Figure 1	SSB based L1-RSRP measurement accuracy.

# 3	Conclusion

Proposal 1: Use 3 samples for SSB based new beam identification.

# References

- R4-1812841, “Side condition of new beam identification”, Ericsson.