3GPP TSG-RAN WG4 Meeting #88-Bis	R4-1812922

Chengdu, China, 8 – 12 October 2018

Agenda Item:	7.9.4.3.7

Source: 	Nokia, Nokia Shanghai Bell

Title: 	TP to TS 38.141-2 on CLTA related MU

Document for:	Approval

# 1.	Introduction

This contribution provides a TP to TS 38.141-2 clarifying the CLTA related MU.

# 2.	Text proposal

<Start of change>

#### 4.1.2.2	Measurement of transmitter

Table 4.1.2.2-1: Maximum OTA Test System uncertainty for FR1 OTA transmitter tests

| Subclause | Maximum OTA Test System uncertainty | Derivation of OTA Test System uncertainty |
| --- | --- | --- |
| 6.2 Radiated transmit power | Normal condition: ±1.1 dB, f ≤ 3 GHz ±1.3 dB, 3 GHz < f ≤ 6 GHz | For FR1 OTA Test System uncertainty derivation for Normal conditions, see TR 37.842 [6], subclause 10.3.2.2. |
|  | Extreme condition: FFS |  |
| 6.3 OTA base station output power | ±1.4 dB, f ≤ 3.0 GHz ±1.5 dB, 3.0 GHz < f ≤ 4.2 GHz ±1.5, 4.2 GHz < f ≤ 6.0 GHz | FFS |
| 6.4.2 OTA RE power control dynamic range | N/A | FFS |
| 6.4.3 OTA total power dynamic range | ±0.4 dB | FFS |
| 6.5.2 OTA transmitter OFF power | FFS (NOTE 1) | FFS |
| 6.5.3 OTA transmitter transient period | N/A | FFS |
| 6.6.1 OTA frequency error | ±12 Hz | FFS |
| 6.6.2 OTA modulation quality | ±1 % | FFS |
| 6.6.3 OTA time alignment error | ±25 ns | FFS |
| 6.7.2 OTA occupied bandwidth | 5 MHz, 10 MHz BS Channel BW: ±100 kHz 15 MHz, 20 MHz, 25 MHz, 30 MHz, 40 MHz, 50 MHz BS Channel BW: ±300 kHz 60 MHz, 70 MHz, 80 MHz, 90 MHz, 100 MHz BS Channel BW: ±600 kHz | FFS |
| 6.7.3 OTA ACLR | ACLR/CACLR f ≤ 3.0 GHz BW ≤ 20MHz: ±1 dB BW > 20MHz: ±1 dB 3.0 GHz < f ≤ 4.2 GHz BW ≤ 20MHz: ±1.2 dB BW > 20MHz: ±1.2 dB 4.2 GHz < f ≤ 6.0 GHz BW ≤ 20MHz: ±1.2 dB BW > 20MHz: ±1.2 dB Absolute power ±2.2 dB, f ≤ 3.0 GHz Absolute power ±2.7 dB, 3.0 GHz < f ≤ 4.2 GHz Absolute power ±2.7 dB, 4.2 GHz < f ≤ 6.0 GHz | FFS |
| 6.7.4 OTA operating band unwanted emissions | Absolute power ±1.8 dB, f ≤ 3.0 GHz Absolute power ±2 dB, 3.0 GHz < f ≤ 4.2 GHz Absolute power ±2 dB, 4.2 GHz < f ≤ 6.0 GHz | FFS |
| 6.7.5.2.2 OTA transmitter spurious emissions, mandatory requirements | ±2.3 dB, 30 MHz < f ≤ 6 GHz ±4.2 dB, 6 GHz < f ≤ 19 GHz |  |
| 6.7.5.2.3 OTA transmitter spurious emissions, protection of BS receiver | ±3.1 dB, f ≤ 3 GHz ±3.3 dB, 3 GHz < f ≤ 4.2 GHz ±3.4, 4.2 GHz < f ≤ 6 GHz |  |
| 6.7.5.2.4 OTA transmitter spurious emissions, additional spurious emissions requirements | ±2.6 dB, f ≤ 3 GHz ±3.0, 3 GHz < f ≤ 4.2 GHz ±3.5, 4.2 GHz < f ≤ 6 GHz |  |
| 6.7.5.2.5 OTA transmitter spurious emissions, co-location | ±3.1 dB, f ≤ 3 GHz ±3.3 dB, 3 GHz < f ≤ 4.2 GHz ±3.4, 4.2 GHz < f ≤ 6 GHz (NOTE 1) |  |
| 6.8 OTA transmitter intermodulation | FFS (NOTE 1) | FFS |
| NOTE 1: Fulfilling the criteria for CLTA selection and placement in subclause 4.15 is deemed sufficient for the test purposes. When these criteria are met, the measurement uncertainty related to the selection of the co-location test antenna and its alignment as specified in the appropriate table in TR 37.843 [28], subclause 10.6 shall be used for evaluating the test system uncertainty. |  |  |

Table 4.1.2.2-2: Maximum OTA Test System uncertainty for FR2 OTA transmitter tests

| Subclause | Maximum OTA Test System uncertainty | Derivation of OTA Test System uncertainty |
| --- | --- | --- |
| 6.2 Radiated transmit power | ±1.7 dB (24.25 - 29.25 GHz) ±2.0 dB (37 – 40 GHz) | FFS |
| 6.3 OTA base station output power | ±2.1 dB (24.25 - 29.25 GHz) ±2.4 dB (37 – 40 GHz) | FFS |
| 6.4.2 OTA RE power control dynamic range | N/A | FFS |
| 6.4.3 OTA total power dynamic range | ±0.4 dB | FFS |
| 6.5.2 OTA transmitter OFF power | FFS | FFS |
| 6.5.3 OTA transmitter transient period | N/A | FFS |
| 6.6.1 OTA frequency error | ±12 Hz | FFS |
| 6.6.2 OTA modulation quality | 1% | FFS |
| 6.6.3 OTA time alignment error | ±25 ns | FFS |
| 6.7.2 OTA occupied bandwidth | [600] kHz | FFS |
| 6.7.3 OTA ACLR | Relative ACLR: ±2.3dB (24.25-29.25GHz) ±2.6dB (37-40GHz) Absolute ACLR: ±2.7dB | FFS |
| 6.7.4 OTA operating band unwanted emissions | ±2.7dB | FFS |
| 6.7.5.3.2 OTA transmitter spurious emissions, mandatory requirements | FFS | FFS |
| 6.7.5.3.3 OTA transmitter spurious emissions, additional spurious emissions requirements | FFS | FFS |

#### 4.1.2.3	Measurement of receiver

Table 4.1.2.3-1: Maximum OTA Test System uncertainty for FR1 OTA receiver tests

| Subclause | Maximum OTA Test System uncertainty | Derivation of OTA Test System uncertainty |
| --- | --- | --- |
| 7.2 OTA sensitivity | ±1.3 dB, f ≤ 3.0 GHz ±1.4 dB, 3.0 GHz < f ≤ 4.2 GHz ±1.6 dB, 4.2 GHz < f ≤ 6.0 GHz | See 3GPP TR 37.842 [6], subclause 10.3.2.2. |
| 7.3 OTA reference sensitivity level | ±1.3 dB, f ≤ 3.0 GHz ±1.4 dB, 3.0 GHz < f ≤ 4.2 GHz ±1.6 dB, 4.2 GHz < f ≤ 6.0 GHz | See 3GPP TR 37.843 [16], subclause 10.3.3. |
| 7.4 OTA dynamic range | ±0.3 dB | See 3GPP TR 37.843 [16], subclause 10.3.4. |
| 7.5.1 OTA adjacent channel selectivity | ±1.7 dB, f ≤ 3.0 GHz ±2.1 dB, 3.0 GHz < f ≤ 4.2 GHz ±2.4 dB, 4.2 GHz < f ≤ 6.0 GHz | See 3GPP TR 37.843 [16], subclause 10.3.5. |
| 7.5.2 In-band blocking (General) | ±1.9 dB, f ≤ 3.0 GHz ±2.2 dB, 3.0 GHz < f ≤ 4.2 GHz ±2.5 dB, 4.2 GHz < f ≤ 6.0 GHz | See 3GPP TR 37.843 [16], subclause 10.7. |
| 7.5.2 In-band blocking (Narrowband) | ±1.7 dB, f ≤ 3.0 GHz ±2.1 dB, 3.0 GHz < f ≤ 4.2 GHz ±2.4 dB, 4.2 GHz < f ≤ 6.0 GHz | See 3GPP TR 37.843 [16], subclause 10.7. |
| 7.6 OTA out-of-band blocking | fwanted ≤ 3 GHz 1 MHz < finterferer ≤ 3 GHz: ±2.0 dB 3 GHz < finterferer ≤ 6 GHz: ±2.1 dB 6 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB 3 GHz < fwanted ≤ 4.2GHz: 1 MHz < finterferer ≤ 3 GHz: ±2.0 dB 3 GHz < finterferer ≤ 6 GHz: ±2.1 dB 6 GHz < finterferer ≤ 12.75 GHz: ±3.6 dB 4.2 GHz < fwanted ≤ 6 GHz: 1 MHz < finterferer ≤ 3 GHz: ±2.2 dB 3 GHz < finterferer ≤ 6 GHz: ±2.3 dB 6 GHz < finterferer ≤ 12.75 GHz: ±3.6 dB | See 3GPP TR 37.843 [16], subclause 10.8. |
| 7.7 OTA receiver spurious emissions | 30 MHz ≤ f ≤ 6 GHz, ±2.5dB 6 GHz < f ≤ 19 GHz, ±4.2dB | See 3GPP TR 37.843 [16], subclause 10.5. |
| 7.8 OTA receiver intermodulation | ±2.0 dB, f ≤ 3.0 GHz ±2.6 dB, 3.0 GHz < f ≤ 4.2 GHz ±3.2 dB, 4.2 GHz < f ≤ 6.0 GHz | See 3GPP TR 37.843 [16], subclause 10.3.6. |
| 7.9 OTA in-channel selectivity | ±1.7 dB, f ≤ 3.0 GHz ±2.1 dB, 3.0 GHz < f ≤ 4.2 GHz ±2.4 dB, 4.2 GHz < f ≤ 6.0 GHz | See 3GPP TR 37.843 [16], subclause 10.3.7. |
| NOTE 1: Fulfilling the criteria for CLTA selection and placement in subclause 4.15 is deemed sufficient for the test purposes. When these criteria are met, the measurement uncertainty related to the selection of the co-location test antenna and its alignment as specified in the appropriate table in TR 37.843 [28], subclause 10.6 shall be used for evaluating the test system uncertainty. |  |  |

Table 4.1.2.3-2: Maximum OTA Test System uncertainty for FR2 OTA receiver tests

| Subclause | Maximum OTA Test System uncertainty | Derivation of OTA Test System uncertainty |
| --- | --- | --- |
| 7.3 OTA reference sensitivity level | ±2.4 dB | See 3GPP TR 38.817-02 [17]. |
| 7.5.1 OTA adjacent channel selectivity | ±3.4 dB | See 3GPP TR 38.817-02 [17]. |
| 7.5.2 In-band blocking (General) | ±3.4 dB | See 3GPP TR 38.817-02 [17]. |
| 7.6 OTA out-of-band blocking | ±4.1 dB | See 3GPP TR 38.817-02 [17]. |
| 7.7 OTA receiver spurious emissions | FFS | FFS |
| 7.8 OTA receiver intermodulation | ±3.9 dB | See 3GPP TR 38.817-02 [17]. |
| 7.9 OTA in-channel selectivity | ±3.4 dB | See 3GPP TR 38.817-02 [17]. |

<End of changes>

# C.2	Measurement of receiver

Table C.2-1: Derivation of test requirements (FR1 OTA receiver tests)

| Test | Minimum requirement in 3GPP TS 38.104 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 7.2 OTA sensitivity | See 3GPP TS 38.104 [2], subclause 10.2 | 1.3 dB, f ≤ 3.0 GHz 1.4 dB, 3.0 GHz < f ≤ 4.2 GHz 1.6 dB, 4.2 GHz < f ≤ 6.0 GHz | Formula: Declared Minimum EIS + TT |
| 7.3 OTA reference sensitivity level | See 3GPP TS 38.104 [2], subclause 10.3 | 1.3 dB, f ≤ 3.0 GHz 1.4 dB, 3.0 GHz < f ≤ 4.2 GHz 1.6 dB, 4.2 GHz < f ≤ 6.0 GHz | Formula: EISREFSENS + TT |
| 7.4 OTA dynamic range | See 3GPP TS 38.104 [2], subclause 10.4 | 0.3 dB, f ≤ 6 GHz | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.5.1 OTA adjacent channel selectivity | See 3GPP TS 38.104 [2], subclause 10.5.1 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.5.2 In-band blocking (General) | See 3GPP TS 38.104 [2], subclause 10.5.2 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.5.2 In-band blocking (Narrowband) | See 3GPP TS 38.104 [2], subclause 10.5.2 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.6 OTA out-of-band blocking (General) | See 3GPP TS 38.104 [2], subclause 10.6 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.6 OTA out-of-band blocking (Co-location) | See 3GPP TS 38.104 [2], subclause 10.6 | 0 dB | Formula: Wanted signal power unchanged Interferer signal power - TT. |
| 7.7 OTA receiver spurious emissions | See 3GPP TS 38.104 [2], subclause 10.7 | 2.5dB, 30 MHz ≤ f ≤ 6 GHz 4.2dB, 6 GHz < f ≤ 19 GHz | Formula: Minimum Requirement + TT |
| 7.8 OTA receiver intermodulation | See 3GPP TS 38.104 [2], subclause 10.8 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged |
| 7.9 OTA in-channel selectivity | See 3GPP TS 38.104 [2], subclause 10.9 | 1.7 dB, f ≤ 3.0 GHz 2.1 dB, 3.0 GHz < f ≤ 4.2 GHz 2.4 dB, 4.2 GHz < f ≤ 6.0 GHz | Formula: Wanted signal power + TT Interferer signal power unchanged |

Table C.2-2: Derivation of test requirements (FR2 OTA receiver tests)

| Test | Minimum requirement in 3GPP TS 38.104 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 7.3 OTA reference sensitivity level | See 3GPP TS 38.104 [2], subclause 10.3 | 2.4 dB, 24.25 GHz < f ≦ 29.5 GHz 2.4 dB, 37 GHz < f ≦ 40 GHz | Formula: EISREFSENS+ TT |
| 7.5.1 OTA adjacent channel selectivity | See 3GPP TS 38.104 [2], subclause 10.5.1 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.5.2 In-band blocking | See 3GPP TS 38.104 [2], subclause 10.5.2 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.6 OTA out-of-band blocking | See 3GPP TS 38.104 [2], subclause 10.6 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged |
| 7.7 OTA receiver spurious emissions | See 3GPP TS 38.104 [2], subclause 10.7 | 2.5 dB, 30 MHz ≤ f ≤ 6 GHz 2.7 dB, 6 GHz < f ≤ 12.75 GHz 0 dB, 12.75 GHz < f ≤ 60 GHz | Formula: Minimum Requirement + TT |
| 7.8 OTA receiver intermodulation | See 3GPP TS 38.104 [2], subclause 10.8 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 7.9 OTA in-channel selectivity | See 3GPP TS 38.104 [2], subclause 10.9 | 3.4 dB, 24.25GHz < f ≦ 29.5GHz 3.4 dB, 37GHz < f ≦ 40GHz | Formula: Wanted signal power + TT Interferer signal power unchanged. |

<End of change>

# References